3GPP TSG-RAN WG5 Meeting #4-5G-NR Adhoc	R5-190029

# Singapore, Singapore, January 21th - 25th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0052 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of TT in TS38.521-2 Annex F during RAN5#NR4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Agreed TTs were not captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Agreed TTs by the end of RAN5#NR4 are captured to TS 38.521-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTs will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1, 7.4, 7.5, 7.6.2, Annex F.3.2, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document will be updated according to agreements during RAN5#NR4. |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

6.5.1	Occupied bandwidth

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty FFS.

- Connection diagram for User Equipment part is FFS

- Default Downlink power levels for FR2 NR is TBD

6.5.1.1	Test purpose

<Unchanged sections skipped>

6.5.1.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.5.1.5-1.

Table 6.5.1.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Channel bandwidth (MHz) | 50 | 100 | 200 | 400 |

6.5.2	Out of band emission

<Unchanged sections skipped>

7.4	Maximum input level

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- UL power level configuration is missing in TS 38.101-2.

- Maximum input level requirement of DL 64QAM is TBD in TS 38.101-2.

- DL RMC of 64QAM is FFS in TS 38.101-2.

- Uplink signals (Annex G) is not complete.

- The UE Rx beam peak direction found procedure with a 3D EIS scan is not complete in TR 38.810.

- Measurement uncertainty is FFS.

Editor’s note: The requirement in this test case is not testable due to maximum input level achievable in OTA test setup. Thus the test case will not be tested.

7.4.1	Test purpose

<Unchanged sections skipped>

7.5	Adjacent channel selectivity

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Measurement Uncertainty is FFS.

-	UL power level configuration is TBD.

-	Throughput calculation procedure is TBD (measurement period as well as dependencies with polarizations).

-	Connection diagram for User Equipment part is FFS.

7.5.1	Test purpose

<Unchanged sections skipped>

7.5.5	Test requirements

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A, under the conditions specified in Table 7.5.5-2 and also under the conditions specified in Table 7.5.5-3.

Table 7.5.5-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| ACS for band n257, n258, n261 | dB | 23 | 23 | 23 | 23 |
| ACS for band n260 | dB | 22 | 22 | 22 | 22 |

Table 7.5.5-2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| PInterferer for band n260 | dBm | REFSENS + 34.5 dB | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 and set-up according to Annex C []. NOTE 2: The REFSENS power level is specified in Table 7.3.2.3-1. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

Table 7.5.5-3: Test parameters for adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258, n261 | dBm | -46.5 | -46.5 | -46.5 | -46.5 |
| Power in Transmission Bandwidth Configuration for band n260 | dBm | -45.5 | -45.5 | -45.5 | -45.5 |
| PInterferer | dBm | -25 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A with one sided dynamic OCNG Pattern TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

7.5A	Adjacent channel selectivity for CA

<Unchanged sections skipped>

7.6.2	In-band blocking

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement uncertainty is FFS.

- UL power level configuration is TBD.

- Throughput calculation procedure is TBD (measurement period as well as dependencies with polarizations).

- Connection diagram for User Equipment part is FFS.

7.6.2.1	Test purpose

<Unchanged sections skipped>

7.6.2.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A with parameters specified in Tables 7.6.2.5-1.

Table 7.6.2.5-1: In band blocking test requirement

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |
| PInterferer for band n260 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB |
| FIoffset | MHz | ≤ 100 & ≥ -100 NOTE 5 | ≤ 200 & ≥ -200 NOTE 5 | ≤ 400 & ≥ -400 NOTE 5 | ≤ 800 & ≥ -800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE2: The REFSENS power level is specified in Section 7.3.2, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A QPSK, R=1/3 with one sided dynamic OCNG pattern as described in Annex A and set-up according to Annex C. NOTE 4: FIoffset is the frequency separation between the centre of the aggregated CA bandwidth and the centre frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted to ([│FInterferer│/SCS] + 0.5)SCSFInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer centre frequencies. |  |  |  |  |  |

7.6.3	Void

<Unchanged sections skipped>

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The TT values defined in this section are valid only until RAN5#85 (November 2019).

- RAN5 expects to have the following actions performed by RAN5#85 in efforts to improve the TT values

- For updated TT evaluation only, MU analysis taking into the account the data sheet from commercially available test systems or any additional experimental data to justify the values

- UE vendors’ have supplied further input on TT based on the performance of the existing mmWave UE against the RF test cases.

- At least one statistical analysis has been done on possibilities to improve test confidence level.

- Measurement results & evaluation of network performance / commercially available devices has been provided.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP and TRP) | PC1 Minimum peak EIRP TBD Max TRP TBD Max EIRP 0 dB PC2 Minimum peak EIRP TBD Max TRP TBD Max EIRP 0 dB PC3 Minimum peak EIRP IFF (DUT ≤ 15 cm) [2.9] dB (FR2a) [3.1] dB (FR2b) IFF (DUT ≤ 30 cm) [2.8] dB (FR2a) [3.0] dB (FR2b) Max TRP IFF (DUT ≤ 15 cm) [2.8] dB (FR2a) [3.0] dB (FR2b) IFF (DUT ≤ 30 cm) [2.7] dB (FR2a) [3.0] dB (FR2b) Max EIRP 0 dB PC4 Minimum peak EIRP TBD Max TRP TBD Max EIRP 0 dB |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 TBD PC4 TBD |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | TBD |  |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.3.1 Minimum output power | TBD |  |
| 6.3.2 Transmit OFF power | 0 dB |  |
| 6.3.3.2 General ON/OFF time mask | TBD |  |
| 6.3.3.3 Transmit power time mask for slot boundaries | TBD |  |
| 6.3.3.4 PRACH time mask | TBD |  |
| 6.3.3.5 PUCCH time mask | TBD |  |
| 6.3.3.5.1 Long PUCCH time mask | TBD |  |
| 6.3.3.5.2 Short PUCCH time mask | TBD |  |
| 6.3.3.7 PUSCH-PUCCH and PUSCH-SRS time masks | TBD |  |
| 6.4.1 Frequency error | [0.005] ppm |  |
| 6.4.2.1 Error vector magnitude | 0%, up to 64QAM | Minimum requirement + TT |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.5.1 Occupied bandwidth | 0 kHz | Minimum requirement + TT |
| 6.5.2.1 Spectrum Emission Mask | IFF (DUT ≤ 15 cm) TBD IFF (DUT ≤ 30 cm) [4.6] dB (23.45GHz ≤ f ≤ 30.3GHz) [5.6] dB (30.3GHz < f ≤ 40.8GHz) |  |
| 6.5.2.2 Additional Spectrum Emissions Mask | TBD |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Absolute requirement 0 dB Relative requirement IFF (DUT ≤ 15 cm) TBD IFF (DUT ≤ 30 cm) BW = 50MHz [4.6] dB (23.45GHz ≤ f ≤ 30.3GHz) [5.0] dB (30.3GHz < f ≤ 40.8GHz) |  |
| 6.5.3.1 Transmitter Spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.3.2 Spurious emission band UE co-existence | 0 dB | Minimum requirement + TT |
| NOTE 1: FR2a: 23.45GHz ≤ f < 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz |  |  |

F.3.3	Measurement of receiver

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The TT values defined in this section are valid only until RAN5#85 (November 2019).

- RAN5 expects to have the following actions performed by RAN5#85 in efforts to improve the TT values

- For updated TT evaluation only, MU analysis taking into the account the data sheet from commercially available test systems or any additional experimental data to justify the values

- UE vendors’ have supplied further input on TT based on the performance of the existing mmWave UE against the RF test cases.

- At least one statistical analysis has been done on possibilities to improve test confidence level.

- Measurement results & evaluation of network performance / commercially available devices has been provided.

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | IFF (DUT ≤ 15 cm, FR2a, FR2b) [3.3] dB IFF (DUT ≤ 30 cm, FR2a, FR2b) [3.2] dB |  |
| 7.4 Maximum input level | TBD |  |
| 7.5 Adjacent channel selectivity | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.6.2 In-band blocking | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.6.3 Out-of-band blocking | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.7 Spurious response | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.9 Spurious emissions | 0 dB | Minimum requirement + TT T-put limit unchanged |
| NOTE 1: FR2a and FR2b are specified in Table F.3.2-1. |  |  |

Annex G (normative): Uplink Physical Channels
<Unchanged sections skipped>