3GPP TSG-RAN WG5 Meeting #4-5G-NR Adhoc	R5-190030

# Singapore, Singapore, January 21th - 25th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0095 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of MU in TS38.521-3 Annex F during RAN5#NR4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Agreed MUs were not captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Agreed MUs by the end of RAN5#NR4 are captured to TS 38.521-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MUs will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.1.2, F.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document will be updated according to agreements during RAN5#NR4. |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC | (LTE: f ≤ 3.0GHz) NR: f ≤ 3.0GHz ±1.0 dB, BWNR ≤ 40MHz ±1.6 dB, 40MHz < BWNR ≤ 100MHz NR: 3.0GHz < f ≤ 4.2GHz ±1.2 dB, BWNR ≤ 40MHz ±1.7 dB, 40MHz < BWNR ≤ 100MHz NR: 4.2GHz < f ≤ 6.0GHz ±1.5 dB, BWNR ≤ 40MHz ±1.7 dB, 40MHz < BWNR ≤ 100MHz (LTE: 3.0GHz < f ≤ 4.2GHz) NR: f ≤ 3.0GHz ±1.2 dB, BWNR ≤ 40MHz ±1.7 dB, 40MHz < BWNR ≤ 100MHz NR: 3.0GHz < f ≤ 4.2GHz ±1.4 dB, BWNR ≤ 40MHz ±1.9 dB, 40MHz < BWNR ≤ 100MHz NR: 4.2GHz < f ≤ 6.0GHz ±1.6 dB, BWNR ≤ 20MHz ±1.8 dB, 20MHz < BWNR ≤ 40MHz ±1.9 dB, 40MHz < BWNR ≤ 100MHz |  |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 | (LTE: f ≤ 3.0GHz) NR: f ≤ 3.0GHz ±1.0 dB, BWNR ≤ 40MHz ±1.6 dB, 40MHz < BWNR ≤ 100MHz NR: 3.0GHz < f ≤ 4.2GHz ±1.2 dB, BWNR ≤ 40MHz ±1.7 dB, 40MHz < BWNR ≤ 100MHz NR: 4.2GHz < f ≤ 6.0GHz ±1.5 dB, BWNR ≤ 40MHz ±1.7 dB, 40MHz < BWNR ≤ 100MHz (LTE: 3.0GHz < f ≤ 4.2GHz) NR: f ≤ 3.0GHz ±1.2 dB, BWNR ≤ 40MHz ±1.7 dB, 40MHz < BWNR ≤ 100MHz NR: 3.0GHz < f ≤ 4.2GHz ±1.4 dB, BWNR ≤ 40MHz ±1.9 dB, 40MHz < BWNR ≤ 100MHz NR: 4.2GHz < f ≤ 6.0GHz ±1.6 dB, BWNR ≤ 20MHz ±1.8 dB, 20MHz < BWNR ≤ 40MHz ±1.9 dB, 40MHz < BWNR ≤ 100MHz |  |
| 6.2B.2.1 UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | Same as 6.2B.1.1 |  |
| 6.2B.2.2 UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | Same as 6.2B.1.2 |  |
| 6.2B.2.3 UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 | Same as 6.2B.1.3 |  |
| 6.2B.3.1 UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC | Same as 6.2B.1.1 |  |
| 6.2B.3.2 UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | Same as 6.2B.1.2 |  |
| 6.2B.3.3 UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 | Same as 6.2B.1.3 |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC | Same as 6.2B.1.1 |  |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC | Same as 6.2B.1.2 |  |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 | Same as 6.2B.1.3 |  |
| 6.3B.1.1 Minimum Output Power for intra-band contiguous EN-DC | Same as 6.3.1 in TS 38.521-1 |  |
| 6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC | Same as 6.3.1 in TS 38.521-1 |  |
| 6.3B.1.3 Minimum output power for inter-band EN-DC within FR1 | Same as 6.3.1 in TS 38.521-1 |  |
| 6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC | Same as 6.3.2 in TS 38.521-1 |  |
| 6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC | Same as 6.3.2 in TS 38.521-1 |  |
| 6.3B.2.3 Transmit OFF Power for inter-band EN-DC within FR1 | Same as 6.3.2 in TS 38.521-1 |  |
| 6.3B.3.1 Tx ON/OFF time mask for intra-band contiguous EN-DC | Same as 6.3.3 in TS 38.521-1 |  |
| 6.3B.3.2 Tx ON/OFF time mask for intra-band non-contiguous EN-DC | Same as 6.3.3 in TS 38.521-1 |  |
| 6.3B.3.3 Tx ON/OFF time mask for inter-band EN-DC within FR1 | Same as 6.3.3 in TS 38.521-1 |  |
| 6.4B.1.1 Frequency Error for intra-band contiguous EN-DC | TBD |  |
| 6.4B.1.2 Frequency Error for intra-band non-contiguous EN-DC | TBD |  |
| 6.4B.1.3 Frequency Error for inter-band EN-DC within FR1 | Same as 6.4.1 in TS 38.521-1 |  |
| 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN-DC | TBD |  |
| 6.4B.2.1.2 Carrier Leakage for intra-band contiguous EN-DC | TBD |  |
| 6.4B.2.1.3 In-band Emissions for intra-band contiguous EN-DC | TBD |  |
| 6.4B.2.1.4 EVM Equalizer Flatness for intra-band contiguous EN-DC | TBD |  |
| 6.4B.2.2.1 Error Vector Magnitude for intra-band non-contiguous EN-DC | TBD |  |
| 6.4B.2.2.2 Carrier Leakage for intra-band non-contiguous EN-DC | TBD |  |
| 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC | TBD |  |
| 6.4B.2.2.4 EVM Equalizer Flatness for intra-band non contiguous EN-DC | TBD |  |
| 6.4B.2.3.1 Error Vector Magnitude for inter-band EN-DC within FR1 | Same as 6.4.2.1 in TS 38.521-1 |  |
| 6.4B.2.3.2 Carrier Leakage for inter-band EN-DC within FR1 | Same as 6.4.2.2 in TS 38.521-1 |  |
| 6.4B.2.3.3 In-band Emissions for inter-band EN-DC within FR1 | Same as 6.4.2.3 in TS 38.521-1 |  |
| 6.4B.2.3.4 EVM Equalizer Flatness for inter-band EN-DC within FR1 | Same as 6.4.2.4 in TS 38.521-1 |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC | 1.5% of aggregated channel bandwidth |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC | Same as 6.5.1 in TS 38.521-1 |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 | Same as 6.5.1 in TS 38.521-1 |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC | Same as 6.5.2.2 in TS 38.521-1 |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC | Same as 6.5.2.3 in TS 38.521-1 |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC | Same as 6.5.2.3 in TS 38.521-1 |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC | Same as 6.5.2.2 in TS 38.521-1 |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC | Same as 6.5.2.3 in TS 38.521-1 |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC | TBD |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 | Same as 6.5.2.2 in TS 38.521-1 |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 | Same as 6.5.2.3 in TS 38.521-1 |  |
| 6.5B.2.3.3 Adjacent channel leakage ratio for inter-band EN-DC within FR1 | Same as 6.5.2.4.1 in TS 38.521-1 |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 | Same as 6.5.3.1 in TS 38.521-1 |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | Same as 6.5.3.1 in TS 38.521-1 |  |

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC | Same as 7.3.2 in TS 38.521-1 |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC | Same as 7.3.2 in TS 38.521-1 |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 | Same as 7.3.2 in TS 38.521-1 |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC | Same as 7.4 in TS 38.521-1 |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC | Same as 7.4 in TS 38.521-1 |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 | Same as 7.4 in TS 38.521-1 |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC | TBD |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC | TBD |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 | Same as 7.5 in TS 38.521-1 |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 | TBD |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 | TBD |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 | Same as 7.6.2 in TS 38.521-1 |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 | TBD |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 | TBD |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 | Same as 7.6.3 in TS 38.521-1 |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 | TBD |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 | TBD |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 | Same as 7.6.4 in TS 38.521-1 |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 | TBD |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 | TBD |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 | Same as 7.7 in TS 38.521-1 |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 | Same as 7.8.2 in TS 38.521-1 |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 | Same as 7.8.2 in TS 38.521-1 |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC within FR1 | Same as 7.8.2 in TS 38.521-1 |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 | Same as 7.9 in TS 38.521-1 |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 | Same as 7.9 in TS 38.521-1 |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 | Same as 7.9 in TS 38.521-1 |  |

F.2	Interpretation of measurement results (normative)

<Unchanged sections skipped>