3GPP TSG-RAN WG5 Meeting #4-5G-NR Adhoc	R5-190051

# Singapore, Singapore, January 21th - 25th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0413 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of test channel bandwidth in TS 38.508-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During RAN5#81 (Nov-2018), following proposal in R5-188055 to update Mid channel bandwidth of Band n257 was endorsed. This endorsement should be captured into the specification. Proposal 2 : Define test Mid Channel Bandwidth of Band n257 as 100MHz if UE supports BW 100MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change Mid ChBW of Band n257 from 200 MHz to 100 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Endorsed proposal will not be captured into the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

4.3	Reference test conditions

4.3.1	Test frequencies

4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8] and TS 38.101-3 [9].

4.3.1.0A	Mid test channel bandwidth

Editor’s Note: The note in table 4.3.1-1 and 4.3.1-2 to be updated based on RAN plenary updates.

The Mid test channel bandwidth definition for RF is given in Table 4.3.1-1 and Table 4.3.1-2 for FR1 and FR2 respectively.

Table 4.3.1-1: Mid Test Channel bandwidths for each NR band, FR1

| NR band / UE Mid Test Channel bandwidth |  |
| --- | --- |
| NR Band | Mid [MHz] |
| n1 | 15 |
| n2 | 15 |
| n3 | 15 |
| n5 | 15 |
| n7 | 15 |
| n8 | 15 |
| n12 | 10 |
| n20 | 15 |
| n25 | 15 |
| n28 | 15 |
| n34 | 10 |
| n38 | 15 |
| n39 | 20 |
| n40 | 30 |
| n41 | 50 |
| n51 | 5 |
| n66 | 20 |
| n70 | 15 |
| n71 | 10 |
| n75 | 15 |
| n76 | 5 |
| n77 | 50 |
| n78 | 50 |
| n79 | 60 |
| n80 | 20 |
| n81 | 15 |
| n82 | 15 |
| n83 | 15 |
| n84 | 15 |
| n86 | 20 |
| Note 1: For UEs where IOT bit declaration is required due to lack of channel BW support in the network, if mid channel BW is not supported by the UE, select the closest lower channel BW supported by the UE in both UL and DL. This shall apply until further updates from RAN plenary and only for Rel 15 UEs. |  |

Table 4.3.1-2: Mid Test Channel bandwidths for each NR band, FR2

| NR band / UE Mid Test Channel bandwidth |  |  |
| --- | --- | --- |
| NR Band | Mid [MHz] |
| n257 | 100 |
| n258 | [200] |
| n260 | [200] |
| NOTE 1: For UEs where IOT bit declaration is required due to lack of channel BW support in the network, if mid channel BW is not supported by the UE, select the closest lower channel BW supported by the UE in both UL and DL. This shall apply until further updates from RAN plenary and only for Rel 15 UEs. |  |

4.3.1.0B	Low test channel bandwidth

Editor’s Note: The note in table 4.3.1.0B-1 and 4.3.1.0B-2 to be updated based on RAN plenary updates.

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1 and Table 4.3.1.0B-2 for FR1 and FR2 respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR band / UE Low Test Channel bandwidth |  |
| --- | --- |
| NR Band | Low [MHz] |
| n1 | 5 |
| n2 | 5 |
| n3 | 5 |
| n5 | 5 |
| n7 | 5 |
| n8 | 5 |
| n12 | 5 |
| n20 | 5 |
| n25 | 5 |
| n28 | 5 |
| n34 | 5 |
| n38 | 5 |
| n39 | 5 |
| n40 | 5 |
| n41 | 10 |
| n51 | 5 |
| n66 | 5 |
| n70 | 5 |
| n71 | 5 |
| n75 | 5 |
| n76 | 5 |
| n77 | 10 |
| n78 | 10 |
| n79 | 40 |
| n80 | 5 |
| n81 | 5 |
| n82 | 5 |
| n83 | 5 |
| n84 | 5 |
| n86 | 5 |
| NOTE 1: For UEs where IOT bit declaration is required due to lack of channel BW support in the network, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 UEs and until further updates are provided from RAN plenary |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR band / UE Low Test Channel bandwidth |  |  |
| --- | --- | --- |
| NR Band | Low [MHz] |
| n257 | 50 |
| n258 | 50 |
| n260 | 50 |
| n261 | 50 |
| NOTE 1: For UEs where IOT bit declaration is required due to lack of channel BW support in the network, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 UEs and until further updates are provided from RAN plenary |  |

4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1 and Table 4.3.1.0C-2 for FR1 and FR2 respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR band / UE High Test Channel bandwidth |  |
| --- | --- |
| NR Band | High [MHz] |
| n1 | 20 |
| n2 | 20 |
| n3 | 30 |
| n5 | 20 |
| n7 | 20 |
| n8 | 20 |
| n12 | 15 |
| n20 | 20 |
| n25 | 20 |
| n28 | 20 |
| n34 | 15 |
| n38 | 20 |
| n39 | 40 |
| n40 | 80 |
| n41 | 100 |
| n51 | 5 |
| n66 | 40 |
| n70 | 151/252 |
| n71 | 20 |
| n75 | 20 |
| n76 | 5 |
| n77 | 100 |
| n78 | 100 |
| n79 | 100 |
| n80 | 30 |
| n81 | 20 |
| n82 | 20 |
| n83 | 20 |
| n84 | 20 |
| n86 | 40 |
| NOTE 1: This UE channel bandwidth is applicable only to uplink. NOTE 2: This UE channel bandwidth is applicable only to downlink. |  |

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR band / UE High Test Channel bandwidth |  |  |
| --- | --- | --- |
| NR Band | High [MHz] |
| n257 | 400 |
| n258 | 400 |
| n260 | 400 |
| n261 | 400 |

4.3.1.0B	Bandwidth part

<Unchanged sections skipped>