3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190103

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0050 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Point analysis for NR spurious response in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test Point analysis for NR spurious response in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed TP selections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

4.1	Test point analysis for FR1

This clause contains information on test point analysis and test point selection for test cases in [2] clause 6 and 7 with information about transmitting test point selection for FR1 listed in table 4.1-1 and receiver test point selection in table 4.1-2.

Table 4.1-1: NR UE transmitter test point selection for FR1

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 6.2.1 UE maximum output power | TBD | “38.521-1_TPanalysis_6.2.1_MaxOP.zip” | RAN5#2-5G-NR Adhoc |
| 6.2.2 Maximum Power Reduction (MPR) | power class 3: 1040 power class 2: 920 | “38.521-1_TPanalysis_6.2.2_MPR.zip” | RAN5#81 |
| 6.2C.1 Configured UE transmitted Output Power | 270 | “38.521-1_TPanalysis_6.2C.1_ConfigOPSUL.zip” | RAN5#80 |
| 6.2.3 UE A-MPR | Table 4.1.1.1-1 | Table 4.1.1.1-1 | See Table 4.1.1.1-1 |
| 6.3.1 Minimum output power | TBD | “38.521-1_TPanalysis_6.3.1_MinOP.zip” | RAN5#2-5G-NR Adhoc |
| 6.3.3.2 General ON/OFF time mask | TBD | “38.521-1_TPanalysis_6.3.3.2_OnOff_M.zip” | RAN5#2-5G-NR Adhoc |
| 6.3.4.2 Absolute power tolerance | 6 | “38.521-1_TPanalysis_6.3.4.2_AbsPtol.zip” | RAN5#78 |
| 6.3.4.3 Relative power tolerance | TBD | “38.521-1_TPanalysis_6.3.4.3_RelPtol.zip” | RAN5#81 |
| 6.3.4.4 Aggregate power tolerance | PUCCH: 6 PUSCH: 6 | “38.521-1_TPanalysis_6.3.4.4_AggPtol.zip” | RAN5#78 |
| 6.4.1 Frequency error | 5 | “38.521-1_TPanalysis_6.4.1_FreqErr_v2.zip” | RAN5#2-5G-NR Adhoc RAN5#80 |
| 6.4.2.1 Error Vector Magnitude | PUSCH: 252 PUCCH: 36 PRACH: 36 | “38.521-1_TPanalysis_6.4.2.1_EVM.zip” | RAN5#80 |
| 6.4.2.2 Carrier leakage | 3 | “38.521-1_TPanalysis_6.4.2.2_CarrLeak.zip” | RAN5#80 |
| 6.4.2.3 In-band emissions | 36 | “38.521-1_TPanalysis_6.4.2.3_IE.zip” | RAN5#80 |
| 6.4.2.4 EVM equalizer spectrum flatness | 90 | “38.521-1 TPanalysis_6.4.2.4_EVMequalizerSpectrumFlatness_v2.zip” | RAN5#3-5G-NR |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK | 45 | “38.521-1 TPanalysis_6.4.2.5_EVMequalizerSpectrumFlatness_BPSK.zip” | RAN5#81 |
| 6.5.1 Occupied bandwidth | 10 | “38.521-1_TPanalysis_6.5.1_OccBW.zip | RAN5#78 |
| 6.5.2.2 Spectrum Emission Mask | 144 for Power Class 3 144 for Power Class 2 | “38.521-1_TPanalysis_6.5.2.2_SEM_v2.zip” | RAN5#3-5G-NR-Adhoc |
| 6.5.2.4.1 NR Adjacent channel leakage ratio | 920 for Power Class 3 920 for Power Class 2 | “38.521-1_TPanalysis_6.5.2.4_ACLR_v2.zip” | RAN5#3-5G-NR-Adhoc |
| 6.5.2.4.2 UTRA Adjacent channel leakage ratio | 680 | “38.521-1_TPanalysis_6.5.2.4.2_UTRA ACLR.zip” | RAN5#3-5G-NR Adhoc |
| 6.5.3.2 Spurious emissions for UE co-existence | TBD | “38.521-1_TPanalysis_6.5.3.2_SEcoex.zip” | RAN5#2-5G-NR Adhoc |
| 6.5.4 Transmit intermodulation | 8 | “38.521-1_TPanalysis_6.5.4_TxIm.zip” | RAN5#80 |

Table 4.1-2: NR UE receiver test point selection for FR1

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 7.3 Reference sensitivity power level | 45 | “38.521-1_TPanalysis_7.3_RefSense.zip” | RAN5#79 |
| 7.4 Maximum input level | 6 | “38.521-1_TPanalysis_7.4_Maximun input level.zip” | RAN5#81 |
| 7.5 Adjacent Channel Selectivity | 3 | “38.521-1_TPanalysis_7.5_ACS.zip” | RAN5#2-5G-NR Adhoc |
| 7.6.2 In Band Blocking | 3 | “38.521-1_TPanalysis_7.6.2_InB_Block.zip” | RAN5#2-5G-NR Adhoc |
| 7.6.3 Out-of-band blocking | 3 | “38.521-1_TPanalysis_7.6.3_OobBlocking.zip” | RAN5#80 |
| 7.7 Spurious response | 3 | “38.521-1_TPanalysis_7.7_Spurious response.zip” | RAN5#4-5G-NR Adhoc |
| 7.8.2 Wide band Intermodulation | 3 | “38.521-1_TPanalysis_7.8.2_WidebandIntermod.zip” | RAN5#81 |
| 7.9 Spurious emissions | 3 | “38.521-1_TPanalysis_7.9_RxSpurious.zip” | RAN5#81 |

<< End of changes >>

Add the following attached .zip file to TR 38.905:

38.521-1_TPanalysis_7.7_Spurious response.zip