3GPP TSG-RAN5 Meeting NR#4 AdHoc	R5-190126

# Singapore, 21th – 25th Jan. 2019

Title:							Test approach for EN-DC TRx tests with CCs > 2

Source:			Anritsu

Agenda Item:			5.1.15

Document for:			Endorsement

# 1.	Introduction

Test cases with multiple-CCs are about to be added to the test specification TS 38.521-3. In this paper, test methodology and structure of EN-DC test cases are discussed and proposed considering the scalability for the any type of EN-DC band combinations with mixed Intra-band/Inter-band types and exceptional/non-exception requirements. This topic is once mentioned in [1], but was postponed to wait for the anchor agnostic approach applicability for EN-DC from RAN4. Now the LTE Anchor Agnostic approach is formally incorporated, we revisit this topic.

# 2.	Discussion

# 2.1.	Applying anchor agnostic approach for multi-CC cases

The main difference from the SA CA test cases is that EN-DC test cases are based on anchor agnostic approach which is summarized in [2]. Though the first discussion was introduced for the simple 1LTE + 1NR cases, we can straightforwardly extend it to the cases with multiple-CCs.

Following tables summarizes the test coverage rule for EN-DC sited from [2].

Table 1: EN-DC Test Coverage/Procedure applicable to TS38.101-3

| UE capability | NR exception reqs in 38.101-3 | NR non-exception reqs in 38.101-1/-2 | LTE exception reqs in 38.101-3 | LTE non-exception reqs in 36.101 |
| --- | --- | --- | --- | --- |
| NSA only | Tested | Tested (LTE agnostic)1 | Tested | Tested (SA LTE per TS 36.521-1) |
| NSA+SA | Tested | Tested (SA NR per 38.101-1/-2) | Tested | Tested (SA LTE per TS 36.521-1) |
| Note 1: Pick LTE test configuration that does not interfere with NR operation as per 36.101 Section 6.1 and 7.1. No LTE test point analysis is required. |  |  |  |  |

Above table is very generic and can be applied for the case of CCs > 2 as is. But, when we implement this principle in the test spec, we need to clarify several things.

For NR non-exceptional requirements, LTE agnostic approach can be applied as well. As the LTE should be selected so as not to interfere with NR carriers, then just having 1 E-UTRA Cell is enough(or should be 1 cell) for multi-LTE CCs case.

Observation 1 : NR non-exception reqs are tested with LTE agnostic approach, with only 1 LTE PCC configured using the LTE configuration in section 4.6 of 38.521-3 is enough.

For LTE non-exceptional requirements, nothing is different from 1LTE+1NR case and are tested in 36.521-1.

Observation 2 : LTE non-exception reqs are tested in LTE 36.521-1.s

For NR exceptional requirements, they will be explicitly tested in 38.521-3. The main points are whether such exceptional requirements should be tested with LTE CCs which does not relating to the NR exceptional requirement. As already discussed in [1], from the perspective of avoiding the increased test system complexity (due to the increasing number of Rx antenna ports(4Rx, 8Rx…) and CCs ) the testing with the minimum number of LTE CCs which is relating to NR exceptional requirements is desired. Also, LTE agnostic approach itself can mean that CCs which does not interfere with other cells are not required to be configured for testing the requirement imposed to “other cells”.  Then this approach is more natural to be applied considering the existence of LTE agnostic approach.

Proposal 1 :  NR exception reqs are tested by configuring min. number of LTE CCs concerning that requirements.

Same can be said for the LTE exceptional requirements.

Proposal 2 : LTE exception reqs are tested by configuring min. number of NR CCs concerning that requirements.

With above observations and proposals, we can think about the following very generic example.

Example 1 : DC_XA-YA-ZA-WA_nSA-nTA-nVA (4DL/2UL LTE, 2 DL/1 UL NR FR1, 1 DL/UL NR FR2)

X : LTE PCC(DL&UL)

Y : LTE SCC1(DL&UL)

Z : LTE SCC2(DL)

W : LTE SCC3(DL)

nS : FR1 NR PCC(DL&UL)

nT : FR1 NR SCC(DL)

nV : FR2 NR PCC(DL&UL)

Assume following exceptional requirements are defined:

- MSD due to dual uplink operation: UL in X and nS causes the MSD for band Z

- MSD due to UL harmonics interference: UL in Y causes MSD for band nV

In this case, performance of DC_XA-YA-ZA-WA_nSA-nTA-nVA is guaranteed by testing:

Table 1 Example of test ceverages

| # | Subset of DC/CA band combination | Test environment | Test Spec/Core Spec | Verified requirement |
| --- | --- | --- | --- | --- |
| 1 | DC_XA-ZA-nSA | Conducted | 38.521-3/ 38.101-3 | Exceptional requirements : MSD i) |
| 2 | DC_XA-YA-nSA-nVA | OTA | 38.521-3/ 38.101-3 | Exceptional requirements : MSD ii) (Only the FR2 NR CC nV will be measured) |
| 3 | CA_XA-YA-ZA-WA | Conducted | 36.521-1/38.101-1 | Non-exceptional requirements among LTE CCs |
|  | For NSA + SA UE |  |  |  |
| 4 | CA_nSA-nTA | Conducted | 38.521-1/38.101-1 | Non-exceptional requirements among FR1 NR CCs |
| 5 | CA_nVA | OTA | 38.521-2/38.101-2 | Non-exceptional requirements among FR2 NR CCs |
|  | For NSA only UE |  |  |  |
| 6 | DC_*A_nSA-nTA “*” is chosen so that they does not interfere with nTA and nSA | Conducted | 38.521-3/38.101-1 | Non-exceptional requirements among FR1 NR CCs |
| 7 | DC_*A_nVA “*” is chosen so that it does not interfere with nVA | OTA | 38.521-3/38.101-2 | Non-exceptional requirements among FR2 NR CCs |

# 2.2.	Test case structure

In the latest WP, the test cases are re-defined for each number of CCs. If Proposal 1 and Proposal 2 are to be agreed, then there are two options how to consider the multiple-CC test case definition.

Option B1 : Test cases are defined with the number of CCs in the DC band combination

Option B2 : Test cases are defined with the configured number of CCs in the DC configuration

Taking the example in above, the test cases are defined for DC_XA-YA-ZA-WA_nSA-nTA-nVA, i.e. test cases with 7CC with Option B1, while with Option B2, no tests defined for DC_XA-YA-ZA-WA_nSA-nTA-nVA, but only the test cases with the tested subsets combinations: DC_XA-ZA-nSA(#1 in Table 2) and DC_XA-YA-nSA-nVA(#2 in Table 1)  are defined in 3CC test cases and 4 CC test cases respectively.

In case of Option B2, description about which test cases needs to be tested should be put somewhere in the test spec so that readers can recognize the required test to be executed.

From the perspective simplicity of certification, it would be tester friendly if the single test case per EN-DC band combination is defined.

# Proposal 3 : Test cases are defined with the number of CCs in the DC band combination (Option B1)

Proposal 3 can be endorsable, then the test configuration table for each EN-DC band combination needs to clarify followings for each test point.

- Which CCs among all the CCs in the EN-DC band combination should be configured and which are not.

- Configurations for the configured CCs

- Requirements for the “tested” CCs

Concrete test configuration table format can be considered at later stage.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Observation 1 : NR non-exception reqs are tested with LTE agnostic approach, with only 1 LTE PCC configured using the LTE configuration in section 4.6 of 38.521-3 is enough.

Observation 2 : LTE non-exception reqs are tested in LTE 36.521-1.s

Proposal 1 :  NR exception reqs are tested by configuring min. number of LTE CCs concerning that requirements.

Proposal 2 : LTE exception reqs are tested by configuring min. number of NR CCs concerning that requirements.

# Proposal 3 : Test cases are defined with the number of CCs in the DC band combination (Option B1)

# 4.	References

[1] R5-185666, “Test approach of EN-DC TRx tests”, Anritsu, RAN5#NR#3 AdHoc

[2] R5-186228, “	Discussion on applicability rules and test coverage for 38.521-3 test cases”, Qualcomm Inc., RAN5 NR#4 AdHoc