3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190131

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0421 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update chapter 4.5 General for PDUs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To update the RRC_IDLE chapter for PDU in 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Different PDUs for procedure NR RRC_IDLE have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The generic procedures used for NR testing with different numbers of PDUs will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### 4.5.2.2	Procedures

Table 4.5.2.2-1: E-UTRA RRC_IDLE

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-9 | Same as TS 36.508 [2] table 4.5.2.3-1, steps 1-9a2. | - | - |
| - | EXCEPTION: Steps 10a1 to 10a2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - |
| 10a1-10a2 | IF Test Mode = On OR Test Loop Function = On THEN same as TS 36.508 [2] table 4.5.2A.3-1, steps 10-11. The ACTIVATE TEST MODE is using the associated condition for the test loop. | - | - |
| - | EXCEPTION: Steps 11a1 to 11b9b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a proc8edure parameter has a particular value. | - | - |
| 11a1-11a | IF Test Mode = On OR Test Loop Function = On THEN same as TS 36.508 [2] table 4.5.2A.3-1, steps 12-19. | - | - |
| 11b1-11b8 | ELSE, same as TS 36.508 [2] table 4.5.2.3-1, steps 10-17. | - | - |

Table 4.5.2.2-2: NR RRC_IDLE

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 |  | <-- | NR RRC: SYSTEM INFORMATION (BCCH) |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message and an REGISTRATION REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST |
| 5 | The SS transmits a DLInformationTransfer message and a AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST |
| 6 | The UE transmits an ULInformationTransfer message and a AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE |
| 8 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND |
| 9 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE |
| - | EXCEPTION: Steps 9a1 to 9a2 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - |
| 9a1 | IF Test Mode = On OR Test Loop Function = On, the SS transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure. The ACTIVATE TEST MODE message is using the associated condition for the test loop. | <-- | RRC: DLInformationTransfer TC: ACTIVATE TEST MODE |
| 9a2 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE |
| 10 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand |
| 11 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete |
| 12 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry |
| 13 | The UE transmits a UECapabilityInformation message. | --> | NR RRC: UECapabilityInformation |
| 14 | The SS transmits a DLInformationTransfer message and an REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT |
| 15 | The UE transmits an ULInformationTransfer message and an REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE |
| - | EXCEPTION: Step 15Aa1 describes behavior depending on UE implementation; the “lower case letter” identify the branch. | - | - |
| 15Aa1 | If pc_PDU_IMS or pc_PDU_Internet is true, THEN the PDU session establishment procedure according to table 4.5.7.1-1 takes place | - | - |
| - | EXCEPTION: Step 15Ba1 describes behavior depending on UE implementation; the “lower case letter” identify the branch. | - | - |
| 15Ba1 | If pc_PDU_IMS_Internet or pc_PDU_Internet_IMS is true, THEN the PDU session establishment procedure according to table 4.5.7.1-2 takes place | - | - |
| 16 | Void | - | - |
| 17 | Void | - | - |
| 18 | Void | - | - |
| 19a1 | Void | - | - |
| 20 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease |

#### 4.5.2.3	Specific message contents

All specific message contents shall be according clause 4.6 and 4.7 and TS 36.508 [2] clause 4.6 and 4.7.

<End of modified section>

<Start of next modified section>

### 4.5.7	UE-requested PDU session establishment procedure

#### 4.5.7.1	Procedures

Table 4.5.7.1-1: 1st PDU session establishment procedure

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The UE transmits an ULInformationTransfer message and a PDU SESSION ESTABLISHMENT REQUEST. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST |
| 2 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB1. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT |
| 3 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |
| - | EXCEPTION: Step 4a1 describes behavior depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - |
| 4a1 | If initiated by the UE, the generic procedure for IP address allocation in the user plane, specified in subclause 4.5.6, takes place performing IP address allocation in the user plane. | - | - |
| - | EXCEPTION: Step 5a1 describes behavior depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - |
| 5a1 | IF pc_PDU_IMS THEN the generic procedure for IMS signalling in the U-plane specified in subclause 4.5.8 takes place. | - | - |

Table 4.5.7.1-2: 2nd PDU session establishment procedure

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The UE transmits an ULInformationTransfer message and a PDU SESSION ESTABLISHMENT REQUEST. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST |
| 2 | The SS transmits an RRCReconfiguration message to establish DRB2. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT |
| 3 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |
| - | EXCEPTION: Step 4a1 describes behavior depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - |
| 4a1 | If initiated by the UE, the generic procedure for IP address allocation in the user plane, specified in subclause 4.5.6, takes place performing IP address allocation in the user plane. | - | - |
| - | EXCEPTION: Step 5a1 describes behavior depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - |
| 5a1 | IF pc_PDU_Internet_IMS THEN the generic procedure for IMS signalling in the U-plane specified in subclause 4.5.8 takes place. | - | - |

#### 4.5.7.2	Specific message contents

All specific message contents shall be according clause 4.6 and 4.7 with the exceptions below.

Table 4.5.7.2-1: RRCReconfiguration (step 2, Table 4.5.7.1-1)

Derivation Path: TS 38.508-1, table 4.6.1-3 and condition NR

Table 4.5.7.2-2: RRCReconfiguration (step 2, Table 4.5.7.1-2)

| Derivation Path: TS 38.508-1, table 4.6.1-3 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
|  |  | RadioBearerConfig with condition DRB2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.7.2-3: Message PDU SESSION ESTABLISHMENT ACCEPT (step 2, Table 4.5.7.1-1)

Derivation path: TS 38.508-1, table 4.7.2-2 IF pc_PDU_IMS is true with condition IMS_DNN.

Table 4.5.7.2-4: Message PDU SESSION ESTABLISHMENT ACCEPT (step 2, Table 4.5.7.1-2)

Derivation path: TS 38.508-1, table 4.7.2-2 IF pc_PDU_Internet_IMS is true with condition IMS_DNN.

### 4.5.8	Procedure for IMS signalling

The purpose of this procedure is to allow the successful completion of IMS signalling.

The procedure is applicable for UEs with IMS support (TS 38.508-2 A.4.4-1/n).

Table 4.5.8-1: Procedure for IMS signaling

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-9 | Registration procedure according TS 34.229-1 [43] subclause C.2 (steps 3-11). | - | - |

<End of modified section>