3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190170

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0346 | rev |  | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Idle Mode TC - Steering of UE in roaming during registration/security check successful using List Type 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 01/14/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of 5GS Idle Mode TC 38.523-1 6.3.1.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new Test Case as part of Idle SA Option 2 WP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA Option 2 Idle Mode WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

6	Idle mode operations

6.1	NR idle mode operations

#### 6.3.1	Steering of Roaming

##### 6.3.1.1	Steering of UE in roaming during registration/security check successful using List Type 1

6.3.1.1.1	Test Purpose (TP)

(1)

with {UE being in automatic PLMN selection mode, current VPLMN not part of “PLMNs where registration was aborted due to SOR” List and not part of "User Controlled PLMN Selector with Access Technology" list and UE's USIM configured with indication that the UE is to receive Steering of Roaming information due to initial registration in a VPLMN}

ensure that {

when {SOR Transparent container indicates ACK has been requested & List Type indicates PLMN ID and Access technology list in REGISTRATION ACCEPT and security check is successful}

then {UE sends ACK in REGISTRATION COMPLETE, waits for network release of the NAS signalling connection and selects higher priority PLMN}

}

6.3.1.1.2	Conformance requirements

[TS 23.122, clause C.2]

6)	The VPLMN AMF to the UE: The VPLMN AMF shall transparently send the received steering of roaming information to the UE in the REGISTRATION ACCEPT message;

7)	If the steering of roaming information is received and the security check is successful, then:

a)	if the steering of roaming information contains a secured packet (see 3GPP TS 31.115 [67]), the ME shall upload the secured packet to the USIM using procedures in 3GPP TS 31.111 [41].

NOTE 1:	How the ME handles UICC responses and failures in communication between the ME and UICC is implementation specific and out of scope of this release of the specification.

When the ME receives a USAT REFRESH command qualifier (3GPP TS 31.111 [41]) of type "Steering of Roaming" it performs the procedure for steering of roaming in subclause 4.4.6 but it does not act as if timer T has expired; or

b)	if the steering of roaming information contains the list of preferred PLMN/access technology combinations, the ME shall replace the highest priority entries in the "Operator Controlled PLMN Selector with Access Technology" list stored in the ME with the received list of preferred PLMN/access technology combinations. Additionally, if the UDM has not requested an acknowledgement from the UE and if the UE has a list of available and allowable PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall either:

i)	release the current N1 NAS signalling connection locally, abort the ongoing registration procedure and then attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired. In this case, steps 8 to 11 are skipped; or

ii)	not release the current N1 NAS signalling connection locally, and complete the registration procedure as specified in 3GPP TS 24.501 [64], and skip steps 8 to 10;

8)	If:

a)	the UE's USIM is configured with indication that the UE is to receive the steering of roaming information due to initial registration in a VPLMN , but neither the list of preferred PLMN/access technology combinations nor the secured packet nor the HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided' is received in the REGISTRATION ACCEPT message, when the UE performs initial registration in a VPLMN or if the steering of roaming information is received but the security check is not successful; and

b)	the current chosen VPLMN is not contained in the list of "PLMNs where registration was aborted due to SOR", not part of "User Controlled PLMN Selector with Access Technology" list , the UE is not in manual mode of operation and the PDU session for emergency services is not pending to be activated;

then the UE shall release the current N1 NAS signalling connection locally, store the PLMN identity in the list of "PLMNs where registration was aborted due to SOR" and attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired, with an exception that the current PLMN is considered as lowest priority, and skip steps 9 to 12;

NOTE 2:	When the UE is in the manual mode of operation or the current chosen VPLMN is part of the "User Controlled PLMN Selector with Access Technology" list, the UE stays on the VPLMN.

9)	The UE to the VPLMN AMF: If the UDM has requested an acknowledgement from the UE and the UE verified that the steering of roaming information  has been provided by the HPLMN in step 7, the UE sends the REGISTRATION COMPLETE message to the serving AMF with an SOR transparent container including the UE acknowledgement;

10)	The AMF to the UDM: If an SOR transparent container is received in the REGISTRATION COMPLETE message, the AMF uses the Nudm_SDM_Info service operation to provide the received SOR transparent container to the UDM. If the HPLMN decided that the UE is to acknowledge the successful security check of the received steering of roaming information in step 4, the UDM verifies that the acknowledgement is provided by the UE as specified in 3GPP TS 33.501 [66];

11)	If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. If the N1 NAS signaling connection is not released after implementation dependent time, the UE may locally release the N1 signaling connection; and

12)	The UE deletes the list of "PLMNs where registration was aborted due to SOR".

The list of "PLMNs where registration was aborted due to SOR" is deleted when the UE is switched off.

6.3.1.1.3	Test Description

6.3.1.1.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.1.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.1.3.1-1.

Table 6.3.1.1.3.1–1: PLMN identifiers

| NR Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 11 | PLMN2 | 002 | 11 |
| 12 | PLMN3 | 003 | 21 |
| 13 | PLMN4 | 004 | 31 |

-	NR Cell 11 is set to ''Serving Cell”;

-	NR Cell 12 is set to ''Serving Cell'';

-     NR Cell 13 is set to “Serving Cell'';

-       System Information Combination NR-4 as defined in TS38.508 clause 4.4.3.1.3 is used in NR cells

UE:-

The UE is in Automatic PLMN selection mode.

-	USIM profile as defined in Table [FFS] in TS38.508-1 will be used.[FFS- Table 6.3.1.1.3.1-2 will be moved to TS38.508-1]

-    The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 Table 4.4A.2-0

Table 6.3.1.1.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 2 3 | PLMN4 PLMN3 PLMN2 Remaining defined entries use default values | NG-RAN NG-RAN NG-RAN |
| EFUST |  | Service n°127 is "available" |  |

6.3.1.1.4	Test procedure sequence

Table 6.3.1.1.4-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE | - |  | - | - |
| 2-14 | Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13 | - | - | - | - |
| 15 | The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception | <-- | RRC: DLInformationTransfer NAS: REGISTRATION ACCEPT | - | - |
| 16 | The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information | --> | RRC: ULInformationTransfer NAS: REGISTRATION COMPLETE | 1 | P |
| 17 | The SS transmits an RRCRelease message. | <-- | RRC: RRCRelease | - | - |
| 18 | Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 within the next 6min? | --> | RRC: RRCSetupRequest | 1 | P |
| 19 | Steps 2 – 6 of the generic test procedure in TS 38.508 Table 4.9.3-1 with condition MOBILITY are performed on NR Cell 11. NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” and the RRC connection is released | - | - | - | - |

6.3.1.1.5	Specific message contents

Table 6.3.1.1.5-1: REGISTRATION ACCEPT for NR Cell 13 (step 15, Table 6.3.1.1.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |

Table 6.3.1.1.5-2: SOR Transparent Container in REGISTRATION ACCEPT for NR Cell 13 (step 15, Table 6.3.1.1.4-1)

| Derivation Path: FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 0 | The list type is a secure packet |  |
| Acknowledgement (ACK) value | 1 | Acknowledgement requested |  |
| PLMN ID 1 | 2 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |

Table 6.3.1.1.5-3: REGISTRATION COMPLETE for NR Cell 13 (step 16, Table 6.3.1.1.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |

Table 6.3.1.1.5-4: SOR Transparent Container in REGISTRATION COMPLETE for NR Cell 13 (step 16, Table 6.3.1.1.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR data type | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |