3GPP TSG-RAN WG5 meeting#4 5G-NR Adhoc	R5-190254

# Singapore, Singapore, 21st – 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0375 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5G-NR Idle Mode TC 6.1.1.6 - PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of 5GS Idle Mode TC 38.523-1 6.1.1.6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new Test Case as part of Idle SA Option 2 WP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA Option 2 WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR xxxx |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

<Start of new section>

#### 6.1.1.6	PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer

# 6.1.1.6.1	Test Purpose (TP)

(1)

with { UE configured with “MinimumPeriodicSearchTimer“ }

ensure that {

when { UE camps on an NG-RAN VPLMN cell upon switch on and cells of a higher priority NG-RAN PLMN available }

then { the MS shall make the first attempt to access the HPLMN or an EHPLMN or higher priority PLMN after a period of at least 2 minutes }

}

(2)

with { UE configured with “MinimumPeriodicSearchTimer“, having made first attempt to higher priority PLMN and camped on an NG-RAN VPLMN cell and cells of a higher priority NG-RAN PLMN available }

ensure that {

when { the higher priority PLMN search timer T stored in the USIM  or the default value for T is less than the MinimumPeriodicSearchTimer  }

then { UE shall not use a value for T that is less than the MinimumPeriodicSearchTimer and selects and camps on a cell of the highest priority PLMN and attempts a location registration on the selected cell upon expiry of MinimumPeriodicSearchTimer }

}

# 6.1.1.6.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 23.122, clauses 4.4.3.1 and 4.4.3.3.1 Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause 4.4.3.1]

At switch on, or following recovery from lack of coverage, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see clause 4.5.2) attempts to perform a Location Registration.

...

If successful registration is achieved, the MS indicates the selected PLMN.

If there is no registered PLMN, or if registration is not possible due to the PLMN being unavailable or registration failure, the MS follows one of the following two procedures depending on its PLMN selection operating mode. At switch on, if the MS provides the optional feature of user preferred PLMN selection operating mode at switch on then this operating mode shall be used.

...

NOTE 1:	If successful registration is achieved, then the current serving PLMN becomes the registered PLMN and the MS does not store the previous registered PLMN for later use.

[TS 23.122, clause 4.4.3.3.1]

If the MS is in a VPLMN, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in subclause 4.4.3.1.1. In the case that the mobile has a stored "Equivalent PLMNs" list the mobile shall only select a PLMN if it is of a higher priority than those of the same country as the current serving PLMN which are stored in the "Equivalent PLMNs" list. For this purpose, a value T minutes may be stored in the SIM, T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value is stored in the SIM, a default value of 60 minutes is used for T.

If the MS is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [50] or 3GPP TS 31.102 [40], the MS shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer.

The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below:

a)	The periodic attempts shall only be performed in automatic mode when the MS is roaming, and not while the MS is attached for emergency bearer services or has a PDN connection for emergency bearer services;

b)	After switch on a period of at least 2 minutes and at most T minutes shall elapse before the first attempt is made;

# 6.1.1.6.3	Test Description

6.1.1.6.3.1	Pre-test conditions

System Simulator:

-	 3 NR cells: NR Cell 1, 13 and 14 as specified in TS 38.508-1[4] table 4.4.2-3 are configured as shown in Table 6.1.1.6.3.1–1.

Table 6.1.1.6.3.1-1: PLMN identifiers

| NR Cell | PLMN names | TA | PLMN |  | TAC | 5G-GUTI |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MCC | MNC |  |  |
| NR Cell 1 | PLMN4 | See TS 38.508-1[4] table 4.4.2-3/NR Cell 1 |  |  |  |  |
| NR Cell 13 | PLMN1 | See TS 38.508-1[4] table 4.4.2-3/NR Cell 13 |  |  |  |  |
| NR Cell 14 | PLMN2 | See TS 38.508-1[4] table 4.4.2-3/NR Cell 14 |  |  |  |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is configured with a value of MinimumPeriodicSearchTimer set to 7 minutes.

-	The UE is equipped with a USIM containing default value as per TS 38.508-1 (FFS) except for those listed in Table 6.1.1.6.3.1-2.

Table 6.1.1.6.3.1-2: USIM configuration (FFS)

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Preamble:

-	The UE performs a successful registration on PLMN1 after which it is switched OFF.

# 6.1.1.6.3.2	Test procedure sequence

Table 6.1.1.6.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” is applied at the point indicated in the Main behaviour description in Table 6.1.1.7.3.2-2. Cell powers are chosen for a serving cell and a non-suitable [Off] cell as defined in TS 38.508-1 table 6.2.2.1-3.

# Table 6.1.1.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 13 | NR Cell 14 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 | -88 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | -88 |  |

# Table 6.1.1.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.6.3.2-1. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3-21 | Steps 2 to 20 of the registration procedure described in TS 38.508-1[4] table 4.5.2.2-2 are performed on NR Cell 13 | - | - | - | - |
| 22 | Check: Does the UE send an RRCConnectionRequest on NR Cell 14 after 120 seconds, but before 420 seconds from power on? | --> | NR RRC: RRCConnectionRequest | 1 | P |
| 23-27 | Steps 2 to 6 of the mobility registration updating procedure described in TS 38.508-1[4] subclause 4.9.3 are performed on NR Cell 14. | - | - | - | - |
| 28 | Check: Is PLMN2 indicated as being selected by the UE? | - | - | 1 | P |
| 29 | SS adjusts cell levels according to row T2 of table 6.1.1.6.3.2-1. | - | - | - | - |
| 30 | Check: Does the UE send an RRCConnectionRequest on NR Cell 1 after 420 seconds from step 22? | --> | NR RRC: RRCConnectionRequest | 2 | P |
| 31-35 | Steps 2 to 6 of the mobility registration updating procedure described in TS 38.508-1[4] subclause 4.9.3 are performed on NR Cell 14. | - | - | - | - |
| 36 | Check: Is PLMN4 indicated as being selected by the UE? | - | - | 2 | P |
| Note: Timers in Steps 22 and 30 are derived from the value defined by the MinimumPeriodicSearchTimer. |  |  |  |  |  |

# 6.1.1.6.3.3	Specific message contents

None

<End of new section>