3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190317

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0480 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to time domain PUSCH allocation parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - k2 value for FR1 SCS30k With the current k2 value in Table 4.6.3-93: PUSCH-TimeDomainResourceAllocationList, UL/DL scheduling is not possible for UL/DL config(DL slot:7, SP slot:1, UL slot:2) which is specified in Table 4.6.3-142 for FR1 SCS 30kHz. [Current config] [Proposal] TS38.214 clasue 6.1.2.1.1 Table 6.1.2.1.1-5 defines the additional subcarrier spacing specific slot delay value for the first transmission of for MSG3 scheduled by the RAR. When the UE transmits a MSG3 scheduled by RAR, the Δ value specific to MSG3 subcarrier spacing µPUSCH is applied in addition to the K2 value. TS38.214 Table 6.1.2.1.1-5: Definition of value Δ µPUSCH; Δ. 0; 2. 1; 3. 2; 4. 3; 6. - PUSCH mapping type According to the notes in the UL RMC(A.2.2 38.521-1), PUSCH mapping typeA is suitable to be used for default value for RF tests. TS38.521-1 A.2.2 Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. |
| Summary of change: |  | Values in Table 4.6.3-93: PUSCH-TimeDomainResourceAllocationList are changed : - k2 prameter is changed from 7 to 4 - PUSCH mapping type is set to typeA for RF tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tests would not be carried out correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

#### –	PUSCH-TimeDomainResourceAllocationList

Table 4.6.3-93: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment |  | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF SEQUENCE { | 1 entry |  |  |  |  |
| k2 | 7 |  |  |  | FR1 |
|  | 4 |  |  |  | FR1_SCS30kHz |
|  | 6 |  |  |  | FR2 |
| mappingType | typeB |  | SIG |
|  | typeA |  | RF |
| startSymbolAndLength | 27 |  | Start symbol(S)=0, Length(L)=14 |  | FR1 |
|  | 14 |  | S=0, L=2 |  | FR2 |
| } |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| RF | Used for RF, performance and RRM testing |
| SIG | Used for protocol testing |
| FR1_SCS30kHz | FR1 is used under the test. SCS is set to 30kHz. |

## << End of changes >>