3GPP TSG-RAN5-Meeting #4-5G-NR Adhoc	R5-190350

# Singapore, Singapore,  21st – 25th January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0022 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on definition of DUT repositioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-1-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Additional clarification is necessary regarding a definition of DUT repositioning. An applicability of it to multiple measurement cases, relations with the quality of quiet zone MU during the over the air measurement are described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated descriptions of DUT repositioning and added occasions depending on measurement cases. Also added descriptions to clarify relationship among measurement cases, applicability of DUT repositioning and a variety of QoQZ MU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains with a definition and an applicability of DUT repositioning depending on measurement cases. Also relationship among measurement cases, applicability of DUT repositioning and a variety of QoQZ MU remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.1.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

B.2.1.26	DUT repositioning

This contributor describes the uncertainty due to a displacement of a DUT.  The DUT may need to be re-positioned to avoid forming its beam toward the support structure during the measurement, or if the battery runs low in charge.

Measurement cases which incorporate DUT repositioning during measurements, reasons to require the DUT repositioning, and a way to reposition the DUT are described below.

(1)	Tx/Rx Beam peak search

The DUT repositioning is needed to avoid forming an uplink beam from a UE towards a support structure of a positioner during the measurement. Also same objective applies to the Rx test case, to avoid forming a sensitivity direction towards the support structure. In those cases the DUT is rotated 180o during the measurement and the test is carried out twice to acquire characteristics of each hemisphere which is located at the opposite side from the support structure of positioner. Image of repositioning can be referred at Figure C.3-1 and C.3-2.

(2)	Tx/Rx spherical coverage measurement

Same reason with Tx/Rx Beam peak search applies to Tx/Rx spherical coverage measurement case. Also the same procedure to rotate a DUT applies to this measurement.

(3)	TRP measurement of spurious emission

For in-band TRP measurement case, since we can know a direction of beam peak, we can avoid forming the beam towards the support structure and thus we can carry out the TRP measurement without the repositioning, assuming that the power from the other side of peak beam direction is small enough. In a case of spurious emission measurement, since we cannot anticipate directions of emissions from a UE, there might be a case that the spurious emissions are pointing to multiple directions and one of the emissions might point to the direction of support structure of the positioner even though the UL beam is pointing away from the support structure. However also same with the in-band TRP measurement, spurious emission test can be carried out without repositioning. If we consider a shape of spurious emission other than harmonics of in-band signals, shape of the emission would be wide and thus the emission towards the support structure cannot be completely hided by the preventer. Also because the major spurious emissions can be assumed as harmonics of in-band signals coming from a same direction with peak beam, i.e. the direction of an antenna emitting UL signals, it can be positioned at the other side of support structure.

(4)	Re-start of measurement after a battery power outage

This case is just to restart a measurement once a battery power runs low and the UE is recharged. Thus the positioning of DUT is not changed.

For other test cases such as in-band TRP, EIRP, EIS do not require the DUT repositioning.

Relationship between the DUT positioning and applicable QoQZ MU value is also important to decide total measurement uncertainty for each test case. Relations for each case are described below. Note that the case of repositioning to re-charge the UE battery is omitted since this depends on other measurement cases.

(a)	Tx/Rx Beam peak search

QoQZ MU which is derived based on an evaluation result of hemisphere can be applied. Here the evaluation result of hemisphere means that the QoQZ evaluation data is obtained by orienting reference AUT only to hemisphere described at D.2.7.2 in TR 38.810 [13]

(b)	Tx/Rx spherical coverage measurement

Same with (a) above.

(c)	TRP measurement of spurious emission

QoQZ MU which is derived based on an evaluation result of full sphere can be applied. Here the evaluation result of full sphere means that the QoQZ evaluation data is obtained by orienting reference AUT to all the directions because of the reason in (3) above. Also due to the same reason, when we derive the QoQZ MU value with a full sphere, we need to include an evaluated data which is obtained by orientating a reference AUT towards a support structure of the positioner. (Usually the direction towards the support structure of the positioner is omitted from the calculation for in-band test case.)

(d)	EIRP/ EIS / TRP (In-band)

Same with (a) above.

Table B.2.1.26-1 shows the summary of assumptions regarding a relationship among cases, DUT repositioning and variety of QoQZ MU.

Table B.2.1.26-1: Relationship among measurement cases, DUT repositioning and variety of QoQZ MU

| Measurement cases | Applicability of repositioning and DUT direction | Variety of QoQZ MU (Orientation of ref. AUT) |
| --- | --- | --- |
| Tx/Rx Beam peak search | Applied (Rotated 180o) | Hemisphere |
| Tx/Rx spherical coverage measurement | Applied (Rotated 180o) | Hemisphere |
| TRP measurement of spurious emission | Not applied | Full sphere |
| EIRP/ EIS / TRP (In-band) | Not applied | Hemisphere |

<< End of changes >>