3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190448

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEXT PROPOSAL |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | N/A | rev | - | Current version: | 0.1.0 |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating FR1 MU for timing measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FR1 MU for timing measurements(both absolute and relative timing measurment) needs to be updated based on R5-190058 |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU for “Uplink signal transmit timing relative to downlink” is updated from 3Ts to 112Tc MU for “Relative transmit timing accuracy during UE timing adjustment” and “Timing Advance Adjustment accuracy “ is updated from 0.5Ts to 88Tc |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR1 MU for timing measurement remains inappropriate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Relating discussion paper : R5-190058 |  |  |  |  |  |  |  |  |

## << Unchanged sections skipped >>

Annex F (normative):
Measurement uncertainties and test tolerances

The requirements of this clause apply to all tests in the present document.

# F.1	Measurement uncertainties and test tolerances for FR1

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tigher measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc | dB | [±2.0] |  |
| Ratio of cell X signal / AWGN, Êsx / Noc | dB | ±0.3 | Same as in LTE |
| Fading profile uncertainty* | dB | ±TBD | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.101-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.101-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-2.

The maximum test system uncertainty for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.1.2-3.

Table F.1.1.2-2: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |

Table F.1.1.2-3 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |

## << End of change >>