3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190703

# Singapore, Singapore, 21st Jan 2019 – 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0344 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5GC TC - Initial registration / 5GS services / NSSAI handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of new NR NAS test case as per 5G-NR work plan Core specification source: Dec-18 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new NR NAS test case - Initial registration / 5GS services / NSSAI handling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact R5-190168r1: 1.) PDU session status IE added in REGISTRATION REQUEST AND REGISTRATION ACCEPT Message to transfer PDU session when UE is moving from non suitable cell to suitable cell and test case step number changed accordingly. R5-190703 is the final version of R5-190168r1 |  |  |  |  |  |  |  |  |

#### <Start>

#### 9.1.5.1.3 	Initial registration / 5GS services / NSSAI handling

9.1.5.1.3.1	Test Purpose (TP)

(1)

with { UE is switched off with a valid USIM inserted }

ensure that {

when { UE has configured NSSAI associated with HPLMN, configured NSSAI associated with a current PLMN with mapping to the Configured NSSAI for the HPLMN and UE is powered up or switched on }

then { UE transmits REGISTRATION REQUEST message with Requested NSSAI using the configured NSSAI associated with current PLMN and mapping of each S-NSSAI of the requested NSSAI to the S-NSSAIs of the configured NSSAI for the HPLMN }

}

(2)

with { UE is switched off with a valid USIM inserted }

ensure that {

when { UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a default configured NSSAI and UE is powered up or switched on }

then { UE transmits REGISTRATION REQUEST message with Requested NSSAI using the default configured NSSAI and also includes Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" }

}

(3)

with { UE is switched off with a valid USIM inserted }

ensure that {

when { UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no configured NSSAI not associated with a PLMN and UE is powered up or switched on }

then { UE transmits REGISTRATION REQUEST message and does not include a Requested NSSAI }

}

(4)

with { UE has sent a REGISTRATION REQUEST message including Requested NSSAI}

ensure that {

when { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject casue “S-NSSAI not available in the current PLMN” }

then { UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and not attempt to use the Rejected NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed }

}

(5)

with { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject casue “S-NSSAI not available in the current PLMN” }

ensure that {

when { UE has been switched off, then switched on }

then { UE shall delete the stored Rejected NSAAI and shall send the NSSAI in Requested NSSAI IE of the REGISTRATION REQUEST message as per the configured and Allowed NSSAI for current PLMN }

}

(6)

with { UE has sent a REGISTRATION REQUEST message including Requested NSSAI}

ensure that {

when { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject casue “S-NSSAI not available in the current registration area” }

then { UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination and not attempt to use the Rejected NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed }

}

(7)

with { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject casue “S-NSSAI not available in the current registration area” }

ensure that {

when { UE has been moved out of the current registration area }

then { UE shall delete the stored Rejected NSAAI for the current PLMN as well as registration area combination and shall send the NSSAI in Requested NSSAI IE of the REGISTRATION REQUEST message as per the configured and Allowed NSSAI for current PLMN }

}

(8)

with { UE has sent a REGISTRATION REQUEST message including requested NSSAI}

ensure that {

when { UE receives REGISTRATION ACCEPT message with allowed NSSAI }

then { UE shall replace any stored allowed NSSAI for the current PLMN with new allowed NSSAI for the current PLMN }

}

9.1.5.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.5.1.2.2 and 5.5.1.2.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501 clause 5.5.1.2.2]

5.5.1.2.1	General

This procedure can be used by a UE for initial registration for 5GS services.

…

The UE shall include the requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wants to register and shall include the mapping of the requested NSSAI which is the mapping of each S-NSSAI of the requested NSSAI to the S-NSSAI(s) of the HPLMN, if available, in the REGISTRATION REQUEST message. If the UE has allowed NSSAI or configured NSSAI for the current PLMN, the requested NSSAI shall be either:

a)	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN;

b)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

c)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and those are neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

If the UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a default configured NSSAI, the UE shall:

a)	include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the default configured NSSAI; and

b)	include the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" in the REGISTRATION REQUEST message.

If the UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no default configured NSSAI, the UE shall not include a requested NSSAI in the REGISTRATION message.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN, if the S-NSSAI is neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for the current PLMN.

NOTE 3:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 4:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

[TS 24.501 clause 5.5.1.2.4]

The AMF shall include the allowed NSSAI for the current PLMN and shall include the mapping of each S-NSSAI of the allowed NSSAI to the S-NSSAI(s) of the HPLMN contained in the requested NSSAI from the UE if available, in the REGISTRATION ACCEPT message if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the requested NSSAI but rejected by the network associated with rejection cause(s).

The AMF may include a new configured NSSAI for the current PLMN in the REGISTRATION ACCEPT message if:

a)	the REGISTRATION REQUEST message did not include the requested NSSAI;

b)	the REGISTRATION REQUEST message included the requested NSSAI containing an S-NSSAI that is not valid in the serving PLMN; or

c)	the REGISTRATION REQUEST message included the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI".

If a new configured NSSAI for the current PLMN is included in the REGISTRATION ACCEPT message, the AMF shall also include the mapping of the configured NSSAI for the current PLMN the S-NSSAI(s) of the to HPLMN if available in the REGISTRATION ACCEPT message. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The AMF shall include the Network slicing indication IE with the Network slicing subscription change indication set to "Network slicing subscription changed" in the REGISTRATION ACCEPT message if the UDM has indicated that the subscription data for network slicing has changed. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in the current PLMN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message or none of the requested NSSAI are present in the subscribed S-NSSAIs, and one or more subscribed S-NSSAIs (containing one or more S-NSSAIs each of which may be associated with a new S-NSSAI) marked as default are available, the AMF shall put the subscribed S-NSSAIs marked as default in the allowed NSSAI of the REGISTRATION ACCEPT message. The AMF shall determine a registration area such that all S-NSSAIs of the allowed NSSAI are available in the registration area.

9.1.5.1.3.3	Test description

9.1.5.1.3.3.1 	Pre-test conditions

System Simulator:

NGC Cell A belongs to Home PLMN and TAI-1;

NGC Cell G, NGC Cell I and NGC Cell C are configured according to table 6.3.2.2-1 in TS 38.508-1 [4]

NGC Cell G belongs to VPLMN, TAI-7 and set as a serving cell;

NGC Cell I belongs to another VPLMN and TAI-9;

NGC Cell C belongs to Home PLMN and TAI-3;

The cells may not be simultaneously activated.

UE:

UE is configured with NSSAI associated with HPLMN with SST value of 1,2.

UE is configured with NSSAI associated with VPLMN (NGC Cell G) with SST value of 4 and 5 and mapped Configured SST value of 1 and 2 accordingly.

UE is configured with default configured NSSAI with SST value of 1.

UE is configured with empty allowed NSSAI list associated with VPLMN (NGC Cell G and NGC Cell I).

The pre-configuration of configured and allowed NSSAI may be performed by MMI or AT command.

Preamble:

-	The UE is in state Switched OFF (state 0-A) according to TS 38.508-1 [4].

9.1.5.1.3.3.2	Test procedure sequence

Table 9.1.5.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 1 | P |
| 3-11 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 12 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 13-18 | Steps 15 to 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 19 | The SS configures NGC Cell G as a “Non-Suitable Off cell” and NGC Cell I as the “Serving cell”. | - | - | - | - |
| 20 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI on NGC Cell I? | --> | REGISTRATION REQUEST | 2 | P |
| 21-29 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 30 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 31 | Step 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 is performed. | - | - | - | - |
| 32 | Step 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 is performed. |  |  |  |  |
| 33 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 38 describes behaviour that depends on the UE capability | - | - | - | - |
| 34 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DEREGISTRATION REQUEST. | --> | DEREGISTRATION REQUEST | - | - |
| 35 | The SS configures NGC Cell I as a “Non-Suitable Off cell”. | - | - | - | - |
| 36 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| 37 | Configure the UE with no Allowed NSSAI for HPLMN, no Configured NSSAI for HPLMN and no default configured NSSAI. Note: The pre-configuration of configured and allowed NSSAI may be performed by MMI or AT command. | - | - | - | - |
| 38 | The UE is switched off. | - | - | - | - |
| 39 | The SS configures NGC Cell A as the “Serving cell”. | - | - | - | - |
| 40 | The UE is switched on. | - | - | - | - |
| 41 | Check: Does UE transmit a REGISTRATION REQUEST message not including Requested NSSAI? | --> | REGISTRATION REQUEST | 3 | P |
| 42-50 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 51 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Configured NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 52-57 | Steps 15 to 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 58 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 51 describes behaviour that depends on the UE capability | - | - | - | - |
| 59 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DEREGISTRATION REQUEST. | --> | DEREGISTRATION REQUEST | - | - |
| 60 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| 61 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 8 | P |
| 62-70 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 71 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Rejected NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 72-77 | Steps 15 to 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 78 | The SS configures NGC Cell A as a “Non-Suitable Off cell” and NGC Cell C as the “Serving cell”. | - | - | - | - |
| 79 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 4 | P |
| 80-88 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 89 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 90 | Step 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 is performed. | - | - | - | - |
| 91 | Step 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 is performed. | - | - | - | - |
| 92 | Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause “S-NSSAI not available in the current PLMN” using AT/MMI. | - | - | 4 | P |
| 93 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 79 describes behaviour that depends on the UE capability | - | - | - | - |
| 94 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DEREGISTRATION REQUEST. | --> | DEREGISTRATION REQUEST | - | - |
| 95 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| 96 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 5 | P |
| 97-105 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 106 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Rejected NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 107-112 | Steps 15 to 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 113 | Check: Is S-NSSAI=1 removed from the Rejected NSSAI list? | - | - | 5 | P |
| 114 | Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause “S-NSSAI not available in the current registration area” using AT/MMI. | - | - | 6 | P |
| 115 | The SS configures NGC Cell C as the "non-Suitable cell" and NGC Cell A as the "Serving cell". | - | - | - | - |
| 116 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 7 | P |
| 117-125 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 126 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 127 | Step 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 is performed. | - | - | - | - |
| 128 | Step 20 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 is performed. | - | - | - | - |
| 129 | Check: Is S-NSSAI=2 removed from the Rejected NSSAI list? | - | - | 7 | P |

9.1.5.1.3.3.3	Specific message contents

Table 9.1.5.1.3.3.3-1: REGISTRATION REQUEST (step 2, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000100’B | 4 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000001’B | 1 |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000101’B | 5 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000010’B | 2 |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: UE may send subset of Configured NSSAI associated with PLMN of NGC Cell G. Allowed NSSAI list associated with PLMN of NGC cell G should be empty as per the Pre-test conditions. |  |  |  |  |

Table 9.1.5.1.3.3.3-2: REGISTRATION ACCEPT (step 12, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000100’B | 4 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000001’B | 1 |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000101’B | 5 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000010’B | 2 |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: NW will send allowed NSSAI based on the Reuqetsed NSSAI sent by UE in step 2. |  |  |  |  |

Table 9.1.5.1.3.3.3-3: REGISTRATION REQUEST (step 20, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘010’B | mobility registration updating |  |
| Requested NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| PDU session status |  | PDU session IDs | PDU session IDs of the ACTIVE PDU sessions to be transferred to a new cell. |  |
| Network slicing indication |  |  |  |  |
| Default configured NSSAI indication |  | 1 | Requested NSSAI created from default configured NSSAI |  |

Table 9.1.5.1.3.3.3-4: REGISTRATION ACCEPT (step 30, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| PDU session status |  | PDU session IDs | PDU session IDs of the ACTIVE PDU sessions to be transferred to a new cell. Same value as sent by the UE in step 20. |  |

Table 9.1.5.1.3.3.3-5: REGISTRATION REQUEST (step 41, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  | Not Present |  |  |

Table 9.1.5.1.3.3.3-6: REGISTRATION ACCEPT (step 51, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.5.1.3.3.3-7: REGISTRATION REQUEST (step 61, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: S-NSSAI =1 will be always included by the UE but may include S-NSSAI =2 from Configured NSSAI list associated with PLMN of NCG Cell A. |  |  |  |  |

Table 9.1.5.1.3.3.3-8: REGISTRATION ACCEPT (step 71, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Rejected NSSAI |  |  |  |  |
| Rejected S-NSSAI-1 |  |  | Rejected S-NSSAI value 1 |  |
| Cause value |  | ‘0000’B | S-NSSAI not available in the current PLMN |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Note: if UE has requested only S-NSSAI =1 in step 61 and S-NSSAI =1 is added in the Rejected NSSAI list by the NW then AMF/NW can include default subscribed S-NSSAI if none of the requested NSSAI are present in the subscribed S-NSSAI, see TS 24.501 sub-caluse 5.5.1.2.4. |  |  |  |  |

Table 9.1.5.1.3.3.3-9: REGISTRATION REQUEST (step 79, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘011’B | mobility registration updating |  |
| Requested NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| PDU session status |  | PDU session IDs | PDU session IDs of the ACTIVE PDU sessions to be transferred to a new cell. |  |

Table 9.1.5.1.3.3.3-10: REGISTRATION ACCEPT (step 89, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| PDU session status |  | PDU session IDs | PDU session IDs of the ACTIVE PDU sessions to be transferred to a new cell. Same value as sent by the UE in step 79. |  |

Table 9.1.5.1.3.3.3-11: REGISTRATION REQUEST (step 96, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: S-NSSAI =2 will be always included by the UE but may include S-NSSAI =1 from Configured NSSAI list associated with PLMN of NCG Cell C. |  |  |  |  |

Table 9.1.5.1.3.3.3-12: REGISTRATION ACCEPT (step 106, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| TAI list |  |  |  |  |
| Type of list |  | ‘01’B | list of TACs belonging to one PLMN, with consecutive TAC values |  |
| Number of elements |  | ‘00001’B | 2 Elements |  |
| TAC |  | PLMN =MCC/MNC stored in EFIMSI TAC 1 = 2 | TAI2, TAI 3 |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Rejected NSSAI |  |  |  |  |
| Rejected S-NSSAI-1 |  |  | Rejected S-NSSAI value 1 |  |
| Cause value |  | ‘0001’B | S-NSSAI not available in the current registration area |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Note: if UE has requested only S-NSSAI =2 in step 96 and S-NSSAI =2 is added in the Rejected NSSAI list by the NW then AMF/NW can include default subscribed S-NSSAI if none of the requested NSSAI are present in the subscribed S-NSSAI, see TS 24.501 sub-caluse 5.5.1.2.4. |  |  |  |  |

Table 9.1.5.1.3.3.3-15: REGISTRATION REQUEST (step 116, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘010’B | mobility registration updating |  |
| Requested NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| PDU session status |  | PDU session IDs | PDU session IDs of the ACTIVE PDU sessions to be transferred to a new cell. |  |
| Note: S-NSSAI =1 will be always included by the UE but may include S-NSSAI =2 from Configured NSSAI list associated with PLMN of NCG Cell A. |  |  |  |  |

Table 9.1.5.1.3.3.3-16: REGISTRATION ACCEPT (step 126, Table 9.1.5.1.3.3.2-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| PDU session status |  | PDU session IDs | PDU session IDs of the ACTIVE PDU sessions to be transferred to a new cell. Same value as sent by the UE in step 116. |  |
| Note: NW will send allowed NSSAI based on the Reuqetsed NSSAI sent by UE in step 116. |  |  |  |  |

#### <End>