3GPP TSG RAN WG5 #5-5G-NR Adhoc	R5-192984

Xi’an, China, 9th – 12th April 2019

Source:		Rohde & Schwarz

Title:	On the SNR for FR2 TRx test cases

Agenda Item:		4.1.16

Document for:	Endorsement

# Introduction

At RAN5 #82, estimations for the SNR have been discussed [1,2] and a way forward [5] has been created to summarize the results for MOP and the assumptions for the SNR analysis. In this document, we present the results of our analysis based on the common assumptions.

# Discussion

A detailed description of the total SNRtotal based on the measurement of total component power for both polarizations can be found in reference [1].

In Table 1, we present our results for SNRtotal under the assumption that the UE is allowed to transmit a maximum peak EIRP of 43 dBm.

Table 1: SNRtotal

| Test case | Frequency (GHz) | Estimated SNRtotal(dB) | Requirement (dBm) | Impact of Noise w/o relaxation (dB) | Required Relaxation |
| --- | --- | --- | --- | --- | --- |
| MOP Min Peak EIRP | 23.45 ≤ f ≤ 32.125 | 23.0 | 20.7 (= 22.4 – 1.7) | 0.022 | no |
|  | 32.125 < f ≤ 40.8 | 18.0 | 18.9 (= 20.6 – 1.7) | 0.069 | no |
| MOP Spherical coverage | 23.45 ≤ f ≤ 32.125 | [12.1]1 | 9.75 (= 11.5 – 1.75) | [0.26]1 | no |
|  | 32.125 < f ≤ 40.8 | [6.7]1 [12.7]1,2 | 7.6 (= 8 – 0.4) | [0.84]1 [0.23]1,2 | no |
| MOP TRP | 23.45 ≤ f ≤ 32.125 | 25.3 | 23 | 0.013 | no |
|  | 32.125 < f ≤ 40.8 | 22.1 | 23 | 0.027 | no |
| Transmit OFF Power (TRP) | 23.45 ≤ f ≤ 32.125 | -32.7 | -35 | 32.7 | do not test (relaxation of at least 42.7 dB if tested) |
|  | 32.125 < f ≤ 40.8 | -35.9 | -35 | 35.9 | do not test (relaxation of at least 45.9 dB if tested) |
| SEM (TRP) | 23.45 ≤ f ≤ 32.125 | 15.4 | -13 | 0.13 | no |
|  | 32.125 < f ≤ 40.8 | 12.1 | -13 | 0.26 | no |
| ACLR AC (TRP) | 23.45 ≤ f ≤ 32.125 | -10.0 | -12.3 (= 20.7 – 16 – 17) | 10.4 | yes |
|  | 32.125 < f ≤ 40.8 | -14.0 | -13.1 (= 18.9 – 16 – 16) | 14.22 | yes |
| ACLR wanted signal (TRP) | 23.45 ≤ f ≤ 32.125 | 7.0 | 4.7 (= 20.7 – 16) | 1.05 | yes |
|  | 32.125 < f ≤ 40.8 | 2.0 | 2.9 (= 18.9 – 16) | 3.00 | yes |
| Spurious Emissions for UE co-existence | work in progress |  |  |  |  |
| Transmitter Spurious Emissions | work in progress |  |  |  |  |
| Receiver Spurious Emissions | see Ref. [4] |  |  |  |  |
| NOTE 1: Rough estimate based on the 50%-tile power level requirement. A more rigorous analysis requires input from UE vendors on an acceptable CDF. NOTE 2: Assumed 100 MHz channel bandwidth |  |  |  |  |  |

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements 6 GHz – 80 GHz for spurious emissions measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, SNR estimations for low PSD test cases have been provided and the impact on the systematic uncertainty has been analyzed.

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise.

# References

- R5-192125, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #82, February 2019

- R5-191794, SNR estimation update for FR2 priority 1 and 2 TRx TCs, Anritsu, 3GPP TSG RAN WG5 Meeting #82, February 2019

- R5-192663, WF on Noise Impact on RF Measurements for FR2, Keysight Technologies, Anritsu, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #82, February 2019

- R5-192982, On testability of Rx spurious emission test case, Rohde & Schwarz, 3GPP TSG RAN WG5 #5-5G-NR Adhoc, April 2019