3GPP TSG-RAN5 NR#5 Meeting	R5-193104

# Xian, China, 7th – 10th April. 2019

Title:							SNR estimation update for FR2 priority 1 and 2 TRx TCs

Source:			Anritsu

Agenda Item:			4.1.16

Document for:			Endorsement

- Discussion

# SNR estimation is provided in Table 1.

# There is no change from the previous document [8] other than followings.

- Align the definition of SNR with [9] (-3dB for SNR for total component, ΔSNR=10log10(1+1/SNR))

- ACLR SNR is updated based on the assumption in WF [10]

- Spherical coverage UL power level assumption: multi-band relaxation is applied as per agreement in WF[7].

# Power level for spherical coverage is set as FFS in WF [10]. We are thinking we can use 50%-tile requirement level(minus multi band relaxation) as is (Ref:[11]), then no change from [8] in Table1.

Table 1: SNR for Priority 1 and 2 Tx test cases

| Test case | Applicable Frequency Range [GHz] | UL signal level [dBm] | Estimated SNR for total component [dB] | Proposed ΔSNR ΔSNR=10log10(1+1/SNR), SNR in linear and defined for total component power | Proposed relaxation of test requirement |
| --- | --- | --- | --- | --- | --- |
| MOP (Min Peak EIRP, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 20.7dBm/ChBW (22.4 – 1.7) | 17.6+ ChBW | 0.1 | 0 |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 18.9dBm/ChBW (20.6 – 1.7) | 11.8+ ChBW | 0.28 | 0 |
| MOP (TRP, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 23dBm/ChBW | 7.6 + ChBW (EIRPMAX> 33dBm) 22.0+ ChBW (EIRPMAX≤ 33dBm) | 0.7 0.1 | 0 0 |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 23dBm/ChBW | -4.2 + ChBW (EIRPMAX > 33dBm) 12.0+ ChBW (EIRPMAX ≤ 33dBm) | 0.41-0.98 0.27 | 10.2-14.2 - ChBW 0 |
| MOP (Spherical Coverage, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 9.75dBm/ChBW (11.5-1.75) | -5.65 + ChBW (EIRPMAX > 33dBm) 8.75 + ChBW (EIRPMAX ≤ 33dBm) | 0.41-0.98 0.37 | 11.65-15.65 - ChBW 0 |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 7.6dBm/ChBW (8.0-0.4) | -19.6 + ChBW (EIRPMAX > 33dBm) 5.9 + ChBW (EIRPMAX ≤ 33dBm) | 0.41-0.98 1.0 | 25.6-29.6 - ChBW 0 |
| Off power | 23.45 GHz ≤ f ≤ 32.125 GHz | -35dBm/ChBW | -18.0 + ChBW | 0.41 | 28.0 - ChBW |
|  | 32.125 GHz < f ≤ 40.8 GHz |  | -26.1 + ChBW | 0.41 | 36.1 - ChBW |
| SEM | 23.45 GHz ≤ f ≤ 32.125 GHz | -13dBm/1MHz | 8.1(EIRPMAX > 33dBm) 17.8(EIRPMAX ≤ 33dBm) | 0.41 0.1 | 1.9 0 |
|  | 32.125 GHz < f ≤ 40.8 GHz |  | 1.2(EIRPMAX > 33dBm) 19.9 (EIRPMAX ≤ 33dBm) | 0.41 0.1 | 8.8 0 |
| ACLR | 23.45 GHz ≤ f ≤ 32.125 GHz | CP: 22.4-1.7-16 = 4.7dBm/ChBW (NOTE3) | 2.7 + ChBW(EIRPMAX > 33dBm) 12.5 + ChBW (EIRPMAX ≤ 33dBm) | 0.41 0.24 | Depends on assumed TRP N/A |
|  |  | ACP : 22.4-1.7-16 -17 = -12.3/ChBW (NOTE3) | -14.3 + ChBW(EIRPMAX > 33dBm) -4.5 + ChBW (EIRPMAX ≤ 33dBm) | 0.41 0.41 | Depends on assumed TRP |
|  | 32.125 GHz < f ≤ 40.8 GHz | CP : 20.6-1.7-16=2.9dBm/ChBW (NOTE3) | -5.9 + ChBW(EIRPMAX > 33dBm) 12.7+ ChBW (EIRPMAX ≤ 33dBm) | 0.41 0.23 | Depends on assumed TRP N/A |
|  |  | ACP : 20.6-1.7-16-16 = -13.1/ChBW(NOTE3) | -21.9 + ChBW(EIRPMAX > 33dBm) -3.3 + ChBW (EIRPMAX ≤ 33dBm) | 0.41 0.41 | Depends on assumed TRP |
| Rx spurious | 6 GHz ≤ f ≤ 20 GHz | -47dBm/1MHz | 3.3 | 0.41 | 6.7 |
|  | 20 GHz < f ≤ 40 GHz |  | -6.2 | 0.41 | 16.2 |
|  | 40 GHz < f ≤ 80 GHz |  | -19.6 | 0.41 | 29.6 |
| Tx spurious | 6 GHz ≤ f ≤ 20 GHz | -13dBm/1MHz | ≥ 17 | 0.1 | 0 |
|  | 20GHz ≤ f ≤ 80 GHz |  | ≥ 10 | 0.41 | 0 |
| Tx Spurious Co-existence | WIP | WIP | WIP | WIP | WIP |
|  | WIP | WIP | WIP | WIP | WIP |
| NOTE ChBW = 10log10(400MHz/ChBW), 9.03dB for ChBW=50MHz. NOTE 2 : According to the agreement in [7], the SNR estimation is only valid between 23.45 and 30.3 GHz. SNR needs to be revisited once the new band and its core requirement is introduced between 30.3GHz and 32.125GHz. NOTE 3 : CP : Min Peak EIRP – Multi-band relaxation – Directivity(2x8 array), ACP : CP – ACLR NOTE 4 : Different SNR from different condition of maximum EIRP came from the LNA availability . 14dB PAPR is assumed in this estimation to judge the LNA availability. |  |  |  |  |  |

Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43 dBm – Pathloss tolerable (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

# All the SNR is provided based on the “single” test setup.

# Proposal 1 : Use the values in Table 1 to determination of MU and relaxation amount

# 2.	Conclusion

Proposal 1 : Use the values in Table 1 to determination of MU and relaxation amount

# 3.	References

[1] R5-185806, “Testability issue of low PSD test cases in FR2 (2)”, Anritsu, RAN5 NR#3 AdHoc

[2] R5-187273, “Consideration on MU and TT for FR2 low PSD tests”, Anritsu, RAN5#80,

[3] R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#81

[4] R5-190446, “On the power level to derive SNR for FR2 TRx tests”, Anritsu, RAN5 NR#4 AdHoc

[5] R5-190460, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5 NR#4 AdHoc

[6] R5-190601, “On relaxation of test requirements for Low and High PSD test cases”, Keysight, RAN5 NR#4 AdHoc

[7] R5-190980, “Way forward for SNR estimation assumptions”, Anritsu/Keysight/R&S, RAN5 NR#4 AdHoc

[8] R5-191794, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5#82

[9] R5-192125, “On the SNR for FR2 TRx test cases”, Rohde & Schwarz, RAN5#82

[10] R5-192663, “WF on Noise Impact on RF Measurements for FR2”, Keysight/Anritsu/R&S, RAN5#82

[11] R5-193105, “On the impact from noise for EIRP spherical coverage”, Anritsu, RAN5 NR#5 AdHoc