3GPP TSG-RAN5 NR#5 AdHoc	R5-193105

# Xian, China , 8th  – 12th April. 2019

Title:							On the impact from noise for EIRP spherical coverage

Source:			Anritsu

Agenda Item:			4.1.16

Document for:			Endorsement

# 1.	Introduction

In the document of SNR estimation in [1] and [2], the SNR estimation is given for 50%-tile power level as is. In the RAN5#82 meeting, the question raised that how the noise impacts depending on the statistics of CDF curve, and it is set as FFS item in WF[1].  This paper provides the analysis for this topic.

# 2.	Discussion

# 2.1.	Impact from noise to EIRP CDF curve

Here we consider the noise impacts EIRP CDF curve for constant density grid and constant step size grid respectively.

#### Constant density grid

Assume the ideal AWGN noise and infinite mean time(or enough long time obtaining the stable measured power of noise), EIRP measured at grid #i is given by.

EIRPmeasi=EIRPθi+N+EIRPφi+N=EIRPNoNoisei+2N

Where N is the noise floor of the test system, and is constant regardless of the measurement grid and polarization. It is clear that the impact from noise always works as constant power offset which is same for all the grids. Hence, the noise does not change the order of the measured EIRP values, i.e. if two EIRPs obtained in noiseless equipment are  EIRPNoNoisei < EIRPNoNoisej, then the corresponding measurement results measured with equipment with some noise are always EIRPmeasi < EIRPmeasj. This means that CDF curve is just shifted by 10log10(1+1/SNR) [dB] towards right side.

Observation 1 : For constant density grid, the impact from noise always works as constant power offset which is same for all the grids. Hence, the noise does not change the order of the measured EIRP values. This means that CDF curve is just shifted by 10log10(1+1/SNR) [dB] towards right side for all the percentile points.

#### Constant step size grid using sin(theta) scaling

As written in TS 38.521-2, Annex M, the sin(theta) scaling needs to be used to derive CDF of EIRP when constant step size grid is used.

For constant step size measurement grids, the CDF analyses require the PDFs to be scaled by sin(theta).

This is because the sampled EIRP on constant step size grid represent the EIRP of different area size over the whole sphere (π2NMsinθi if the area of whole sphere is 1), then the CDF(or PDF) needs to be scaled by its area size to emulate the sampling on equal area grid(e.g. constant density grid). We here need to put stress here that we do not modify the measured EIRP value, but we modify (scale) the probability density, and that scaling only depends on the grid position θ.

Observation 2: For constant step size grid, we do not modify the measured EIRP value, but we modify (scale) the probability density, and that scaling only depends on the grid positionθ.

Observation 2 means that the magnitude relationship between sampled data with and without noise does not change, which is same as constant density grid. This means that CDF curve is just shifted by 10log10(1+1/SNR) [dB] towards right side even for constant step size grid.

Observation 3 : Even for constant step size grid, CDF curve is just shifted by 10log10(1+1/SNR) [dB] towards right side for all the percentile points.

# 2.2.	Simulation of EIRP CDF with Noise

Here we confirm observation 1 and observation 3 using simulation. EIRP CDF curve with and without test system noise can be simulated by assuming antenna array model and beam forming model. We conducted a simulation using the same antenna array and beam forming model as used for the study of measurement grid for spherical coverage in TR38.810.

-	Two 8x2 antenna arrays are integrated in the UE for the spherical coverage analyses

-	The implementation loss for the antenna near the front is 5dB less than that for the antenna near the back

-	For Beam Steering Assumptions

-	In the xz plane, 45o beam steering granularity (from 45o to 135o) has been used

-	In the xy plane, 22.5o beam steering granularity (from -90o to 90o) has been used

Figure G.3.1-1:  Illustration of the two antenna arrays integrated in the UE.

Figure 2 shows the simulation result of EIRP CDF curves with and without test system noise. Noise level of 10dBm is assumed in this simulation. As seen, the CDF curve is just shifted towards right direction by 10log10(1+1/SNRlin) dB.

Figure 2　Example of spherical coverage simulation with and without noise
(266 point constant step size grid)

Note that in figure 2, only the one realization of random relative orientation between the simulated UE and the respective measurement grid is shown. For other realization the conclusion is the same, and all the CDF curve realization will be shifted by 10log10(1+1/SNRlin) dB. Table 1 shows the result of mean and STD of 50%-tile EIRPs.
NOTE : To save the time 10 realization for @1deg and 1000 realization for @15deg is used for random relative orientation.

Table 1 50%-tile value MU result

| Grid | Noise [dBm] | 50%-tile Mean [dB] | 50%-tile STD [dB] |
| --- | --- | --- | --- |
| Constant step size @1deg | -inf | 15.921 | 0.0020 |
|  | 10 (SNR=5.921dB@50%-tile) | 16.910 | 0.0016 |
| Constant step size @ 15 deg | -inf | 15.923 | 0.138 |
|  | 10 (SNR=5.923dB@50%-tile) | 16.912 | 0.110 |

The mean power shift matches with 10log10(1+1/1015.921-1010)=0.989 dB. Hence not only each realization, but also statistically, the mean power shift is 10log10(1+1/SNRlin).

# 2.3.	Power level used for evaluation of “impact from noise” and test limit issue

Observations 1 and 3, and experiments in section 2.2 indicate that  it is enough to evaluate the SNR for the core requirement level(minus permitted relaxation) regardless of the CDF curve shape, measurement grids for the purpose of test limit evaluation.

Proposal 1 : Use core requirement level(50%-tile value for PC3) minus multi-band relaxation for the evaluation of “impact from noise” and test limit issue.

# 3.	Conclusion

Observation 1 : For constant density grid, the impact from noise always works as constant power offset which is same for all the grids. Hence, the noise does not change the order of the measured EIRP values. This means that CDF curve is just shifted by 10log10(1+1/SNR) [dB] towards right side for all the percentile points.

Observation 2: For constant step size grid, we do not modify the measured EIRP value, but we modify (scale) the probability density, and that scaling only depends on the grid position θ.

Observation 3 : Even for constant step size grid,  CDF curve is just shifted by 10log10(1+1/SNR) [dB] towards right side for all the percentile points.

Proposal 1 : Use core requirement level(50%-tile value for PC3) minus multi-band relaxation for the evaluation of “impact from noise” and test limit issue.

# 5.	References

[1] R5-185806, “Testability issue of low PSD test cases in FR2 (2)”, Anritsu, RAN5 NR#3 AdHoc

[2] R5-187273, “Consideration on MU and TT for FR2 low PSD tests”, Anritsu, RAN5#81,

[3] R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#82

[4] R5-190446, “On the power level to derive SNR for FR2 TRx tests”, Anritsu, RAN5 NR#4 AdHoc

[5] R5-190460, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5 NR#4 AdHoc

[6] R5-190601, “On relaxation of test requirements for Low and High PSD test cases”, Keysight, RAN5 NR#4 AdHoc

[7] R5-190980, “Way forward for SNR estimation assumptions”, Anritsu/Keysight/R&S, RAN5 NR#4 AdHoc