3GPP TSG-RAN5 Meeting #82	R5-193189

# Athens, Greece, 25th Feb 2019 – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 0285 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing Wideband Intermodulation for EN-DC including FR1 (3 CCs) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Wideband Intermodulation for EN-DC including FR1 (3 CCs) is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing Wideband Intermodulation for EN-DC including FR1 (3 CCs) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case remains missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<Unchaged sections skipped>>

#### 7.8B.2.6	Wideband Intermodulation for EN-DC including FR1 (3 CCs)

Editor’s Note : This test case is incomplete. The following aspects are either missing or not yet determined:

- Test procedure is TBD

- Message contents is TBD

- Test reqreuiement is TBD

- Test applicability in 38.522 is missing

- MU/TT is TBD

##### 7.8B.2.6.1	Test Purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

##### 7.8B.2.6.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC within FR1 (3 CCs).

##### 7.8B.2.6.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.

##### 7.8B.2.6.4	Test Description

7.8B.2.6.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 7.8B.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.8B.2.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS38.508-1 [6] subclause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  |  |  |  |  |  |  | Refer to "NRB” column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1[8] |  |  |  |  |  |  |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | CC (NOTE1) |  | Band |  | SCS | NRB |  |  | Range/ Wgap |  | UL MOD |  | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-nYA-nZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | NOTE2 |  |  | NOTE2 |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | PCC(S) |  | As per CA_nXA-nZA in 7.8A.2.1 of 38.521-1 [1] |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SCC1(S) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-nYC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | NOTE2 |  |  | NOTE2 |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | PCC(S) |  | As per CA_nYC in 7.8A.2.1 of 38.521-1 [1] |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SCC1(S) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-XA-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | NOTE2 |  |  | NOTE2 |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | SCC1(M) |  | X3 |  | N/A | N/A |  |  | N/A |  | N/A |  | N/A | N/A | N/A |
|  |  | PCC(S) |  | As per 7.8.2.1 of 38.521-1[1] |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | NOTE2 |  |  | NOTE2 |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | SCC1(M) |  | X3 |  | N/A | N/A |  |  | N/A |  | N/A |  | N/A | N/A | N/A |
|  |  | PCC(S) |  | As per 7.8.2.1 of 38.521-1[1] |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA-nZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | NOTE2 |  |  | NOTE2 |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | SCC1(M) |  | Y3 |  | N/A | N/A |  |  | N/A |  | N/A |  | N/A | N/A | N/A |
|  |  | PCC(S) |  | As per 7.8.2.1 of 38.521-1[1] |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC-nXA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | Highest NRB_agg |  |  | TBD |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | SCC1(M) |  | X2 |  | N/A |  |  |  | TBD |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | PCC(S) |  | nX |  |  |  |  |  | TBD |  |  |  |  |  |  |
| Default Test Settings for a DC_(n)XCA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X |  | N/A | Lowest NRB_agg |  |  | Mid/CC1 |  | QPSK |  | QPSK | REFSENS | All RBs |
|  |  | SCC1(M) |  | X |  | N/A |  |  |  | Mid/CC2 |  | QPSK |  | QPSK | N/A | All RBs |
|  |  | PCC(S) |  | nX |  | Highest |  |  |  | Mid/CC1 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | REFSENS | All RBs |
| 2 |  | PCC(M) |  | X |  | N/A | Highest NRB_agg |  |  | Mid/CC1 |  | QPSK |  | QPSK | REFSENS | All RBs |
|  |  | SCC1(M) |  | X |  | N/A |  |  |  | Mid/CC2 |  | QPSK |  | QPSK | N/A | All RBs |
|  |  | PCC(S) |  | nX |  | Highest |  |  |  | Mid/CC1 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nXA-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X2 |  | N/A | Highest NRB_agg |  |  | High with maxWGap |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | PCC(S) |  | nX |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SCC1(S) |  | nY |  |  |  | Highest |  | Mid |  |  |  |  |  |  |
| 2 |  | PCC(M) |  | X2 |  | N/A | Highest NRB_agg |  |  | High with maxWGap |  | NOTE2 |  | NOTE2 | NOTE2 | NOTE2 |
|  |  | SCC1(S) |  | nX |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC(S) |  | nY |  |  |  | Highest |  | Mid |  |  |  |  |  |  |
| Default Test Settings for a DC_YA-(n)XAA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | PCC(M) |  | X |  | N/A | Lowest NRB_agg |  |  | Mid/CC1 |  | QPSK |  | QPSK | REFSENS | All RBs |
|  |  | PCC(S) |  | nX |  | Highest |  |  |  | Mid/CC1 |  | QPSK |  | QPSK | REFSENS | All RBs |
|  |  | SCC1(M) |  | Y3 |  | N/A | N/A |  |  | N/A |  | N/A |  | N/A | N/A | N/A |
| 2 |  | PCC(M) |  | X |  | N/A | Highest NRB_agg |  |  | Mid/CC1 |  | QPSK |  | QPSK | REFSENS | All RBs |
|  |  | PCC(S) |  | nX |  | Highest |  |  |  | Mid/CC1 |  | QPSK |  | QPSK | N/A | All RBs |
|  |  | SCC1(M) |  | Y3 |  | N/A | N/A |  |  | N/A |  | N/A |  | N/A | N/A | N/A |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: Anchor agnostic approach applies. Configurations as per section 4.6. NOTE 3: Note configured NOTE 4: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 5: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.8B.2.6.4.2	Test procedure

TBD

7.8B.2.6.4.3	Message contents

TBD

##### 7.8B.2.6.5	Test Requirement

TBD

#### 7.8B.2.7	Wideband Intermodulation for EN-DC including FR1 (4 CCs)

## <<End of change>>