3GPP TSG-RAN5 Meeting #100	R5-233848

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0314 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - R5-233492 has introduced pre-established session functionality for MCData with a NOTE 1 in Table 5.5.3.1.1-3 but the note ist still missing at the end of the table |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NOTE 1 added to Table 5.5.3.1.1-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.5.3.1.1	SDP Message from the UE

##### -	MCPTT

Table 5.5.3.1.1-1: SDP Message from the UE for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media description for audio | a= line attribute = key-mgmt (NOTE 2) |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Session attribute | optional (NOTE 3) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| "RS" | any allowed value if present |  | RFC 3556 [113] |  |
| "RR" | any allowed value if present |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec NOTE: In addition to the parameters below the UE may provide further parameters |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media attribute | optional | a= line attribute =sendrecv Indicates send and receive mode being activated |  |  |
| sendrecv |  | Parameter has no value |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in speech media's c= line or in the session's c= line if the speech media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for speech |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in speech media's c= line or in the session's c= line if the speech media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for speech incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Media description[2] |  | Media description for media control |  | NOT WITHOUT_FLOORCONTROL OR PRE_ESTABLISHED_SESSION |
| media description |  | m= line media = application SDP media-level section for a media-control entity (NOTE 2) |  |  |
| media | "application" |  |  |  |
| port | any allowed value | The port for the media-control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCPTT" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER AND NOT WITHOUT_FLOORCONTROL |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  | present | Parameter has no value |  | pc_MCPTT_FloorRequestQueueing |
| mc_priority | any allowed value | Any integer value in the range of 1..255 | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 | INITIAL_SDP_OFFER |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 | IMPLICIT_GRANT_REQUESTED |
| mc_ssrc | any value if present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_no_floor_ctrl | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| format specific parameters |  |  |  | SDP_ANSWER AND NOT WITHOUT_FLOORCONTROL |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  | present | Parameter has no value |  | pc_MCPTT_FloorRequestQueueing |
| mc_priority | same value as in the offer |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_granted | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| format specific parameters |  |  |  | WITHOUT_FLOORCONTROL |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_priority | not present |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 14.3.3 |  |
| mc_granted | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_no_floor_ctrl | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Media Control messages |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in application media's c= line or in the session's c= line if the application media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for application |  |  |  |
| cand-type | "host" |  |  |  |
| NOTE 1: If "ssrc" media attributes are included, then at least one "ssrc=" line shall contain a "cname" source attribute according to RFC 5576 [116] clause 6.1. NOTE 2: Even though there is no clarity in core specs it is assumed that a key-mgmt attribute at session level does not affect the media control security, i.e. the key-mgmt attribute is not applicable for the "application" media description for which still the CSK is used as security key. This is in contrast to RFC 4566 [27] clause 5 saying "In general, session-level values are the default for all media unless overridden by an equivalent media-level value." NOTE 3: If the UE is configured as lite implementation according to RFC 5245 [115], it shall include "a=ice-lite" session-level attribute; nevertheless this is not a test requirement unless specified otherwise in a test case. |  |  |  |  |

##### -	MCVideo

Table 5.5.3.1.1-2: SDP Message from the UE for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media descriptions for audio and video | a= line attribute = key-mgmt (NOTE 2) |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Session attribute | optional (NOTE 3) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "audio component of MCVideo" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  |  |  |
| "RS" | any allowed value if present |  | RFC 3556 [113] |  |
| "RR" | any allowed value if present |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec NOTE: In addition to the parameters below the UE may provide further parameters |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media attribute | optional | a= line attribute =sendrecv Indicates send and receive mode being activated |  |  |
| sendrecv |  | Parameter has no value |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in audio media's c= line or in the session's c= line if the audio media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for audio |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in audio media's c= line or in the session's c= line if the audio media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for audio incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A (NOTE 4) | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[2] |  | Media description for video |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | any allowed value | The port for the media-transmission control entity |  |  |
| proto | "RTP/SAVPF" or "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) |  |  |  |
| media title | "video component of MCVideo" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  |  |  |
| "RS" | any allowed value if present |  | RFC 3556 [113] |  |
| "RR" | any allowed value if present |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "H264" |  |  |  |
| clock rate | 90000 |  | RFC 4867 [59] clause 8.3 |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of H264 codec NOTE: In addition to the parameters below the UE may provide further parameters | RFC 6184 [129] |  |
| profile-level-id | any allowed value |  |  |  |
| packetization-mode | 0 |  |  | SDP_ANSWER |
| media attribute | present if proto="RTP/AVP" in the m=line | a= line attribute = tcap | RFC 5939 [128] TS 26.114 [64] clause 6.2.1a.2 | SDP_OFFER |
| tcap |  |  |  |  |
| trpr-cap-num | 1 |  |  |  |
| proto-list | RTP/AVPF |  |  |  |
| media attribute | present if proto="RTP/AVP" in the m=line | a= line attribute = pcfg | RFC 5939 [128] TS 26.114 [64] clause 6.2.1a.2 | SDP_OFFER |
| pcfg |  |  |  |  |
| config-number | 1 |  |  |  |
| pot-cfg-list | t=1 |  |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in video media's c= line or in the session's c= line if the video media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for video |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in video media's c= line or in the session's c= line if the video media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for video incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A (NOTE 4) | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[3] |  | Media description for media control |  | NOT WITHOUT_TRANSMISSIONCONTROL OR PRE_ESTABLISHED_SESSION |
| media description |  | m= line media = application SDP media-level section for a media-control entity (NOTE 2) |  |  |
| media | "application" |  | 3GPP TS 24.581 [88] clause 12 |  |
| port | any allowed value | The port for the media-control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER AND NOT WITHOUT_TRANSMISSIONCONTROL |
| mc_queueing | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
|  | present | Parameter has no value. |  | pc_MCVideo_TransmissionRequestQueueing |
| mc_priority | any allowed value if present | Any integer value in the range of 1..255 Shall be present when priority other than the default priority is required | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | any allowed value if present | Any integer value in the range of 0…255 Shall be present when priority other than the default reception priority is required | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | INITIAL_SDP_OFFER |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_GRANT_REQUESTED |
| format specific parameters |  |  |  | SDP_ANSWER AND NOT WITHOUT_TRANSMISSIONCONTROL |
| mc_queueing | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
|  | present | Parameter has no value |  | pc_MCVideo_TransmissionRequestQueueing |
| mc_priority | same value as in the SDP offer if present, not present otherwise |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | same value as in the SDP offer if present, not present otherwise |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Transmission Control Messages |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in application media's c= line or in the session's c= line if the application media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for application |  |  |  |
| cand-type | "host" |  |  |  |
| NOTE 1: If "ssrc" media attributes are included, then at least one "ssrc=" line shall contain a "cname" source attribute according to RFC 5576 [116] clause 6.1. NOTE 2: Even though there is no clarity in core specs it is assumed that a key-mgmt attribute at session level does not affect the media control security, i.e. the key-mgmt attribute is not applicable for the "application" media description for which still the CSK is used as security key. This is in contrast to RFC 4566 [27] clause 5 saying "In general, session-level values are the default for all media unless overridden by an equivalent media-level value." NOTE 3: If the UE is configured as lite implementation according to RFC 5245 [115], it shall include "a=ice-lite" session-level attribute; nevertheless this is not a test requirement unless specified otherwise in a test case. NOTE 4: If present the a=key-mgmt attributes for audio and video carry the same keys. |  |  |  |  |

##### -	MCData

Table 5.5.3.1.1-3: SDP Message from the UE for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Session attribute | optional (NOTE 1) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media description for audio | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message | RFC 4867 [59] TS 24.282 [31] |  |
| media | "message" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "TCP/MSRP" |  |  |  |
| fmt | “*” |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = sendonly |  | SDP_OFFER AND NOT SDS_SESSION |
| sendonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = recvonly |  | SDP_ANSWER AND NOT SDS_SESSION |
| recvonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = sendrecv |  | SDS_SESSION |
| sendrecv |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = path |  |  |
| path | MSRP URI according to RFC 4975 [120] clause 6 and 9 | attribute containing its own MSRP URI. An example: msrp://mcdata.example.com:7654/abcde1; tcp | TS 24.282 [31] |  |
| scheme | "msrp" |  |  |  |
| authority |  |  | RFC 3986 [123] clause 3.2 |  |
| userinfo | any value if present |  |  |  |
| host | any allowed value | domain name or IP address of the UE |  |  |
| port | same value as in the media line if present | port at which the UE may be connected to for MSRP; mandatory when hostname is an IP address |  |  |
| session id | any allowed value if present |  |  |  |
| transport | "tcp" | mandatory for MSRP according to RFC 4975 [120] clause 6 |  |  |
| URI-parameter | not present |  |  |  |
| media attribute |  | a= line attribute = accept-types | RFC 4975 [120] |  |
| accept-types |  |  |  |  |
| format-entry[1] | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| format-entry[2] | "application/vnd.3gpp.mcdata-payload" |  |  | MCDATA_SDS |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for TCP/MSRP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "TCP/MSRP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in media's c= line or in the session's c= line if the media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a= line attribute = setup | RFC 4145 [119] |  |
| setup | “actpass” |  |  | SDP_OFFER |
|  | "active" or "passive" |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-transfer-id | RFC 5547 [124] | MCDATA_FD |
| file-transfer-id | any allowed value |  | RFC 5547 [124] clause 8.2.1 | SDP_OFFER |
|  | same value as in the sdp offer |  | RFC 5547 [124] clause 8.2.2 | SDP_ANSWER |
| media attribute |  | a= line attribute = file-selector | RFC 5547 [124] | MCDATA_FD |
| file-selector |  |  |  | SDP_OFFER |
| selector[1] |  |  |  |  |
| filename | any allowed value | e.g. "TestFile.txt" |  |  |
| filesize | size of the file to be transferred |  |  |  |
| filetype | any allowed value | e.g. "text/plain" |  |  |
| hash |  |  |  |  |
| algorithm | "sha-1" |  |  |  |
| value | hash value of the file to be transferred |  |  |  |
| file-selector | same value as in the sdp offer |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-date | RFC 5547 [124] | MCDATA_FD AND SDP_OFFER |
| file-date |  |  |  |  |
| date-param | at least one entry with an allowed value |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCDATA | RFC 4567 [44] |  |
| NOTE 1: If the UE is configured as lite implementation according to RFC 5245 [115], it shall include "a=ice-lite" session-level attribute; nevertheless this is not a test requirement unless specified otherwise in a test case. |  |  |  |  |