3GPP TSG-RAN5 Meeting #100	R5-233922

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2697 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 13.6.2.6 for NTN Downlink channel quality reporting accuracy in RRC_CONNECTED under enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Hefei) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 13.6.2.6 is missing in 36.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 13.6.2.6 in 36.521-3. The test time of NPDCCH pm-dsg calculation is same as R5-233917. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IoT NTN WI cannot be finalised. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.6.2.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-2 CR 1009... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT is TBD

13.6.2.6.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22.16 for NB-IoT SAN PCell.

13.6.2.6.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

13.6.2.6.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.6.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.101[5] Clause 7.3 for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25 for a corresponding Band.

Table 13.6.2.6.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.6.

13.6.2.6.4	Test description

13.6.2.6.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.6.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

During the test defined for channel quality reporting accuracy for satellite access, the test system shall emulate and send the GNSS signal to the test UE by AT command. The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.6.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

The test procedures are the same as test procedures defined in 13.6.2.5.4.2.

13.6.2.6.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.6.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.6.4.3-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

13.6.2.6.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The tests consist of two successive time periods of length T1 and T2, respectively, at different SNR levels. The start of T2 coincides with the start of the channel quality measurement period specified in section TS36.133 8.14.4. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables 13.6.2.6.5-1 and 13.6.2.6.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.6.5-1: General Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| NPUSCH repetition level |  |  | 1 |  |
| T1 |  | s | 1 | Initialization period |
| T2 Note 1 |  | s | - | Evaluation period |
| Satellite information | Config 1 | SSC.1 | GSO |  |
|  | Config 2 | SSC.2 | NGSO |  |
| Note 1: This time period starts at the beginning of the NPDCCH period that carries the uplink grant for the channel quality report (section 8.14.4). |  |  |  |  |

Table 13.6.2.6.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | kHz | 200 |  |
| NPDCCH parameter |  | R.30 HD-FDD |  |
| NPDCCH repetition Note4 |  | 16 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98+TT |  |
| NRS | dB | 0+TT | -12+TT |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Channel quality IE Note3 |  | CQI-NPDCCH-NB |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See TS 36.331 [2]. Note 4: The NPDCCH repetition level shall be adjusted during T2 based on the DL channel quality report so that the requirements in Table 9.1.22.16-1 can be verified. |  |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.5.4.2.

<< End of changes>>