3GPP TSG-RAN5 Meeting #100	R5-234000

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2477 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Redcap test case 16.1.2.1, 16.1.2.2, 16.1.2.3 and 16.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - No Table 4.2B.2.5-1 and Table 4.2B.2.5-2 in TS 38.533. Table 4.2B.2.5-2 content doens't align with core spec. - In TC 16.1.2.1, there is no step 14. - In TC 16.1.2.2 and 16.1.2.4, the requirements refer to TS 38.133 clause 4.2.2.5 which is the legacy NR requirements not for Redcap. - In TC 16.1.2.3, there is no T3, and it is unreasonable to jump from step 9 to step 12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update table index, and update table content to align with core spec. - In TC 16.1.2.1, in step 9, correct to be step 12. - In TC 16.1.2.2 and 16.1.2.4, correct the requirement refererence to be TS 38.133 clasue 4.2B.2.5 which is requirement for Redcap. - In TC 16.1.2.3, in step 9, correct to be T2 and correct to be step 12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test requirement is wrong. - Test procedure is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 16.1.2	NR – E-UTRA cell re-selection

#### 16.1.2.0	Minimum conformance requirements

##### 16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] in clause 4.2B.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The parameter NEUTRA_carrier_RedCap is the total number of configured E-UTRA carriers in the neighbour frequency list. The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN_RedCap/2.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.x.y] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [30] within (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least [6dB] for RSRP reselections based on absolute priorities or [4dB] for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

Cells which have been detected shall be measured at least every (NEUTRA_carrier_RedCap) * Tmeasure,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN_RedCap. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [30] within (NEUTRA_carrier_RedCap) * Tevaluate,EUTRAN_RedCap when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least [6dB] for RSRP reselections based on absolute priorities or [4dB] for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

If Treselection timer has a non-zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in Table 16.1.2.0.1-1.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in Table 16.1.2.0.1-2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap when multiple PTWs are used.

Table 16.1.2.0.1-1: Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap, and Tevaluate,EUTRAN_RedCap for 1 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Table 16.1.2.0.1-2: Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap, and Tevaluate,EUTRAN_RedCap for UE configured with eDRX_IDLE cycle

| Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | N/A | N/A | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | N/A | N/A | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | N/A | N/A | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. NOTE 5: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

##### 16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx

Same Minimum conformance requirements as in clause 16.1.2.0.1.

##### 16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

The requirements defined in TS 38.133 [6] clause 4.2B.2.5 apply for this clause except that:

-	Tdetect,EUTRAN_RedCap Relax as specified in Table 16.1.2.0.3-1 and Table 16.1.2.0.3-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,EUTRAN RedCap Relax as specified in Table 16.1.2.0.3-1 and Table 16.1.2.0.3-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,EUTRAN RedCap Relax as specified in Table 16.1.2.0.3-1 and Table 16.1.2.0.3-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 16.1.2.0.3-3 are applicable for eDRX cycle < 10.24 s.

If the UE is configured with eDRX_IDLE cycle ≥ 10.24 s, then the requirements in Table 16.1.2.0.3-4 apply provided that filtering of a measurement is done within a single PTW and provided that the eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

Table 16.1.2.0.3-1: Tdetect,EUTRAN_RedCap_Relax, Tmeasure,EUTRAN_RedCap_Relax, and Tevaluate,EUTRAN_RedCap_Relax for 1 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K5 (36 x K5) | 1.28 x K5 (4 x K5) | 5.12 x K5 (16 x K5) |
| 0.64 | 17.92 x K5 (28 x K5) | 1.28 x K5 (2 x K5) | 5.12 x K5 (8 x K5) |
| 1.28 | 32 x K5 (25 x K5) | 1.28 x K5 (1 x K5) | 6.4 x K5 (5 x K5) |
| 2.56 | 58.88 x K5 (23 x K5) | 2.56 x K5 (1 x K5) | 7.68 x K5 (3 x K5) |
| Note 1: K5 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 16.1.2.0.3-2: Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax, and Tevaluate,EUTRAN_Relax for 2 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K5 (36 x K5) | 1.28 x K5 (4 x K5) | 5.12 x K5 (16 x K5) |
| 0.64 | 17.92 x K5 (28 x K5) | 1.28 x K5 (2 x K5) | 5.12 x K5 (8 x K5) |
| 1.28 | 32 x K5 (25 x K5) | 1.28 x K5 (1 x K5) | 6.4 x K5 (5 x K5) |
| 2.56 | 58.88 x K5 (23 x K5) | 2.56 x K5 (1 x K5) | 7.68 x K5 (3 x K5) |

Table 16.1.2.0.3-3: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | Tdetect,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 16.1.2.0.3-4: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 10.24 ≤ eDRX_IDLE cycle length ≤ 2621.444 | 0.32 | ≥1.28 (1) | K3 x (23 x K3) | 0.32 x K3 (1 x K3) | 0.64 x K3 (2 x K3) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

#### 16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

16.1.2.1.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to higher priority E-UTRA cell, for 1 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.1.2	Test applicability

This test applies to all types of NR RedCap UEwith 1Rx from release 17 and forward.

16.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.1.

16.1.2.1.4	Test description

16.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.1.4.1-1.

Table 16.1.2.1.4.1-1: Supported test configurations for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.1-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.1-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.1-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.1.4.1-2.

Table 16.1.2.1.4.1-2: Initial conditions for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.1.4.1-3.

2.	Message contents are defined in clause 16.1.2.1.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1. E-UTRAN cell 2 is of higher priority than cell 1.

Table 16.1.2.1.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | 2 during T2. |
| T3 end | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | 1 during T3 for iteration of the tests. |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 | 4 |  |
| E-UTRAN PRACH |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in |
| configuration index |  |  | 4, 5, 6 | 4 | TS 36.211 [23] |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively.

Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off. At the start of T2 the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2. At the start of T3 cell 2 becomes weaker than cell 1, and the UE reselects to Cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T1 in Table 16.1.2.1.5-1 and 16.1.2.1.5-2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008 for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.1.2.1.5-1 and 16.1.2.1.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a higher priority Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then count a success for the event “re-select higher priority E-UTRA Cell 2”, continue with step 7. Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, count a fail for the event “re-select higher priority E-UTRA Cell 2”, and the TE shall switch off and, on the UE, and skip to step 12.

7.	The SS shall send an RRCConnectionRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 16.1.2.1.5-1 and 16.1.2.1.5-2. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to a lower priority Cell 1.

10.	If the UE has re-selected Cell 1 within T3, then count a success for the event “re-select lower priority NR Cell 1” and continue with step 11. Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, count a fail for the event “re-select lower priority NR Cell 1” and the TE shall switch off and, on the UE and skip to step 12.

11. The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until a test verdict has been achieved. Each of the events “re-select higher priority E-UTRA Cell 2” and “re-select lower priority NR Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

16.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 1 and higher priority for configuration 16.1.2.1-2, 16.1.2.1-3, 16.1.2.1-5 and 16.1.2.1-6 Table H.2.3-2 with Condition SMTC 2 and higher priority for configuration 16.1.2.1-1, 16.1.2.1-4, 16.1.2.1-7 and 16.1.2.1-8 Table H.2.3-3 with Condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

Table 16.1.2.1.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |  |
| threshServingLow |  | 22 | Actual value = 44 dB |  |
| cellReselectionPriority |  | 5 |  | higher priority E-UTRA |
| } |  |  |  |  |
| } |  |  |  |  |

Table 16.1.2.1.4.3-3: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 4 |  | higher priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

<< Skip unchanged texts >>

#### 16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX

16.1.2.2.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to higher priority E-UTRA cell, for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward.

16.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.2.

<< Skip unchanged texts >>

#### 16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

16.1.2.3.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to lower priority E-UTRA cell, for 1 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 and forward.

16.1.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.3.

16.1.2.3.4	Test description

16.1.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.2.3.4.1-1.

Table 16.1.2.3.4.1-1: Supported test configurations for NR SA FR1 – E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.2.1.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.3-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 16.1.2.1.3-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 16.1.2.1.3-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.2.3.4.1-2.

Table 16.1.2.3.4.1-2: Initial conditions for NR SA FR1 – E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | Without faders. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.2.3.4.1-3.

2.	Message contents are defined in clause 16.1.2.3.4.3.

3.	There are two cells in the test on two carriers. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 16.1.2.3.4.1-3: General test parameters for NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE camps on cell 1 in the initial phase. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6, 7, 8 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6, 7, 8 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 7 | 534 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6, 8 |  |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6, 7, 8 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

16.1.2.3.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T2 in Table 16.1.2.3.5-1 and 16.1.2.3.5-2. SS waits at least for 32sec(Tdetect,EUTRAN)  to ensure the UE can detect E-UTRA neighbour cell (Cell2).

3.	Set the parameters according to T1 in Table 16.1.2.3.5-1 and 16.1.2.3.5-2. T1 starts.

4.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

5.	If the UE responds on Cell 2 during time duration T1 within 8 seconds from the beginning of time period T1 then count a success for the event “re-select lower priority E-UTRA Cell 2” and continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, count a failure for the event “re-select lower priority E-UTRA Cell 2”, the TE shall switch off and on the UE and skip to step 11.

6	The SS shall send an RRCConnectionRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 16.1.2.3.5-1 and 16.1.2.3.5-2. T2 starts.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

9.	If the UE has re-selected Cell 1 within T2, then count a success for the event “re-select higher priority NR Cell 1” and continue with step 10. Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, count a fail for the event “re-select higher priority NR Cell 1” and the TE shall switch off and, on the UE and skip to step 11.

10.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.1.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.2.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-3 with Condition lower priority |
| Default RRC messages and information elements contents exceptions |  |

Table 16.1.2.3.4.3-2: SIB2

| Derivation Path: Table H.2.3-2 with condition lower priority |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present | UE applies the default value of infinity |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.2.2-2, 16.1.2.2-3, 16.1.2.2-5, 16.1.2.2-6 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | 16.1.2.2-1, 16.1.2.2-4, 16.1.2.2-7, 16.1.2.2-8, |
| } |  |  |  |
| } |  |  |  |

16.1.2.3.4.3-3: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |  |
| threshServingLow |  | 22 | Actual value = 44 dB |  |
| cellReselectionPriority |  | 4 |  | lower priority E-UTRA |
| } |  |  |  |  |
| } |  |  |  |  |

16.1.2.3.4.3-4: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 5 |  | lower priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

<< Skip unchanged texts >>

#### 16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX

16.1.2.4.1	Test purpose

The purpose of this test is to verify the requirement for the NR – E-UTRA inter-RAT cell reselection to lower priority E-UTRA cell, for 2 Rx RedCap UEs. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.5.

16.1.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward.

16.1.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.2.4.

<< End of changes >>