3GPP TSG-RAN5 Meeting #100	R5-234017

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3880 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM TC 9.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align with the implementation of TC 9.1.1.3 and 9.1.1.6, step 53 needs to be updated to remove the condition “connected without release”. fl_TC_9_1_1_3_6_Body(false); // Finish f_NR_Postamble(v_NGC_NASCellD, STATE_IDLE_1A); |  |  |  |  |  |  |  |  |
| Summary of change: |  | Step 53 was fixed to enter Idle state in the postamble with editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1.1.6	5G AKA based primary authentication and key agreement / Abnormal

9.1.1.6	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the SS initiates a 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST with ngKSI is already in use }

then { the UE sends an AUTHENTICATION FAILURE message to the network, with the 5GMM cause #71 "ngKSI already in use" }

}

(2)

with { the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the third time SS initiates 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST with ngKSI is already in use }

then { the UE locally releases the RRC connection and treats the active cell as barred }

}

(3)

with { the UE in 5GMM-REGISTERED-INITIATED state, the SS sends an AUTHENTICATION REQUEST with ngKSI is already in use and the UE sends an AUTHENTICATION FAILURE message }

ensure that {

when { T3520 times out }

then { the UE locally releases the RRC connection and treats the active cell as barred }

}

(4)

Void

(5)

with { the UE in 5GMM-REGISTERED state and initiates a mobility registration update procedure }

ensure that {

when { the SS initiates a 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST and the UE fails on transmission of AUTHENTICATION RESPONSE message with the indication from lower layers }

then { the UE re-initiate the mobility registration update procedure }

}

9.1.1.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.4.1.3.7. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.4.1.3.7]

e)	Authentication failure (5GMM cause #71 "ngKSI already in use").

The UE shall send an AUTHENTICATION FAILURE message, with 5GMM cause #71 "ngKSI already in use", to the network and start the timer T3520 (see example in figure 5.4.1.3.7.1). Furthermore, the UE shall stop any of the retransmission timers that are running (e.g. T3510, T3517 or T3521). Upon the first receipt of an AUTHENTICATION FAILURE message from the UE with 5GMM cause #71 "ngKSI already in use", the network performs necessary actions to select a new ngKSI and send the same 5G authentication challenge to the UE.

…

g)	Network failing the authentication check.

If the UE deems that the network has failed the authentication check, then it shall request RRC to locally release the RRC connection and treat the active cell as barred (see 3GPP TS 38.304 [28]). The UE shall start any retransmission timers (e.g. T3510, T3517 or T3521), if they were running and stopped when the UE received the first AUTHENTICATION REQUEST message containing an incorrect authentication challenge data causing authentication failure.

h)	Transmission failure of AUTHENTICATION RESPONSE message or AUTHENTICATION FAILURE message indication from lower layers (if the 5G AKA based primary authentication and key agreement procedure is triggered by a registration procedure for mobility and periodic registration update).

The UE shall stop the timer T3520, if running, and re-initiate the registration procedure for mobility and periodic registration update.

…

i)	Transmission failure of AUTHENTICATION RESPONSE message or AUTHENTICATION FAILURE message indication with TAI change from lower layers (if the 5G AKA based primary authentication and key agreement procedure is triggered by a service request procedure).

The UE shall stop the timer T3520, if running.

If the current TAI is not in the TAI list, the 5G AKA based primary authentication and key agreement procedure shall be aborted and a registration procedure for mobility and periodic registration update shall be initiated.

If the current TAI is still part of the TAI list, it is up to the UE implementation how to re-run the ongoing procedure that triggered the 5G AKA based primary authentication and key agreement procedure.

…

For items c, d, e, and f whether or not the UE is registered for emergency services:

…

The UE shall deem that the network has failed the authentication check or assume that the authentication is not genuine and proceed as described in item g above if any of the following occurs:

-	the timer T3520 expires;

-	the UE detects any combination of the 5G authentication failures: 5GMM causes #20 "MAC failure", #21 "synch failure", #26 "non-5G authentication unacceptable" or #71 "ngKSI already in use", during three consecutive authentication challenges. The 5G authentication challenges shall be considered as consecutive only, if the 5G authentication challenges causing the second and third 5G authentication failure are received by the UE, while the timer T3520 started after the previous 5G authentication failure is running.

9.1.1.6.3	Test description

9.1.1.6.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A, NGC Cell B, NGC Cell C and NGC Cell D are configured according to table 6.3.2.2-1 in TS 38.508-1 [4].

-	The SS configures the NGC Cell A as the "Serving cell" and other NGC Cells as "Non-suitable "Off" cell".

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	None.

Preamble:

-	The UE is in test state 0N-B on NGC Cell A according to TS 38.508-1 [4]. The ngKSI-1 has been assigned and security context has been established.

9.1.1.6.3.2	Test procedure sequence

Table 9.1.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise. | - | - | - | - |
| 0 | The UE is switched on. | - | - | - | - |
| 0A | The UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4] and transmits a REGISTRATION REQUEST message. | --> | 5GMM: REGISTRATION REQUEST | - | - |
| 1 | The SS initiates a 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST with ngKSI is already in use (ngKSI-1). | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 2 | Check: Does the UE send an AUTHENTICATION FAILURE message to the network, with the 5GMM cause #71 "ngKSI already in use"? | --> | 5GMM: AUTHENTICATION FAILURE | 1 | P |
| 3 | The SS initiates a 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST with ngKSI is already in use (ngKSI-1). | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 4 | Check: Does the UE send an AUTHENTICATION FAILURE message to the network, with the 5GMM cause #71 "ngKSI already in use"? | --> | 5GMM: AUTHENTICATION FAILURE | 1 | P |
| 5 | Void | - | - | - | - |
| 6 | The SS initiates a 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST with ngKSI is already in use (ngKSI-1). | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 6a1 | EXCEPTION: The UE may send an AUTHENTICATION FAILURE before locally releasing the RRC Connection. | --> | 5GMM: AUTHENTICATION FAILURE | - | - |
| 6A | Check: Does the UE transmit a RRCSetupRequest on NGC Cell A in the next 30 seconds? (Note 1) | --> | 5G RRC: RRCSetupRequest | 2 | F |
| 6B | The SS configures: -NGC Cell B as the "Serving cell". -NGC Cell A as a "Suitable neighbour intra-frequency cell". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell B unless explicitly stated otherwise. | - | - | - | - |
| 7-9 | The UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 10 | Check: Does the UE transmit a REGISTRATION REQUEST message with the 5GS registration type IE setting as initial registration? | --> | 5GMM: REGISTRATION REQUEST | 2 | P |
| 11 | The SS initiates a 5G AKA based primary authentication and key agreement procedure by sending AUTHENTICATION REQUEST with ngKSI is already in use (ngKSI-1). | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 12 | The UE sends an AUTHENTICATION FAILURE message to the network, with the 5GMM cause #71 "ngKSI already in use". | --> | 5GMM: AUTHENTICATION FAILURE | - | - |
| 12A | SS starts timer of t_Waits=T3520. | - | - | - | - |
| 13 | Void | - | - | - | - |
| 14 | SS waits for the expiration of t_Waits. | - | - | - | - |
| 14A | Check: Does the UE transmit a RRCSetupRequest on NGC Cell B in the next 30 seconds? (Note 1) | --> | 5G RRC: RRCSetupRequest | 2 | F |
| 14B | The SS configures: -NGC Cell C as the "Serving cell". -NGC Cell B as a "Suitable neighbour intra-frequency cell". -NGC Cell A as a "Non-suitable "Off" cell" | - | - | - | - |
| - | The following messages are to be observed on NGC Cell C unless explicitly stated otherwise. | - | - | - | - |
| 15-17 | The UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 18 | Check: Does the UE transmit a REGISTRATION REQUEST message with the 5GS registration type IE setting as initial registration? | --> | 5GMM: REGISTRATION REQUEST | 3 | P |
| 19-34a1 | Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 35-39 | Void | - | - | - | - |
| 40 | The SS configures: - NGC Cell D as the “Serving cell”. - NGC Cell C as a “Non-suitable "Off" cell “. - NGC Cell B as a "Non-suitable "Off" cell ". | - | - | - | - |
| 41-42 | Void | - | - | - | - |
| - | The following messages are to be observed on NGC Cell D unless explicitly stated otherwise. | - | - | - | - |
| 43-45 | The UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 46 | The UE transmits a REGISTRATION REQUEST message with the 5GS registration type IE setting as Mobility registration updating. | --> | 5GMM: REGISTRATION REQUEST | - | - |
| 47 | The SS cuts off the UL grant and RA Response, so that the UE cannot send the AUTHENTICATION RESPONSE to SS. | - | - | - | - |
| 48 | SS transmits an AUTHENTICATION REQUEST message with ngKSI-2 to initiate the 5G-AKA procedure. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 49 | SS starts timer of t_Delay =10s. (Note2). | - | - | - | - |
| 50 | SS performs local release. | - | - | - | - |
| 51 | Check whether t_Delay is still running, if it’s running, then waiting for timeout. | - | - | - | - |
| 52 | SS configures the RA Response. | - | - | - | - |
| 53 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE is camped on NGC Cell D? | - | - | 5 | P |
| Note 1: If the cell is not barred, after the transmission of REGISTRATION REQUEST, the UE will start T3510 and T3511. After 25s (T3510+T3511), the UE shall send REGISTRATION REQUEST. Note 2: To send the AUTHENTICATION RESPONSE, the UE will initiate RACH to get UL grant. Since there is no RA Response, registration failure due to lower layer failure will occur, then T3511 will start. Timer t_Delay is derived from T3511. During timer t_Delay, UE fails on transmission of AUTHENTICATION RESPONSE message with the indication from lower layers. |  |  |  |  |  |

9.1.1.6.3.3	Specific message contents

Table 9.1.1.6.3.3-1: AUTHENTICATION REQUEST (step 1, step 3, step 6 and step 11, Table 9.1.1.6.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI | ngKSI-1 | The same with the ng-KSI assigned in Preamble. |  |

Table 9.1.1.6.3.3-2: AUTHENTICATION FAILURE (step 2, step 4, step 6a1 and step 12, Table 9.1.1.6.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM cause | ‘0100 0111’B | ngKSI already in use |  |

Table 9.1.1.6.3.3-3: AUTHENTICATION REQUEST (step 48, Table 9.1.1.6.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI | ngKSI-2 | Different from the ng-KSI assigned in step 19 |  |

Table 9.1.1.6.3.3-4: REGISTRATION REQUEST (step 10 and step 18, Table 9.1.1.6.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type |  |  |  |
| 5GS registration type value | '001'B | initial registration |  |

Table 9.1.1.6.3.3-5: REGISTRATION REQUEST (step 46, Table 9.1.1.6.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type |  |  |  |
| 5GS registration type value | '010'B | mobility registration updating |  |

[End of change]