3GPP TSG WG RAN5 Meeting #100	R5-234239

# Toulouse, FR, 21 – 25 Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test case 6.4B.2.1 Error Vector Magnitude (EVM) for Category NB1 and NB2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-17 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding test case 6.4B.2.1 Error Vector Magnitude (EVM) for Category NB1 and NB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete test case for 6.4B.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4B.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.4B.2	Transmit modulation quality for Category NB1 and NB2

6.4B.2.1    Error Vector Magnitude (EVM) for Category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.4B.2.1.1	Test purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

6.4B.2.1.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of UE category NB1 and NB2 that support satellite access operation.

6.4B.2.1.3	Minimum conformance requirements

The RMS average of the basic EVM measurements for 240/LCtone slots excluding any transient period for the average EVM case, where LCtone = {1, 3, 6, 12} is the number of subcarriers for the NB-IoT transmission, for the different modulations schemes shall not exceed the values specified in Table 6.5.2.1F.1.3-1 for the parameters defined in Table 6.5.2.1F.1.3-2. For EVM evaluation purposes, both NPRACH formats are considered to have the same EVM requirement as QPSK modulated.

Table 6.4B.2.1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| BPSK or QPSK | % | 17.5 | 17.5 |

Table 6.4B.2.1.3-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -40 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 36.102 [11] clause 6.4B.2.

6.4B.2.1.4	Test description

6.4B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.4B.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.4B.2.1.4.1-1: Test Configuration for NPUSCH for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones, start position | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | QPSK | 1@0 | 15 |
| 4 |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.4B.2.1.4.1-2: Test Configuration for NPRACH for FDD

| Initial Conditions |  |
| --- | --- |
| Test Environment (as specified in TS 36.508 [12] subclause 8.1.1) | Normal |
| Test Frequencies (as specified in TS36.508 [12] subclause 8.1.3.1) | Frequency ranges defined in Annex K.1.1 |
| NPRACH preamble format | 1 |
| NRS EPRE setting for test point (dBm/15kHz) | -110 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 8.1.4.3

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H.

4.	The UL Reference Measurement channels are set according to in Table 6.4B.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is State 2A-NB with CP CIoT Optimisation to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.4B.2.1.4.3.

6.4B.2.1.4.2	Test procedure

Test procedure for NPUSCH:

1.1. SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.4B.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2	Configure the UE to transmit at PUMAX level.

1.3	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). The measurement period of EVM for 240/Ntones slots should exclude any transient period for the average EVM case, where Ntones = {1, 3, 6, 12} is the number of subcarriers for the NB-IoT transmission.

1.4 Release the connection through State 3A-NB.

1.5 Modify system information elements according to Table 6.4B.2.1.4.3-1 and Table 6.4B.2.1.4.3-2 and notify the UE via paging message with SystemInformationModification included (test point 2).

1.6 Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the new UL power control setting.

1.7 SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.4B.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.8	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). The measurement period of EVM for 240/Ntones slots should exclude any transient period for the average EVM case, where Ntones = {1, 3, 6, 12} is the number of subcarriers for the NB-IoT transmission.

Test procedure for NPRACH:

2.1	The SS shall set RS EPRE according to Table 6.4B.2.1.4.1-2.

2.2	NPRACH is set according to Table 6.4B.2.1.4.1-2.

2.3	The UE shall send a preamble to the SS.

2.4	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

2.5	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated NPRACH transmission power.

2.6	Repeat step 4 and 5 until the SS collect enough NPRACH preambles (64 preambles). Measure the EVM in NPRACH channel using Global In-Channel Tx-Test (Annex E).

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.4B.2.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions:

Table 6.4B.2.1.4.3-1: P0-NominalNPUSCH-r13 configuration for test point 2

| Derivation Path: TS 36.508 [12] clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -117 (dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

Table 6.4B.2.1.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT configuration for test point 2

| Derivation Path: TS 36.508 [12] clause 8.1.6.3 Table 8.1.6.3-4: NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 21 (dBm) |  |  |
| } |  |  |  |

Table 6.4B.2.1.4.3-3: RACH-ConfigCommon-NB-DEFAULT NPRACH EVM Measurement

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-8 RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB0 | 0 dB |  |
| } |  |  |  |
| ) |  |  |  |

Table 6.4B.2.1.4.3-4: NPRACH-ConfigSIB-NB-DEFAULT for NPRACH EVM Measurement

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-5 NPRACH-ConfigSIB-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us266dot7 | 8192*Ts |  |
| } |  |  |  |

6.4B.2.1.5	Test requirement

The NPUSCH EVM derived in E.8.1 shall not exceed 17,5% for BPSK and QPSK.

The NPUSCH derived in E.8.2 shall not exceed 17,5 % when embedded with data symbols of BPSK and QPSK.

The NPRACH EVM derived in E.8.3 shall not exceed 17.5%.

<< End of changes >>