3GPP TSG-RAN5 Meeting #100	R5-234487

# Toulouse, France, Augest 21nd - May 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3934 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to mobility enhancement SIG TC 8.1.4.4.3 - CHO failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The note in Table 8.1.4.4.3.3.2-3 should be “The wait time is selected to cover T304 (2000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE” according to Table 8.1.4.4.3.3.3-9 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Note in Table 8.1.4.4.3.3.2-3 is corrected. - Title of Table 8.1.4.4.3.3.3-9 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion is caused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Editorial |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.4.4.3	Conditional handover / Failure

8.1.4.4.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting conditional handover and receiving a RRCReconfiguration message including ConditionalReconfiguration }

ensure that {

when { UE detects conditional handover failure and finds a selectable cell which is the candidate cell included in ConditionalReconfiguration before T311 expires }
    then { UE applies the stored condRRCReconfig associated to the selected cell and perform conditional handover to the selected cell }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting conditional handover and receiving a RRCReconfiguration message including ConditionalReconfiguration }

ensure that {

when { UE detects conditional handover failure and cannot find any selectable cell before T311 expires }
    then { UE goes to RRC_IDLE and release the stored conditional handover configurations }

}

8.1.4.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300 clause 9.2.3.1, 9.2.3.4.1, and 9.2.3.4.2 and TS 38.331 clause 5.3.7.3, 5.3.7.6 and 5.3.11. Unless otherwise stated these are Rel-16 requirements.

[TS 38.300, clause 9.2.3.1]

Timer based handover failure procedure is supported in NR. RRC connection re-establishment procedure is used for recovering from handover failure except in certain CHO or DAPS handover scenarios:

-	When DAPS handover fails, the UE falls back to the source cell configuration, resumes the connection with the source cell, and reports DAPS handover failure via the source without triggering RRC connection re-establishment if the source link has not been released.

-	When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

[TS 38.300, clause 9.2.3.4.1]

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed (legacy handover or conditional handover execution).

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in [12]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 9.2.3.2, regardless of any previously received CHO configuration.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

CHO is not supported for NG-C based handover in this release of the specification.

[TS 38.300, clause 9.2.3.4.2]

As in intra-NR RAN handover, in intra-NR RAN CHO, the preparation and execution phase of the conditional handover procedure is performed without involvement of the 5GC; i.e. preparation messages are directly exchanged between gNBs. The release of the resources at the source gNB during the conditional handover completion phase is triggered by the target gNB. The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.4.2-1: Intra-AMF/UPF Conditional Handover

0/1.	Same as step 0, 1 in Figure 9.2.3.2.1-1 of clause 9.2.3.2.1.

2.	The source gNB decides to use CHO.

3.	The source gNB requests CHO for one or more candidate cells belonging to one or more candidate gNBs. A CHO request message is sent for each candidate cell.

4.	Same as step 4 in Figure 9.2.3.2.1-1 of clause 9.2.3.2.1.

5.	The candidate gNB(s) sends CHO response (HO REQUEST ACKNOWLEDGE) including configuration of CHO candidate cell(s) to the source gNB. The CHO response message is sent for each candidate cell.

6.	The source gNB sends an RRCReconfiguration message to the UE, containing the configuration of CHO candidate cell(s) and CHO execution condition(s).

NOTE 1:	CHO configuration of candidate cells can be followed by other reconfiguration from the source gNB.

NOTE 1a:	A configuration of a CHO candidate cell cannot contain a DAPS handover configuration.

7.	The UE sends an RRCReconfigurationComplete message to the source gNB.

7a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

8.	The UE maintains connection with the source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source gNB, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to the target gNB. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

8a/b	The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE 2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

8c.	The source gNB sends the HANDOVER CANCEL message toward the other signalling connections or other candidate target gNBs, if any, to cancel CHO for the UE.

[TS 38.331, clause 5.3.7.3]

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

…

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in VarConditionalReconfig:

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

1>	else:

2>	if UE is configured with conditionalReconfiguration:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	suspend all RBs, except SRB0;

2>	remove all the entries within VarConditionalReconfig, if any;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

[TS 38.331, clause 5.3.7.6]

Upon T311 expiry, the UE shall:

1>	if the procedure was initiated due to radio link failure or handover failure:

2>	set the noSuitableCellFound in the VarRLF-Report to true;

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

[TS 38.331, clause 5.3.11]

The UE shall:

1>	reset MAC;

…

1>	remove all the entries within VarConditionalReconfig, if any;

1>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

2>	for the associated reportConfigId:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

3>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

1>	discard the KgNB key, the S-KgNB key, the S-KeNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if any;

1>	release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	except if going to RRC_IDLE was triggered by inter-RAT cell reselection while the UE is in RRC_INACTIVE or RRC_IDLE or when selecting an inter-RAT cell while T311 was running or when selecting an E-UTRA cell for EPS fallback for IMS voice as specified in 5.4.3.5:

2>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20];

8.1.4.4.3.3	Test description

8.1.4.4.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 4.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR cells

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.4.4.3.3.2	Test procedure sequence

Tables 8.1.4.4.3.3.2-1 and 8.1.4.4.3.3.2-2 illustrate the downlink power levels to be applied for NR cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1", T2 and "T3", are applied at the point indicated in the Main behaviour description in Table 8.1.4.4.3.3.2-3.

Table 8.1.4.4.3.3.2-1: Power levels in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -113 | -113 | Power levels are such that UE registered on NR Cell 1 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -113 | -113 | -79 | The power level is such that only SrxlevNR Cell 4 > 0 |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -79 | -91 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 1 |
| T4 | SS/PBCH SSS EPRE | dBm/ SCS | -113 | -113 | -113 | Power levels are such that all Srxlev NR Cell < 0 |
| T5 | SS/PBCH SSS EPRE | dBm/ SCS | -113 | -79 | -113 | The power level is such that only SrxlevNR Cell 2 > 0 |

Table 8.1.4.4.3.3.2-2: Power levels in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | FFS | Power levels are such that UE registered on NR Cell 1 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | FFS | The power level is such that only SrxlevNR Cell 4 > 0 |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for NR Cell 1 |
| T4 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | FFS | Power levels are such that all Srxlev NR Cell < 0 |
| T5 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | FFS | The power level is such that only SrxlevNR Cell 2 > 0 |

Table 8.1.4.4.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as the target candidate cells. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| - | EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. | - | - | - | - |
| 4 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. | - | - | - | - |
| 5 | The SS does not respond. | - | - | - | - |
| 6 | SS adjusts the cell-specific reference signal level according to row "T2" right before T304 expire. | - | - | - | - |
| 7 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? | --> | (PRACH Preamble) | 1 | P |
| 8 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 9 | The UE transmits RRCReconfigurationComplete message in NR Cell 4. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? | - | - | 1 | - |
| 11 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 2 as the target candidate cells. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message in NR Cell 4. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 13 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| - | EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. | - | - | - | - |
| 14 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 1. | - | - | - | - |
| 15 | The SS does not respond. | - | - | - | - |
| 16 | SS adjusts the cell-specific reference signal level according to row "T4". | - | - | - | - |
| 17 | Wait 5s to ensure that T304 and T311 expire. (Note 1) |  |  |  |  |
| 18 | SS adjusts the cell-specific reference signal level according to row "T5". | - | - | - | - |
| 19 | Check: Is the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 performed on NR Cell 2? | - | - | 2 | - |
| Note 1: The wait time is selected to cover T304 (2000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE. |  |  |  |  |  |

8.1.4.4.3.3.3	Specific message contents

Table 8.1.4.4.3.3.3-1: SIB1 for NR Cell 2 (preamble and all steps, Table 8.1.4.4.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityInfoList SEQUENCE { |  |  |  |
| trackingAreaCode | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-2: RRCReconfiguration (Step1 and step 11, Table 8.1.4.4.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 8.1.4.4.3.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| conditionalReconfiguration-r16 |  | ConditionalReconfiguration | Table 8.1.4.4.3.3.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.4.4.3.3.3-3: MeasConfig (Table 8.1.4.4.3.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.4.4.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA3 | Table 8.1.4.4.3.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-4: MeasObjectNR (Table 8.1.4.4.3.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 | The SSB of NR Cell 1, NR Cell 2 and NR Cell 4 have the same ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-5: ReportConfigNR-condEventA3 (Table 8.1.4.4.3.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO AND EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB (2*0.5 dB) | FR1 |
|  | FFS |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-6: ConditionalReconfiguration (Table 8.1.4.4.3.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList-r16 | Table 8.1.4.4.3.3.3-7 |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-7: CondReconfigToAddModList-r16 (Table 8.1.4.4.3.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entries |  |  |
| CondReconfigToAddMod-r16[1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR Cell 2 | Table 8.1.4.4.3.3.3-8 | Step 1 |
|  | RRCReconfiguration-HO with condition HO_NR Cell 1 | Table 8.1.4.4.3.3.3-8 | Step 11 |
| } |  |  |  |
| CondReconfigToAddMod-r16[2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigId-r16 | 2 |  |  |
| condExecutionCond-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR Cell 4 | Table 8.1.4.4.3.3.3-8 | Step 1 |
|  | RRCReconfiguration-HO with condition HO_NR Cell 2 | Table 8.1.4.4.3.3.3-8 | Step 11 |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-8: RRCReconfiguration-HO (Table 8.1.4.4.3.3.3-7)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig with condition PCI_NR Cell 1 | Table 8.1.4.4.3.3.3-9 | HO_NR Cell 1 |
|  | CellGroupConfig with condition PCI_NR Cell 2 | Table 8.1.4.4.3.3.3-9 | HO_NR Cell 2 |
|  | CellGroupConfig with condition PCI_NR Cell 4 | Table 8.1.4.4.3.3.3-9 | HO_NR Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.3.3.3-9: CellGroupConfig (Table 8.1.4.4.3.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 1 |  | PCI_NR Cell 1 |
|  | Physical Cell Identity of NR Cell 2 |  | PCI_NR Cell 2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR Cell 4 |
| } |  |  |  |
| t304 | ms2000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>