3GPP TSG-RAN5 Meeting #100	R5-234585

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0320 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for Resource-lists for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The default Resource-lists element for MCData did not have IEs for pre-established session which was introduced in Rel-16. New test cases called the element and the test cases need the pre-established session IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added pre-established IEs to the Resource-lists element for MCData in Table 5.5.3.3.1-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 5.5.3.3	Resource-lists

##### 5.5.3.3.1	Resource-lists from the UE for call control

##### -	MCPTT

Table 5.5.3.3.1-1: Resource-lists from the UE for call control in MCPTT

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | encrypted (NOTE 1) |  |  |  |
| list[1] | encrypted (NOTE 1) |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 1, 2 |  |  |  |
| uri attribute | px_MCPTT_ID_User_B | The MCPTT ID of the invited user |  |  |
|  | SIP-URI with px_MCPTT_Group_A_ID (NOTE 3) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: prearranged MCPTT group identity or chat group identity extended with header fields |  | PRE-ESTABLISH AND (GROUP-CALL OR CHAT-GROUP-CALL) |
|  | SIP-URI with px_MCPTT_ID_User_B (NOTE 3) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: MCPTT ID of the called user extended with header fields |  | PRE-ESTABLISH AND (PRIVATE-CALL OR FIRST-TO-ANSWER) |
| display-name | not present |  |  |  |
| entry[2] | NOTE 1, 2 |  |  | FIRST-TO-ANSWER |
| uri attribute | px_MCPTT_ID_User_C |  |  |  |
| display-name | not present |  |  |  |
| entry[2] | NOTE 1, 2 |  |  | PRE-ESTABLISH AND FIRST-TO-ANSWER |
| uri attribute | SIP-URI with px_MCPTT_ID_User_C (NOTE 3) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: MCPTT ID of the called user extended with header fields |  |  |
| display-name | not present |  |  |  |
| NOTE 1: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry’s uri attribute as described in Table 5.5.13.3-1 NOTE 2: When a resource-lists document contains more than one entry, the entries may be in any order NOTE 3: TS 23.179 [8] specifies MCPTT ID and MCPTT group ID (clause 8.1.3.1) to be a URIs but does not mandate them to be a SIP URIs; nevertheless according to TS 24.379 [9] (clauses 10.1.1.2.2.1, 10.1.2.2.2.1) the URI in the uri attribute of the resource-lists' <entry> element needs to be a SIP URI. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRE-ESTABLISH | Call establishment using a pre-established session |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.3.1-2: Resource-lists from the UE for call control in MCVideo

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | encrypted (NOTE 1) |  |  |  |
| list[1] | encrypted (NOTE 1) |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 1, 2 |  |  |  |
| uri attribute | px_MCVideo_ID_User_B | The MCVideo ID of the invited user |  |  |
| display-name | Not present |  |  |  |
| NOTE 1: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry's uri attribute as described in Table 5.5.13.3-1 NOTE 2: When a resource-lists document contains more than one entry, the entries may be in any order. |  |  |  |  |

##### -	MCData

Table 5.5.3.3.1-3: Resource-lists from the UE for call control in MCData

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | encrypted (NOTE 1) |  |  |  |
| list | encrypted (NOTE 1) |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 1, 2 |  |  |  |
| uri attribute | px_MCData_ID_User_B | The MCData ID of the target MCData user |  |  |
|  | SIP-URI with px_MCData_Group_A_ID (NOTE 3) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: prearranged MCData group identity with header fields |  | PRE-ESTABLISH AND MCD_grp |
|  | SIP-URI with px_MCData_ID_User_B (NOTE 3) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: MCData ID of the called user extended with header fields |  | PRE-ESTABLISH AND MCD_1to1 |
| display-name | not present |  |  |  |
| NOTE 1: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry's uri attribute as described in Table 5.5.13.3-1 NOTE 2: When a resource-lists document contains more than one entry, the entries may be in any order. NOTE 3: According to TS 24.282 [87] (clauses 9.2.5.2.1.1, 9.2.5.3.1.1) the URI in the uri attribute of the resource-lists' <entry> element needs to be a SIP URI. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRE-ESTABLISH | Call establishment using a pre-established session |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A group MCData call |
| For further conditions see table 5.5.1-1 |  |

<END OF CHANGES>