3GPP TSG-RAN5 Meeting #100	R5-234664

# Toulouse, France, August 21st - August 25th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2560 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Redcap RLM, BFR and BWP switch tests including TTs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm India Pvt Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Redcap test cases 16.5.1.5, 16.5.1.6, 16.5.2.3, 16.5.2.4, 16.5.3.1.1, 16.5.3.1.2, 16.5.3.2.1 and 16.5.3.2.2 have been added to the test specification. TT needs to be updated in the test case accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated test parameters according to the TT calculation to Redcap test cases 16.5.1.5, 16.5.1.6, 16.5.2.3, 16.5.2.4, 16.5.3.1.1, 16.5.3.1.2, 16.5.3.2.1 and 16.5.3.2.2 - Removed editor’s notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1.5, 16.5.1.6, 16.5.2.3, 16.5.2.4, 16.5.3.1.1, 16.5.3.1.2, 16.5.3.2.1, 16.5.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

16.5.1.5.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used for 1Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards.

16.5.1.5.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.5.

16.5.1.5.4	Test description

Same as the test description given in clause 6.5.1.3.4.

16.5.1.5.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.3.4.1 with following exceptions:

-	Table 6.5.1.3.4.1-1 is replaced by Table 16.5.1.5.4.1-1.

-	Table 6.5.1.3.4.1-2 is replaced by Table 16.5.1.5.4.1-2.

-	Table 6.5.1.3.4.1-4 is replaced by Table 16.5.1.5.4.1-3.

Table 16.5.1.5.4.1-1: Supported test configurations for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.1.5-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.5-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.5-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.5-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.5.4.1-2: Initial conditions for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.5.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.5.4.1-3: General test parameters for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 52 |
|  |  | Config 4 |  | 10: NRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
|  |  | Config 4 |  | Not Applicable |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1, 2, 4 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2, 4 |  | Table A.7.1, PRACH.1 FR1 |
|  |  | Config 3 |  | Table A.7.1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.5.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.3.4.2.

16.5.1.5.4.3	Message Contents

Same as the message contents given in clause 6.5.1.3.4.3.

16.5.1.5.5	Test Requirement

Same as the test requirements given in clause 6.5.1.3.5 with following exceptions:

-	Table 6.5.1.3.5-1 is replaced by Table 16.5.1.5.5-1.

Table 16.5.1.5.5-1: Cell specific test parameters (Cell 1) for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1, 4 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
|  | Config 1, 4 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1, 4 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.3.4-1. |  |  |  |  |  |

#### 16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

16.5.1.6.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used for 2Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.

16.5.1.6.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards.

16.5.1.6.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.6.

16.5.1.6.4	Test description

Same as the test description given in clause 6.5.1.3.4.

16.5.1.6.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.3.4.1 with following exceptions:

-	Table 6.5.1.3.4.1-1 is replaced by Table 16.5.1.6.4.1-1.

-	Table 6.5.1.3.4.1-2 is replaced by Table 16.5.1.6.4.1-2.

-	Table 6.5.1.3.4.1-4 is replaced by Table 16.5.1.6.4.1-3.

Table 16.5.1.6.4.1-1: Supported test configurations for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.1.6-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.6-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.6-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.6-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.6.4.1-2: Initial conditions for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.6.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.6.4.1-3: General test parameters for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 52 |
|  |  | Config 4 |  | 10: NRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
|  |  | Config 4 |  | Not Applicable |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1, 2, 4 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2, 4 |  | Table A.7.1, PRACH.1 FR1 |
|  |  | Config 3 |  | Table A.7.1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.6.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.3.4.2.

16.5.1.6.4.3	Message Contents

Same as the message contents given in clause 6.5.1.3.4.3.

16.5.1.6.5	Test Requirement

Same as the test requirements given in clause 6.5.1.3.5 with following exceptions:

-	Table 6.5.1.3.5-1 is replaced by Table 16.5.1.6.5-1.

Table 16.5.1.6.5-1: Cell specific test parameters (Cell 1) for NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1, 4 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
|  | Config 1, 4 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1, 4 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS Res. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.3.4-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |

<< Skipped sections >>

#### 16.5.2.3	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

16.5.2.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5B.2.

16.5.2.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards supporting link recovery and long DRX cycle.

16.5.2.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.3.

16.5.2.3.4	Test description

Same as the test description given in clause 6.5.5.2.4.

16.5.2.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.2.4.1 with following exceptions:

-	Table 6.5.5.2.4.1-1 is replaced by Table 16.5.2.3.4.1-1.

-	Table 6.5.5.2.4.1-2 is replaced by Table 16.5.2.3.4.1-2.

-	Table 6.5.5.2.4.1-3 is replaced by Table 16.5.2.3.4.1-3.

Table 16.5.2.3.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.2.3-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.3-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.3-3 | TDD duplex mode, 30 kHz SSB SCS, 20 MHz bandwidth |
| 16.5.2.3-4 | HD-FDD duplex mode, 15kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.2.3.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1and and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.3.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2, 3 |  | TDD |  |
|  |  |  | Config 4 |  | HD-FDD |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 20: NRB,c = 51 |  |
|  |  |  | Config 4 |  | 10: NRB,c = 52 |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 RedCap |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 RedCap |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
|  |  |  | Config 4 |  | Not Applicable |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
|  |  |  | Config 4 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |
|  |  |  | Config 4 |  | CCR.1.1 FDD |  |
| SSB Configuration |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
|  |  |  | Config 4 |  | SSB.3 FR1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  | Config 3 |  | SMTC.1 |  |
|  |  |  | Config 4 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz |  |
|  |  |  | Config 4 |  | 15 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | PRACH.1 | Table A.7-1 |
|  |  |  | Config 3 |  | PRACH.1 | Table A.7-1 |
|  |  |  | Config 4 |  | PRACH.1 | Table A.7-1 |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x1 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  | 2 |  |
|  |  | Aggregation level |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | A.5 |
| Gap pattern ID |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | Absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| Rsrp-ThresholdSSB | Config 1, 2 |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  | -95 |  |
|  | Config 4 |  |  |  | -98 |  |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
|  |  | Config 4 |  |  | CSI-RS.1.1 FDD |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
|  |  | Config 4 |  |  | TRS.1.1 FDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 5.17 |  |
| T3 |  |  |  | s | [5.84] |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

16.5.2.3.4.2	Test Procedure

Same as the test procedure given in clause 6.5.5.2.4.2.

16.5.2.3.4.3	Message Contents

Same as the message contents given in clause 6.5.5.2.4.3.

16.5.2.3.5	Test Requirement

Same as the test requirements given in clause 6.5.5.2.5 with following exceptions:

-	Table 6.5.5.2.5-1 is replaced by Table 16.5.2.3.5-1.

Table 16.5.2.3.5-1: Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 4 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -13.25 | -13.25 | 13.25 | 13.25 | 13.25 |
|  | Config 2 |  | -13.25 | -13.25 | 13.25 | 13.25 | 13.25 |
|  | Config 3 |  | -13.25 | -13.25 | 13.25 | 13.25 | 13.25 |
|  | Config 4 |  | -13.25 | -13.25 | 13.25 | 13.25 | 13.25 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 2 |  | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 3 |  | -108.25 | -108.25 | -81.75 | -81.75 | -81.75 |
|  | Config 4 |  | -111.25 | -111.25 | -84.75 | -84.75 | -84.75 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 4 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting is assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS Res. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.1.4-1. |  |  |  |  |  |  |  |

#### 16.5.2.4	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

16.5.2.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5B.2.

16.5.2.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards supporting link recovery and long DRX cycle.

16.5.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.2.0.1 and 16.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.2.4.

16.5.2.4.4	Test description

Same as the test description given in clause 6.5.5.2.4.

16.5.2.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.5.2.4.1 with following exceptions:

-	Table 6.5.5.2.4.1-1 is replaced by Table 16.5.2.4.4.1-1.

-	Table 6.5.5.2.4.1-2 is replaced by Table 16.5.2.4.4.1-2.

-	Table 6.5.5.2.4.1-3 is replaced by Table 16.5.2.4.4.1-3.

Table 16.5.2.4.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.2.4-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.4-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 16.5.2.4-3 | TDD duplex mode, 30 kHz SSB SCS, 20 MHz bandwidth |
| 16.5.2.4-4 | HD-FDD duplex mode, 15kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.2.4.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.2.4.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2, 3 |  | TDD |  |
|  |  |  | Config 4 |  | HD-FDD |  |
| Bwchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 20: NRB,c = 51 |  |
|  |  |  | Config 4 |  | 10: NRB,c = 52 |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 RedCap |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 RedCap |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
|  |  |  | Config 4 |  | Not Applicable |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
|  |  |  | Config 4 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |
|  |  |  | Config 4 |  | CCR.1.1 FDD |  |
| SSB Configuration |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  | Config 3 |  | SSB.2 RedCap FR1 |  |
|  |  |  | Config 4 |  | SSB.3 FR1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  | Config 3 |  | SMTC.1 |  |
|  |  |  | Config 4 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz |  |
|  |  |  | Config 4 |  | 15 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | PRACH.1 | Table A.7-1 |
|  |  |  | Config 3 |  | PRACH.1 | Table A.7-1 |
|  |  |  | Config 4 |  | PRACH.1 | Table A.7-1 |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  | 2 |  |
|  |  | Aggregation level |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | A.5 |
| Gap pattern ID |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | Absent | When the field is absent, the UE applies the value 0. (38.133 [6] Table 8.1.1-1). |
| Rsrp-ThresholdSSB | Config 1, 2 |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  | -95 |  |
|  | Config 4 |  |  |  | -98 |  |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
|  |  | Config 4 |  |  | CSI-RS.1.1 FDD |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
|  |  | Config 4 |  |  | TRS.1.1 FDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 5.17 |  |
| T3 |  |  |  | s | 3.24 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

16.5.2.4.4.2	Test Procedure

Same as the test procedure given in clause 6.5.5.2.4.2.

16.5.2.4.4.3	Message Contents

Same as the message contents given in clause 6.5.5.2.4.3.

16.5.2.4.5	Test Requirement

Same as the test requirements given in clause 6.5.5.2.5 with following exceptions:

-	Table 6.5.5.2.5-1 is replaced by Table 16.5.2.4.5-1.

Table 16.5.2.4.5-1: Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 4 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 4 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 4 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 4 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting is assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS Res. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. |  |  |  |  |  |  |  |

<< Skipped sections >>

#### 16.5.3.1	DCI-based and time-based active BWP switch for RedCap

##### 16.5.3.1.0	Minimum conformance requirements

16.5.3.1.0.1	Minimum conformance requirements for DCI-based and time-based active BWP switch for RedCap

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where,

-	Y=0, if the serving cell where UE receives DCI for BWP switch request is same as the serving cell on which BWP switch occurs.

-	Y equals to the length of 1 slot, if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. In this scenario, TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change.

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n except DCI triggering BWP switch on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [13] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay after bwp-InactivityTimer [13] expires on the cell where timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 16.5.3.1.0.1-1.

Table 16.5.3.1.0.1-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in 38.133 [6] Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in 38.133 [6] Clause 8.10 in the new BWP.

If the BWP switch is triggered within or outside DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 12], UE shall be able to complete active BWP switching within the time duration of

-	TdormantBWPswitchDelay =TBWPswitchDelay+ X, provided that the dormancy indication is received in any of the first 3 OFDM symbols of a slot in the serving cell where DCI for dormancy indication is received, or

-	TdormantBWPswitchDelay =TBWPswitchDelay + X + Z, provided that the dormancy indication is received after the first 3 OFDM symbols of a slot in the serving cell where DCI for dormancy indication is received, where

-	TBWPswitchDelay is defined in Table 16.5.3.1.0.1-1 corresponding to the smallest value among the SCS of the serving cell where UE receives dormancy indication and the SCSs of the dormant BWP and the active BWP immediately before or after switching the BWP of the serving cell where BWP switching occurs.

-	X equals to the length of 1 slot corresponding to the smallest value among the SCS of the serving cell where UE receives dormancy indication and the SCSs of the dormant BWP and the active BWP immediately before or after switching the BWP of the serving cell where BWP switching occurs.

-	Z equals to the length of 1 slot corresponding to the SCS of the serving cell where UE receives dormancy indication.

For DCI-based BWP switch, if the new BWP is a dormant BWP, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive CSI-RS (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL slot occurs right after a time duration of TdormantBWPswitchDelay which starts from the beginning of DL slot n.

The normative reference for this requirement is TS 38.133 [6] clauses 8.6A.2

##### 16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

16.5.3.1.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for 1Rx RedCap defined in TS 38.133 [6] clause 8.6A.

16.5.3.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2.

16.5.3.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.3.0.1 and 16.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.1.1.

16.5.3.1.1.4	Test description

Same as the test description given in clause 6.5.6.1.2.4.

16.5.3.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.1.2.4.1 with following exceptions:

-	Table 6.5.6.1.2.4.1-1 is replaced by Table 16.5.3.1.1.4.1-1.

-	Table 6.5.6.1.2.4.1-2 is replaced by Table 16.5.3.1.1.4.1-2.

-	Table 6.5.6.1.2.4.1-3 is replaced by Table 16.5.3.1.1.4.1-3.

Table 16.5.3.1.1.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.1.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.1.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.1.1.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.1.1.4.1-3: General test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

16.5.3.1.1.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.1.2.4.2.

16.5.3.1.1.4.3	Message Contents

Same as the message contents given in clause 6.5.6.1.2.4.3.

16.5.3.1.1.5	Test Requirement

Tables 16.5.3.1.1.4.1-3 and 16.5.3.1.1.5-1 define the primary level settings including test tolerances.

Table 16.5.3.1.1.5-1: Cell specific test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1,4 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
| TDD configuration |  |  | Config 1,4 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
|  |  |  | Config 4 |  | 10 MHz: NRB,c = 52 |
| Active BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2,3,4 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  |  | Config 1,2,3,4 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  |  | Config 1,2,3,4 |  | DLBWP.1.1 RedCap Note 4 |
| Initial UL BWP Configuration |  |  | Config 1,2,3,4 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  |  | Config 1,2,3,4 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  |  | Config 1,4 |  | N/A |
|  |  |  | Config 2,3 |  | ULBWP.1.1 RedCap Note 4 |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | SR.1.1 FDD |
|  |  |  | Config 2 |  | SR.1.1 TDD |
|  |  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1,4 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1,4 |  | CCR.1.2 FDD |
|  |  |  | Config 2 |  | CCR.1.2 TDD |
|  |  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x1 Low |
| TRS Configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2,4 |  |  | dBm/SCS | -104 |
|  | Config 3 |  |  |  | -101 |
| NocNote 2 |  |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2,4 |  |  | dBm/SCS | -87 |
|  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2,4 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.1 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 16.5.3.1.0.1-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### 16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

16.5.3.1.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for 2Rx RedCap defined in TS 38.133 [6] clause 8.6A.

16.5.3.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2.

16.5.3.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.1.2.

16.5.3.1.2.4	Test description

Same as the test description given in clause 6.5.6.1.2.4.

16.5.3.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.1.2.4.1 with following exceptions:

-	Table 6.5.6.1.2.4.1-1 is replaced by Table 16.5.3.1.2.4.1-1.

-	Table 6.5.6.1.2.4.1-2 is replaced by Table 16.5.3.1.2.4.1-2.

-	Table 6.5.6.1.2.4.1-3 is replaced by Table 16.5.3.1.2.4.1-3.

Table 16.5.3.1.2.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.1.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.1.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.1.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.1.2.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.1.2.4.1-3: General test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

16.5.3.1.2.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.1.2.4.2.

16.5.3.1.2.4.3	Message Contents

Same as the message contents given in clause 6.5.6.1.2.4.3.

16.5.3.1.2.5	Test Requirement

Tables 16.5.3.1.2.4.1-3 and 16.5.3.1.2.5-1 define the primary level settings including test tolerances.

Table 16.5.3.1.2.5-1: Cell specific test parameters for NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1,4 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
| TDD configuration |  |  | Config 1,4 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
|  |  |  | Config 4 |  | 10 MHz: NRB,c = 52 |
| Active BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2,3,4 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  |  | Config 1,2,3,4 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  |  | Config 1,2,3,4 |  | DLBWP.1.1 RedCap Note 4 |
| Initial UL BWP Configuration |  |  | Config 1,2,3,4 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  |  | Config 1,2,3,4 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  |  | Config 1,4 |  | N/A |
|  |  |  | Config 2,3 |  | ULBWP.1.1 RedCap Note 4 |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | SR.1.1 FDD |
|  |  |  | Config 2 |  | SR.1.1 TDD |
|  |  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1,4 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1,4 |  | CCR.1.2 FDD |
|  |  |  | Config 2 |  | CCR.1.2 TDD |
|  |  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2,4 |  |  | dBm/SCS | -104 |
|  | Config 3 |  |  |  | -101 |
| NocNote 2 |  |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2,4 |  |  | dBm/SCS | -87 |
|  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2,4 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.1 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 16.5.3.1.0.1-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### 16.5.3.2	RRC-based active BWP switch for RedCap

##### 16.5.3.2.0	Minimum conformance requirements

16.5.3.2.0.1	Minimum conformance requirements for RRC-based active BWP switch for RedCap

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where

DL slot n is the last slot overlapping with the PDSCH containing the RRC command, and

NR Slot length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves changing of SCS.

TRRCprocessingDelay is the length of the RRC procedure delay in ms as defined in clause 11.2 in TS 36.331 [29] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the length of the RRC procedure delay in ms as defined in clause 12 in TS 38.331 [13], and

TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs. When  THARQ> TRRCprocessingDelay a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [8].

The normative reference for this requirement is TS 38.133 [6] clauses 8.6A.3.

##### 16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

16.5.3.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6A.

16.5.3.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs.

16.5.3.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.3.0.1 and 16.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.2.1.

16.5.3.2.1.4	Test description

Same as the test description given in clause 6.5.6.2.1.4.

16.5.3.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.1.2.4.1 with following exceptions:

-	Table 6.5.6.2.1.4.1-1 is replaced by Table 16.5.3.2.1.4.1-1.

-	Table 6.5.6.2.1.4.1-2 is replaced by Table 16.5.3.2.1.4.1-2.

-	Table 6.5.6.2.1.4.1-3 is replaced by Table 16.5.3.2.1.4.1-3.

Table 16.5.3.2.1.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.2.1.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.2.1.4.1-3: General test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

16.5.3.2.1.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.2.1.4.2.

16.5.3.2.1.4.3	Message Contents

Same as the message contents given in clause 6.5.6.2.1.4.3.

16.5.3.2.1.5	Test Requirement

Tables 16.5.3.2.1.4.1-3 and 16.5.3.2.1.5-1 define the primary level settings including test tolerances.

Table 16.5.3.2.1.5-1: Cell specific test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1,4 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1,4 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 20 MHz: NRB,c = 51 |
| Active BWP ID |  |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,2, 3,4 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3,4 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | DLBWP.1.1 RedCap |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | ULBWP.1.1 RedCap |
| Final Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  |  | CCR.1.2 FDD |
|  |  | Config 2 |  |  | CCR.1.2 TDD |
|  |  | Config 3 |  |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.1 TDD |
| Antenna Configuration |  |  |  |  | 1x1 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.1 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

##### 16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

16.5.3.2.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6A.

16.5.3.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards supporting BWP adaptation of at least 2 BWPs.

16.5.3.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.3.2.2.

16.5.3.2.2.4	Test description

Same as the test description given in clause 6.5.6.2.1.4.

16.5.3.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.6.2.1.4.1 with following exceptions:

-	Table 6.5.6.2.1.4.1-1 is replaced by Table 16.5.3.2.2.4.1-1.

-	Table 6.5.6.2.1.4.1-2 is replaced by Table 16.5.3.2.2.4.1-2.

-	Table 6.5.6.2.1.4.1-3 is replaced by Table 16.5.3.2.2.4.1-3.

Table 16.5.3.2.2.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Configuration | Description |
| --- | --- |
| 16.5.3.2.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.3.2.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.2-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.3.2.2-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.5.3.2.2.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.3.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.3.2.2.4.1-3: General test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

16.5.3.2.2.4.2	Test Procedure

Same as the test procedure given in clause 6.5.6.2.1.4.2.

16.5.3.2.2.4.3	Message Contents

Same as the message contents given in clause 6.5.6.2.1.4.3.

16.5.3.2.2.5	Test Requirement

Tables 16.5.3.2.2.4.1-3 and 16.5.3.2.2.5-1 define the primary level settings including test tolerances.

Table 16.5.3.2.2.5-1: Cell specific test parameters for NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1,4 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1,4 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 20 MHz: NRB,c = 51 |
| Active BWP ID |  |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,2, 3,4 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3,4 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | DLBWP.1.1 RedCap |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | ULBWP.1.1 RedCap |
| Final Condition | Active DL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3,4 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  |  | CCR.1.2 FDD |
|  |  | Config 2 |  |  | CCR.1.2 TDD |
|  |  | Config 3 |  |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.1 TDD |
| Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2,4 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2,4 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.2 RedCap is linked with ULBWP.1.1 RedCap defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Changes >>