3GPP TSG-RAN5 Meeting #100	R5-234694

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2881 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for establishing unicast mode ProSe Direct communication / Peer UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test procedure for establishing unicast mode ProSe Direct communication / Peer UE side is missing in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test procedure for establishing unicast mode ProSe Direct communication / Peer UE side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases of NR sidelink U2N relay will be not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508 CR 2882, 2883 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.9.Y	Test procedure for establishing unicast mode ProSe Direct communication / Peer UE side

4.9.Y.1	Scope

The purpose of this procedure is for establishing unicast mode ProSe Direct communication / Peer UE side.

4.9.Y.2	Procedure description

4.9.Y.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.Y.2.2	Procedure sequence

Table 4.9.Y.2.2-1: Procedure for establishing unicast mode ProSe Direct communication / Peer UE side

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The NR-SS-UE sends a PROSE DIRECT LINK ESTABLISHMENT REQUEST message. | <-- | PC5-S: PROSE DIRECT LINK ESTABLISHMENT REQUEST |
| 2 | The UE sends a PROSE DIRECT LINK SECURITY MODE COMMAND message. | --> | PC5-S: PROSE DIRECT LINK SECURITY MODE COMMAND |
| 3 | The NR-SS-UE sends a PROSE DIRECT LINK SECURITY MODE COMPLETE message. | <-- | PC5-S: PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 4 | The UE sends a PROSE DIRECT LINK ESTABLISHMENT ACCEPT message. | --> | PC5-S: PROSE DIRECT LINK ESTABLISHMENT ACCEPT |
| 5 | The NR-SS-UE sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink |
| 6 | The UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5-RRC: RRCReconfigurationCompleteSidelink |

4.9.Y.3	Specific message contents

All specific message contents shall be according subclause 4.6 and 4.7B.

Table 4.9.Y.3-1: PROSE DIRECT LINK ESTABLISHMENT REQUEST (Table 4.9.Y.2.2-1, Step 1)

Derivation Path: Table 4.7.7-1 with condition Rx

Table 4.9.Y.3-2: PROSE DIRECT LINK SECURITY MODE COMMAND (Table 4.9.Y.2.2-1, Step 2)

Derivation Path: Table 4.7.7-2 with condition Tx

Table 4.9.Y.3-3: PROSE DIRECT LINK SECURITY MODE COMPLETE (Table 4.9.Y.2.2-1, Step 3)

Derivation Path: Table 4.7.7-14 with condition Rx

Table 4.9.Y.3-4: PROSE DIRECT LINK ESTABLISHMENT ACCEPT (Table 4.9.Y.2.2-1, Step 4)

Derivation Path: Table 4.7.7-2 with condition Tx

Table 4.9.Y.3-5: RRCReconfigurationSidelink (Table 4.9.23.2.2-1, Step 5)

Derivation Path: Table 4.6.1A-3 with condition RX and SL_DRB

Table 4.9.Y.3-5: RRCReconfigurationCompleteSidelink (Table 4.9.23.2.2-1, Step 6)

Derivation Path: Table 4.6.1A-4 with condition TX