3GPP TSG-RAN5 Meeting #100	R5-234696

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2883 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Generic procedures for NR sidelink U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Generic procedure of NR sidelink U2N Relay UE should be clarified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding of generic procedure of NR sidelink U2N Relay UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Remote UE will not be implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | '4.9.X' shall be resolved according to TS38.508-1 CR 2880 and '4.9.Y' shall be resolved according to TS38.508-1 CR 2881 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.5.8	NR sidelink U2N Relay

4.5.8.1	Initiation

The SS shall:

1>	If sidelink is On

2>	use neither NR cell nor E-UTRA cell;

2>	use 1 GNSS simulator;

2>	use 1 NR-SS-UE;

2>perform the procedure according to the table 4.5.8.2-1: NR sidelink U2N Relay;

4.5.8.2	Procedures

Table 4.5.8.2-1: NR sidelink U2N Relay

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Same as table 4.5.7.2-1, steps 1a1-1A. | - | - | - | - |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.X is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.Y is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3a1-4 | Same as table 4.5.7.2-1, steps 3a1-4. | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message. | --> | PC5-RRC: RRCSetupRequest | - | - |
| 6 | The NR-SS-UE transmits an RRCSetup message. | <-- | PC5-RRC: RRCSetup | - | - |
| 7 | The UE transmits an RRCSetupComplete message. | --> | PC5-RRC: RRCSetupComplete | - | - |
| 8 | The NR-SS-UE transmits a SecurityModeCommand message. | <-- | PC5-RRC: SecurityModeCommand | - | - |
| 9 | The UE transmits a SecurityModeComplete message. | --> | PC5-RRC: SecurityModeComplete | - | - |
| 10 | The NR-SS-UE transmits an RRCReconfiguration message. | <-- | PC5-RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message. | --> | PC5-RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 8a1-8a3 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 12a1 | IF connected without release is not present THEN, the NR-SS-UE transmits an RRCRelease message. | <-- | PC5-RRC: RRCRelease | - | - |
| 12a2 | The UE sends a RemoteUEInformationSidelink message. | --> | PC5-RRC: RemoteUEInformationSidelink | - | - |
| 12a3 | The NR-SS-UE transmits a UuMessageTransferSidelink message. | <-- | PC5-RRC: UuMessageTransferSidelink | - | - |

4.5.8.3	Specific message contents

All specific message contents shall be according subclause 4.7B with the exceptions below.

Table 4.5.8.3-1: Message RRCSetup (steps 6, Table 4.5.8.2-1)

Derivation path: Table 4.6.1-21 with condition L2RemoteUE

Table 4.5.8.3-2: Message RRCReconfiguration (steps 10, Table 4.5.8.2-1)

Derivation path: Table 4.6.1-13 with condition L2RemoteUE and RELAY

Table 4.5.8.3-3: Message RRCRelease (steps 12a1, Table 4.5.8.2-1)

Derivation path: Table 4.6.1-16 with condition L2RemoteUE

<End of Changed Section>