3GPP TSG-RAN5 Meeting # 100	R5-234943

# Tolouse, France, 21st August 2023 – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2901 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test frequencies for band n24 and asymmetric channel bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for band n24 and asymmetric channel bandwidths are not following uniform criteria defined in R5-234935. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for band n24 and asymmetric channel bandwidths are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing frequencies for band n24 will not be correct or complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-234935. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 4.3.1.1.1.24	Reference test frequencies for NR operating band n24

Table 4.3.1.1.1.24-1a: Test frequencies for NR operating band n24,SCS 15 kHz and ΔFRaster 15 kHz

| UL/DL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1531 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1632.5 | 326500 | 1539.53 | 307906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |  | - | - | - | - | - | - |
| 10/5 ) | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1531 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1632.5 | 326500 | 1537.1 | 307420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |
| 10/10 |  | 52 |  | Downlink |  | Low |  | 1531.0 |  | 306200 |  | 1526.32 |  | 305264 |  | 0 |  | 15 |  | 3823 |  | 305810 |  | 2 |  | 1 |  | 2 (4) |  | 5 |  |
|  |  |  |  |  |  | Mid |  | 1531.0 |  | 306200 |  | 1507.96 |  | 301592 |  | 102 |  |  |  | 3823 |  | 305810 |  | 2 |  | 1 |  | 2 (4) |  | 107 |  |
|  |  |  |  |  |  | High |  | 1531.0 |  | 306200 |  | 1435.6 |  | 287120 |  | 504 |  |  |  | 3823 |  | 305810 |  | 2 |  | 1 |  | 2 (4) |  | 509 |  |
|  |  |  |  | Uplink |  | Low |  | 1632.5 |  | 326500 |  | 1627.82 |  | 325564 |  | 0 |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  |  |  |  |  | Mid |  | 1632.5 |  | 326500 |  | 1537.1 |  | 307420 |  | 504 |  |  |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  |  |  |  |  | High |  | 1651.5 |  | 330300 |  | 1645.74 |  | 329148 |  | 6 |  |  |  | - |  | - |  | - |  | - |  | - |  | - |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-2a: Test frequencies for NR operating band n24, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 |  |  | 24 |  |  | Downlink |  |  | Low |  |  | 1531.0 |  |  | 306200 |  |  | 1526.68 |  |  | 305336 |  |  | 0 |  |  | 30 |  |  | 3829 |  |  | 306290 |  |  | 6 |  |  | 0 |  |  | 3 (3) |  |  | 6 |  |  |
|  |  |  |  |  |  |  |  |  | Mid |  |  | 1531.0 |  |  | 306200 |  |  | 1489.96 |  |  | 297992 |  |  | 102 |  |  |  |  |  | 3829 |  |  | 306290 |  |  | 6 |  |  | 0 |  |  | 3 (3) |  |  | 210 |  |  |
|  |  |  | High | 1531.0 | 306200 | 1345.24 | 269048 | 504 |  | 3829 | 306290 | 6 | 0 | 3 (3) | 1014 |
|  |  |  |  |  |  | Uplink |  |  | Low |  |  | 1632.5 |  |  | 326500 |  |  | 1628.18 |  |  | 325636 |  |  | 0 |  |  | - - |  |  | - |  |  | - |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
|  |  |  | Mid | 1632.5 | 326500 | 1446.74 | 289348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  |  | High |  |  | 1651.5 |  |  | 330300 |  |  | 1645.02 |  |  | 329004 |  |  | 6 |  |  |  |  |  | - |  |  | - |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Low and Mid test frequencies are specified using Tx-Rx spacing of -101.5 MHz and High test frequency is specified using Tx-Rx spacing of -120.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-3a: Test frequencies for NR operating band n24, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low Mid | 1531.0 | 306200 | 1527.04 | 305408 | 0 | 15 | - | 305768 |
|  |  |  | Mid | 1531.0 | 306200 | 1453.6 | 290720 | 102 |  |  | 305768 |
|  |  |  | High | 1531.0 | 306200 | 1164.16 | 232832 | 504 |  | - | 305768 |
|  |  | Uplink | Low | 1632.5 | 326500 | 1628.54 | 325708 | 0 | - | - | - |
|  |  |  | Mid | 1632.5 | 326500 | 1265.66 | 253132 | 504 |  | - | - |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |  | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: Low and Mid test frequencies are specified using Tx-Rx spacing of -101.5 MHz and High test frequency is specified using Tx-Rx spacing of -120.5 MHz |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>