3GPP TSG-RAN5 Meeting #100	R5-234960

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

Source:	MediaTek Inc.

Title:	Discussion on handling SIB configuration for NGSO

# Document for:	Endorsement

Agenda Item:	6.3.23.5

1.	Introduction and Background

As defined in TS 36.300 [1] Clause 4.12, support of non-terrestrial network (NTN) typically includes GSO and NGSO satellites.

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

And in TS 36.508 [2], System information blocks, especially SIB-31 for eMTC and SIB31-NB for NB-IoT, have already specified for GSO. However, the SIBs for NGSO are still FFS in TS 36.508 [2] and the specific test cases.

2.	Discussion

This discussion paper aims to provide a way to define the SIB configuration of NGSO satellite for IoT NTN.

As NGSO includes both LEO and MEO, considering the current industrial deployment and the characteristic of propagation channel, LEO should be the proper satellite platform to satisfy the requirements of IoT NTN use case. Therefore, in this discussion paper only LEO is discussed and MEO is not in the scope.

In TS 36.331 [3], the ephemeris within SIB31(-NB) is defined as below:

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

}

EphemerisStateVectors-r17 ::=	SEQUENCE {

positionX-r17					PositionStateVector-r17,

positionY-r17					PositionStateVector-r17,

positionZ-r17					PositionStateVector-r17,

velocityVX-r17					VelocityStateVector-r17,

velocityVY-r17					VelocityStateVector-r17,

velocityVZ-r17					VelocityStateVector-r17

}

EphemerisOrbitalParameters-r17 ::= SEQUENCE {

semiMajorAxis-r17					INTEGER (0..8589934591),

eccentricity-r17					INTEGER (0..1048575),

periapsis-r17						INTEGER (0..268435455),

longitude-r17						INTEGER (0..268435455),

inclination-r17						INTEGER (-67108864..67108863),

anomaly-r17							INTEGER (0..268435455)

}

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are only valid for the duration as defined by ul-SyncValidityDuration and epochTime.

stateVectors Instantaneous values of the satellite state vectors. The signalled values are only valid for the duration as defined by ul-SyncValidityDuration and epochTime.

However, there is no clear requirement/guideline on how to define ephemeris for GSO or NGSO, so the stateVectors IE should be applicable to both GSO and NGSO. Using stateVectors rather than orbitalParameters for NGSO could be feasible to align with current SIB configuration of GSO.

Proposal #1: With the implementation complexity of System simulator (SS) considered, stateVectors is proposed to be used for NGSO/LEO.

For stateVectors IE, as the LEO moves fast, the satellite position and corresponding parameters, such as, common TA and Koffset, change frequently. So the SS needs to update the SIB-31 when needed.

As the UE position is stationary as defined in TS 36.508 [1] and the moving speed is usually known when the satellite height is fixed, a look-up table is proposed to be used for the SS to estimate the satellite position/velocity and then to update the SIB31.

Below is an example of look-up table for LEO (600 km), T1 is the starting time of service available and T2 is the time the satellite flies away.

Table 1: satellite position table for a 600 km LEO.

If the time the SS needs to update SIB31 is not in this table, SS should find the two nearest timestamp and their related positions and velocities. Then using such information and the satellite speed to calculate the exact position and velocity. Figure 1 illustrates how the SS estimate the ephemeris information.

The output should be like below figure 2.

Proposal #2: A look-up table to be used to update the ephemeris information for LEO.

3.	Proposal/Way Forward

In the contribution, we discuss the analysis of applicable legacy eMTC cases for IoT NTN with following proposals:

Proposal #1: With the implementation complexity of System simulator (SS) considered, stateVectors is proposed to be used for NGSO.

Proposal #2: A look-up table of position and velocity stored in SS to be used to update the ephemeris information for LEO.

4.	References

[1]	TS 36.300: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2; (cl. 4.12)

[2]	TS 36.508: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing

[3]	TS 36.331: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification