3GPP TSG-RAN5 Meeting #100	R5-235327

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

# Source:	Qualcomm Incorporated, Rohde & Schwarz, Mediatek

# Title:	Applicability of legacy NB-IOT testcases to NTN GSO only UE

# Document for:	Discussion

## 1.	Introduction

Discussion paper [3] focused on NB-IOT NTN GSO scenario categorized two types of UEs (Type 1 UE – supporting NB-IOT legacy features and Release 17 NB-IOT NTN GSO scenario, Type 2 UE – supporting only NB-IOT NTN GSO scenario) and provided preliminary analysis of adapted legacy test cases to a Type 2 UE.

Key points from discussion paper [3]:

- Summary of agreed execution guidelines:

- Type 1 UE (supporting both TN and NTN)

  - Release 17 NB-IoT test cases in NTN bands.

  - Release 13 to 15 NB-IoT test cases in TN bands.

- Type 2 UE (supporting only NTN)

  - Release 17 NB-IoT test cases in NTN bands.

  - Adapted test cases (from Release 13 to 15) in NTN bands.

This discussion paper aims to complete the analysis and take further steps to conclude on the adapted legacy test case list for a Type 2 UE.

## 2.	Discussion

### 2.1	Adapted legacy test cases for Type 2 UE

As mentioned in Proposal-1.1 of discussion paper [3], spreadsheet attached with this discussion paper includes the list of adapted legacy test cases considering typical NB-IOT NTN GSO deployments and industry needs.

Test cases falling under the below listed categories are considered “Not Applicable” for a Type 2 UE.

- All non-approved (‘compliable’, ‘verifiable’) test cases as per TTCN CR Agreement Status 2023-wk16.

- Test cases having corresponding NTN Release 17 variant (ex: t-reordering)

- As NTN GSO cell area is expected to be huge, NTN UE may not perform PLMN changes and cell reselections. Hence, most of the Idle mode operation test cases (22.2.x) would be non-relevant.

- Majority of the test cases having additional applicabilities (Ex: S1-U, ROHC support, LAP etc) based on existing deployments.

- Test cases with negative scenarios (ex: TAU Reject, abnormal detach) not exercised in real deployments.

In addition to the test cases listed in [3] few more test cases listed below have been considered applicable for NB-IoT NTN GSO UE:

- Access Class: as the NTN cell has a much larger coverage than TN cell, network operator may use AC to bar some UEs or for different service categories. The impacted cases include 22.4.4/5/8/9. New system information combinations to be added in [8] and newly proposed test cases to be added in applicability list defined in [6].

- abnormal procedure: some abnormal procedure may happen in real network and impact the service experience, such as Attach reject or service request reject. The impacted cases include 22.5.1/2/3/4/6/9. Newly proposed test cases to be added in applicability list defined in [6].

### 2.2	UE capability audit test case

Based on the agreed execution guidelines, Type 1 UE (supporting both TN and NTN) lacks NTN coverage for UE Capability audit test case as the execution happens in TN bands.

## 3.   Proposal/Way Forward

- New NTN system information combinations shall be defined in [4] for adapted legacy test cases. This is handled in [5] submitted in this meeting.

- Changes related to NTN test environment shall be added to identified adapted legacy test cases and CRs [7] [8] [9] are submitted in this meeting.

- As RTT delays are huge for NTN environment, few L2 test cases of [1] needs additional test case specific updates to cater RTT delays. CR [10] has been submitted in this meeting.

- All identified adapted legacy test cases shall be added in clause 5 of [2]. CR [6] has been submitted to [2] in this meeting.

- Define new NTN UE Capability audit test case 22.4.13a. If agreed, this will be handled in RAN5#101.

- Spreadsheet attached with this discussion paper contains the list of adapted test cases and needs endorsement.

## 4.	References

[1]	TS 36.523-1	LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet  Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

[2]	TS 36.523-2	LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification

[3] 	R5-233495	Applicability of legacy NB-IOT testcases to NTN GSO capable UE

[4]	TS 36.508	LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing

[5] 	R5-235106	System information updates for NB-IOT NTN UEs

[6] 	R5-235109	Applicability updates to NB-IoT NTN GSO only UEs

[7] 	R5-235111	NB-IOT NTN updates to idle mode test cases

[8] 	R5-235113	NB-IOT NTN updates to RRC test cases

[9] 	R5-235116	NB-IOT NTN updates to EMM test cases

[10] 	R5-235118	Correction to RLC test cases for NTN UEs

## Annex

Key points from discussion paper [3]:

- Summary of agreed execution guidelines:

- Type 1 UE (supporting both TN and NTN)

  - Release 17 NB-IoT test cases in NTN bands.

  - Release 13 to 15 NB-IoT test cases in TN bands.

- Type 2 UE (supporting only NTN)

  - Release 17 NB-IoT test cases in NTN bands.

  - Adapted test cases (from Release 13 to 15) in NTN bands.