3GPP TSG-RAN5 Meeting #100	R5-235360

# Toulouse, FR, Aug. 21st - 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3865 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5.3.2 for Rel-17 eNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of Rel-17 eNPN TC 6.5.3.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Rel-17 eNPN TC 6.5.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNPN WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0368 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impacts |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version revised from R5-233990r2 |  |  |  |  |  |  |  |  |

6.5.3.2	SNPN Selection in Manual Mode / Onboarding services in SNPN

6.5.3.2.1	Test Purpose (TP)

(1)

with { UE in Manual network selection mode and an SNPN not supporting onboarding services is available }

ensure that {

when { UE is switched on or return to coverage }

then { UE does not indicate to the user any available SNPN }

}

(2)

with { UE in Manual network selection mode }

ensure that {

when { an SNPN indicating that onboarding is allowed is available and matches the onboarding SNPN selection information }

then { UE indicates user the SNPN and user selects the SNPN }

}

6.5.3.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clauses 4.9.3.1.4. Unless otherwise stated these are Rel-17 requirements.

[TS 23.122, clause 4.9.3.1.4]

The MS shall indicate to upper layers one or more SNPNs, which are available and indicate that onboarding is allowed.

These include SNPNs in the list of "permanently forbidden SNPNs" for onboarding services and the list of "temporarily forbidden SNPNs" for onboarding services. The MS may indicate to the user whether the available SNPNs are present in the list of "temporarily forbidden SNPNs" or the list of "permanently forbidden SNPNs".

The MS shall limit its search for the SNPN to the NG-RAN access technology.

For each SNPN indicated to upper layers, the MS shall indicate to the upper layers along with the SNPN identity:

a)	whether the SNPN matches the onboarding SNPN selection information, if pre-configured; and

b)	the human-readable network name, if the system information broadcast includes the human-readable network name for the SNPN.

6.5.3.2.3	Test description

6.5.3.2.3.1	Pre-test conditions

System Simulator:

-	2 SNPN cells: NR Cell 1and NR Cell 2 are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-26 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Manual SNPN selection mode.

-	The UE is provisioned default UE credentials and is pre-configured with onboarding SNPN selection information to allow access to SNPN identified by NR Cell 2.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN, and the UE is in state Switched OFF (state 0-A).

6.5.3.2.3.2	Test procedure sequence

Table 6.5.3.2.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 6.5.3.2.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.3.2.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |

Table 6.5.3.2.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | “Off” |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -91 |  |

Table 6.5.3.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.5.3.2.3.2-1/2. | - | - | - | - |
| 2 | The UE is Switched ON. | - | - | - | - |
| 3 | Manual selection of SNPN identified by NR Cell 1 is performed (NOTE 1). | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCSetupRequest on NR Cell 1? | --> | RRCSetupRequest | 1 | F |
| 5 | SS adjusts cell levels according to row T2 of table 6.5.3.2.3.2-1/2. | - | - | - | - |
| 6 | Manual selection of SNPN identified by NR Cell 2 is performed (NOTE 1). | - | - | - | - |
| 7 | Check: Does the UE send a RRCSetupRequest on NR Cell 2? | --> | RRCSetupRequest | 2 | P |
| 8-25 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NR Cell 2. | - | - | - | - |
| NOTE 1: This is performed via MMI or AT command. |  |  |  |  |  |

6.5.3.2.3.3	Specific message contents

Table 6.5.3.2.3.3-1: REGISTRATION REQUEST (step 9, Table 6.5.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘101’B |  | SNPN_ONBOARDING |

Table 6.5.3.2.3.3-2: SIB1 for NR Cell 1 (all steps, Table 6.5.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition eNPN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17{ | 1 entry |  |  |
| SNPN-AccessInfo-r17[1] SEQUENCE { |  | entry 1 | eNPN |
| onboardingEnabled-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.2.3.3-3: SIB1 for NR Cell 2 (all steps, Table 6.5.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition eNPN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17{ | 1 entry |  |  |
| SNPN-AccessInfo-r17[1] SEQUENCE { |  | entry 1 | eNPN |
| onboardingEnabled-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |