3GPP TSG-RAN5 Meeting #100	R5-235428

# Toulouse, France,  21 th Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5226 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NTN eMTC TC 8.5.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, MediaTek Inc., ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an inconsistency in the UE test loop mode procedure and the PDU types to be send and looped back. At steps 3-20 the procedure to put UE to TESTLOOP MODE A is performed. Due to this SS should transmit PDCP PDUs. After expiry of T317, UE in connected mode needs to acquire SIB31-NB as defined in TS 36.331 clause 5.3.18, this will cause missing PDSCH data, therefore steps 20 & 23, 24D & 24E should be switched. Uplink grant is already configured before sending of ESM data and therefore the UE will loop back the data once it has received, in order to wait for the expiry of timer, the SS grant shall be stopped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 8.5.6.1.3.2-1 replaced references to MAC PDU by PDCP PDU. Steps 22 & 25, 26D & 26E were swapped. Conformance requirements updated. Added Steps 21A and 28B to stop and to restore the periodic uplink grant allocation. After step 25, as T317 expires, UE needs to acquire SIB31 as mentioned in TS 36.331 cl. 5.3.18. Then UE may miss the single UL grant in step 26. Therefore it is proposed to issue the periodic grant instead. Step 26CA was added to stop the corresponding transmission. To stiffen the test case, step26F is modified to transmit a periodic grant. Removed step 28B that became redundant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case prose will remain inconsistent and the test case cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-233825. |  |  |  |  |  |  |  |  |

#### 8.5.6.1	eMTC / NTN / Ephemeris information update / T317 Expiry / T318 Expiry

8.5.6.1.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType31 and starts timer T317 with the duration ul-SyncValidityDuration }

ensure that {

when { T317 expires in RRC_CONNECTED state }

then { UE informs lower layers that the UL synchronisation is lost and start T318 }

}

(2)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { SystemInformationBlockType31 is acquired successfully }

then { UE stops T318 and informs lower layers that the UL synchronization is restored }

}

(3)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { T318 expires }

then { UE considers radio link failure and initiates the connection re-establishment procedure }

}

8.5.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.1.3, 5.2.2.4, 5.2.2.39, 5.3.18, 5.3.11.3 and TS 36.321 clause 5.2a. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.1.3]

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

[TS 36.331, clause 5.2.2.4]

The UE shall:

…

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

[TS 36.331, clause 5.2.2.39]

Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall:

1>	start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime.

[TS 36.331, clause 5.3.18]

The UE shall:

1>	if in RRC_CONNECTED:

2>	inform lower layers that the UL synchronisation is lost;

2>	start timer T318;

2>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2;

2>	upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

3>	stop timer T318;

3>	inform lower layers that the UL synchronisation is restored;

NOTE:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

[TS 36.331, clause 5.3.11.3]

The UE shall:

…

- upon T318 expiry; or

…

2>	consider radio link failure to be detected for the MCG i.e. RLF;

…

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.321, clause 5.2a]

If upper layer informs that the UL synchronization is lost according to the clause 5.3.18 of TS 36.331 [8], the MAC entity shall:

-	flush all HARQ buffers;

-	not perform any uplink transmission.

If upper layer informs that the UL synchronization is restored for the SpCell according to the clause 5.3.18 of TS36.331 [8], uplink transmissions can be performed.

8.5.6.1.3	Test description

8.5.6.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508 [18] clause 4.4.1.1 is configured.

-	System information combination 32 as defined in TS 36.508 [18] clause 4.4.3.1.1 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in in state Switched OFF (State 1) according to TS 36.508 [18] clause 4.5.1.

8.5.6.1.3.2	Test procedure sequence

Table 8.5.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | UE acquires SystemInformationBlockType31 and start T317 = 60 seconds. | - | - | - | - |
| 3-20 | Steps 1 to 18 of the registration and test mode activated procedure described in TS 36.508 [18] subclause 4.5.2A.3 with TEST LOOP MODE A are performed on Cell 1. Note: After receiving the message in step3, SS start a timer T1 with value = T317. | - | - | - | - |
| 20A-20B | Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 4.5.4.3 are performed on Cell 1. | - | - | - | - |
| 21 | SS stops broadcasting SystemInformationBlockType31. | - | - | - | - |
| 21A | SS stops periodic grant allocation | - | - | - | - |
| 22 | The SS transmits a PDCP PDU (Note 1). | <-- | PDCP PDU | - | - |
| 23-24 | Void | - | - | - | - |
| 25 | Wait until expiry of T1. Note: UE starts timer T318. SS start a timer T2 with value = T318 | - | - | - | - |
| 26 | The SS starts the UL periodic grant, allowing the UE to return the PDCP PDU as received in step 22. | - | - | - | - |
| 26A | SS resumes broadcasting SystemInformationBlockType31. Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. | - | - | - | - |
| 26B | Check: Does the UE transmit a PDCP PDU corresponding to grant in step 26? | --> | PDCP PDU | 2 | P |
| 26C | SS stops broadcasting SystemInformationBlockType31. | - | - | - | - |
| 26 CA | SS stops periodic grant allocation | - | - | - | - |
| 26D | The SS transmits PDCP PDU | <-- | PDCP PDU | - | - |
| 26E | Wait until expiry of T3. Note: UE starts timer T318. SS starts a timer T4 with value = T318. | - | - | - | - |
| 26F | The SS starts the UL periodic grant, allowing the UE to return the PDCP PDU as received in step 26D. | - | - | - | - |
| 27 | Check: Does the UE transmit a PDCP PDU corresponding to grant in step 26F? | --> | PDCP PDU | 1 | F |
| 28 | Wait until expiry of T4. | - | - | - | - |
| 28A | SS resumes broadcasting SystemInformationBlockType31. | - | - | - | - |
| 29 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on SRB0? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 30 | The SS transmits a RRCConnectionReestablishment message on SRB0 | <-- | RRCConnectionReestablishment | - | - |
| 31 | The UE transmits RRCConnectionReestablishmentComplete message on SRB1. | --> | RRCConnectionReestablishmentComplete | - | - |
| Note 1: This could be done near the expiry of T1 in step 25 and T3 in step 26E. |  |  |  |  |  |

8.5.6.1.3.3	Specific message contents

Table 8.5.6.1.3.3-1: Void

Table 8.5.6.1.3.3-2: SystemInformationBlockType31 (steps 2, 27A and 28A, Table 8.5.6.1.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ul-SyncValidityDuration-r17 | s60 | Value of T317 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.6.1.3.3-3: ACTIVATE TEST MODE (Step 20A, Table 8.5.6.1.3.2-1)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE A