3GPP TSG-RAN WG5 Meeting #100	R5-235678

Toulouse, France, 21st August 2023-25th August 2023

Title:	Delays in NR NTN communications

Source:	Keysight Technologies UK Ltd

Agenda item:	5.3.18.9

Document for:	Discussion and endorsement

# 1.	Introduction

3GPP has been discussing the delays present in Non-Terrestrial Networks Communications and the responsibilities of compensating such effect to enable the communication ([1]).

This document analyses the outcome of such discussions and proposes a way forward to progress on test specifications implications related to this topic.

This document is a re-submission from R5-233227 ([13]).

# 2. 	Discussion

## 2.1 Dynamic delay in service and feeder links

According to 3GPP TS 38.300 section 16.14.2.1 ([1]), in Non-Terrestrial Networks communications there will be delays associated either to the service link as well as to the feeder link (refer to Figure 1). DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset .

Figure 1: Illustration of timing relationship

To enable the communications, pre-compensation is required by the UE:

The following parameters will play a key role on achieving proper timing and synchronization:

-	Common TA is a configured offset that corresponds to the RTT between the Reference Point (RP) and the NTN payload.

According to 3GPP TS 38.211 ([2]) section 4.3.1 Uplink frame number i for transmission for the UE shall start at TTA before the start of the corresponding downlink frame at the UE:

Additionally, according to 3GPP TS 38.213 ([3]) section 4.2:

-	Koffset is a configured scheduling offset that need to be larger or equal to the sum of the service link RTT and the Common TA.

-	kmac is a configured offset that need to be larger or equal to the RTT between the RP and the gNB.

All these values are provided by the network in the specific SIB message for NTN (SIB19).

Additionally, these parameters are somehow related to the satellite orbit emulation, i.e. to the ephemeris of the satellite. In the test cases it is important that TE emulates appropriate delays in both feeder and service links according to information in NTN SIB message (SIB19). However, considering proposals in section 2.2, test equipment will need to emulate only delay in the service link.

Proposal 1: Test equipment shall emulate the delay only in service link according to information in NTN SIB message.

## 2.2 Kmac and Common TA parameters

As described in section 2.1 in this document, DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset .

Hence, there are two options for the definition of the reference point (RP):

- Option 1: DL and UL frame boundaries aligned at gNB (kmac=0, koffset=2xDelay)

- Option 2: DL and UL not aligned at gNB (kmac≠0, Koffset≠2xDelay)

Considering that:

- kmac needs to be larger than or equal to the RTT between the RP and the gNB (as per 3GPP TS 38.300 section 16.14.2.1 ([1])), having a value larger than RTT between the RP and the gNB would require a consensus on how larger kmac needs to be compared to the RTT between the RP and the gNB,

- According to R4-2214473 ([4]), during RRM discussions for NR NTN, it was indicated that “In order to alleviate the impact of uncertainty on UE uplink transmission timing due to quantized feeder link delay information, the feeder link delay can be set to zero.” so the agreement in RRM test cases was to use kmacas not configured, ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0 (as per R4-2303147 ([5])).

As the same uncertainty impact due to quantized feeder link delay might impact all test cases, it is proposed to define reference point so DL and UL frame boundaries are aligned at gNB (kmac=0 or not configured) and ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0. ta-Report seems to be not needed.

Proposal 2: Define Reference point so DL and UL frame boundaries are aligned at gNB (kmac=0 or not configured).

Proposal 3: Define ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0.

Proposal 4: Define ta-Report as Not configured.

## 2.3 Koffset

According to 3GPP TS38.300 ([1]), koffset is a configured scheduling offset that need to be larger or equal to the sum of the service link RTT and the Common TA. Having kmac=0 implies that koffset should be equal to or greater than twice of the satellite-UE one-way delay.

koffset (= kcell,offset - kUE,offset) , where kcell,offset is provided by cellSpecificKoffset and kUE,offset is provided by a Differential koffset MAC CE command; otherwise, if not respectively provided, kcell,offset = 0 or kUE,offset = 0 according to 3GPP TS 38.213 ([3]) section 4.2.

During RAN4 discussions, one analysis on the Satellite-UE RTT delay and associated koffset values was carried out (R4-2213860 ([6])) as shown below:

As it can be seen in the above analysis, koffset value is dependent on the type of satellite as well as on the minimum elevation angle (α in the figure above).

In RRM, according to R4-2303147 ([5]), the following agreement was achieved based on R4-2300775 ([7]):

The above agreement was based on 30º minimum elevation angle agreement in R4-2303295 ([8]):

R4-2119586 ([9]) and R4-2218407 ([10]) describe somehow the trade-off required between satellite visibility duration and performance (SNR/RSRP/SINR):

In demod test cases, as only PDSCH for NGSO was defined, the koffset value agreed was also 8 slots (refer to R4-2214386 ([11]) and R4-2217480 Issue 2-1 ([12])).

As according to [7], kUE,offset has been agreed as 0, it is proposed that same kcell,offset values agreed in demod and RRM are used for all test cases: 256 slots for GSO and 8 slots for NGSO.

Value for NGSO might require further updates for NGSO satellites in case it is necessary to maximize satellite visibility duration in some RF test cases.

Proposal 5: Use kcell,offset equal to 256 slots for GSO and 8 slots for NGSO in all test cases.

# 3. 	Conclusion

This document analyses discussions in different 3GPP working groups regarding the propagation delays in Non-Terrestrial Network communications and proposes a way forward to progress on test specifications implications related to this topic.

The following proposals are made:

Proposal 1: Test equipment shall emulate the delay only in service link according to information in NTN SIB message.

Proposal 2: Define Reference point so DL and UL frame boundaries are aligned at gNB (kmac=0 or not configured).

Proposal 3: Define ta-Common=ta-CommonDrift=ta-CommonDriftVariant=0.

Proposal 4: Define ta-Report as Not configured.

Proposal 5: Use kcell,offset equal to 256 slots for GSO and 8 slots for NGSO in all test cases.

# 4. 	References

- 3GPP TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2 (Release 17)”

- 3GPP TS 38.211, “NR; Physical channels and modulation (Release 17)”

- 3GPP TS 38.213, “NR; Physical layer procedures for control (Release 17)”

- R4-2214473, “WF on performance part for NR NTN”, Xiaomi, RAN4#104-e

- R4-2303147, “WF on Rel-17 NR NTN RRM Performance requirements”, Xiaomi, RAN4#106

- R4-2213860, “Discussion on UE NTN demod PDSCH”, Huawei, HiSilicon, RAN4#104-e

- R4-2300775, “Discussion on RRM test parameter configuration for NTN”, CMCC, RAN4#106

- R4-2303295, “draftCR on setup for NTN RRM test cases”, Huawei, HiSilicon, RAN4#106

- R4-2119586, “General and RRM requirements impacts”, Qualcomm Incorporated, RAN4#101-e

- R4-2218407, “Open Issues in UE and Satellite position details”, RAN4#105

- R4-2214386, “WF for NTN demodulation requirements - general and PDSCH”, Qualcomm Incorporated, RAN4#104-e

- R4-2217480, “Email discussion summary for [104-bis-e][317] NR_NTN_Demod_Part1”, Moderator (Qualcomm incorporated), RAN4-bis-e

- R5-233227, “Delays in NR NTN communications”, Keysight Technologies UK Ltd, RAN5#99