3GPP TSG-RAN5 Meeting #100	 R5-235912

# Toulouse, France, 21st August 2023 - 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2504 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR1 fast SCell activation test case for 160 ms SCell measurement cycle |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR SA FR1 fast Scell activation test case for 160 ms SCell measurement cycle is present in RAN 4 specification and needs to be added in RAN5 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR SA FR1 fast Scell activation test case for 160 ms SCell measurement cycle is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0307 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 R4-2311190 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Following changes: - Added editor’s notes: Message contents for Aperiodic CSI-RS for SCell activation are FFS. |  |  |  |  |  |  |  |  |

## 6.5	Signaling characteristics

### 6.5.3	SCell activation and deactivation delay

-------- Skipped unchanged clauses --------

----- start of changes --------

#### 6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle

Editor’s note: This test case is incomplete. The following aspects are missing:

- Message contents for Aperiodic CSI-RS for SCell activation are FFS.

- TT analysis is missing

- Measurement uncertainties are missing

6.5.3.10.1		Test purpose

The purpose of this test is to verify that the fast SCell activation times are within the requirements stated in TS 38.133 [6] clause 8.3.16, when the SCell in FR1 is known by the UE at the time of activation.

6.5.3.10.2		Test applicability

This test applies to all types of NR UE from Release 17 onwards and supporting 2DL CA in NR and fast SCell activation.

6.5.3.10.3		Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause 8.3.16.

6.5.3.10.4		Test description

6.5.3.10.4.1		Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for NR PCell and NR SCell are shown in Table 6.5.3.10.4.1-1.

Table 6.5.3.10.4.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.3.10.4.1-2 and Table 6.5.3.10.4.1-3.

Table 6.5.3.10.4.1-2: Initial conditions for fast known FR1 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.3.10.5-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | Connection Diagram |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

Table 6.5.3.10.4.1-3: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13] |

1.	Message contents are defined in clause 6.5.3.10.4.3.

2.	There are two NR carriers, each with one cell. Cell 1 is NR FR1 PCell and Cell 2 is the SCell. Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

6.5.3.10.4.2		Test procedure

The test consists of two successive time periods, with duration of T1and T2, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell and triggering the aperiodic CSI-RS for fast SCell activation.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n (where n mode 20=1), defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in TS 38.133 [6] clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3, where Ninterruption is the interruption length given in TS 38.133 [6] clause 8.2.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation of SCell.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 6.5.3.10.4.1-3 and 6.5.3.10.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1 and provide measurement configurations.

3a.	The UE sends a MeasurementReport message.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.55) in a slot # denoted n and T2 starts in slot n. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 7.

6.	After at least one CSI-RS transmission occasion for channel measurement, the UE shall start sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell according to the following criteria:

-	If the first CSI report for SCell is received by the SS no later than slot sfirst_CSI=n+1+THARQ+3ms+TCSI_ReportingNR slot length,

-	or slot sfirst_CSIint=n+1+THARQ+3ms+TCSI_Reporting+TXNR slot length+Ninterruption+1 if the slot sfirst_CSI was subject to interruption,

-	or the next CSI report occasion if there is no CSI report occasion in slots sfirst_CSI or sfirst_CSIint,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot snonzero_CQI=n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, where Tactivation_time = TFirstATRS + 5ms, as defined in TS 38.133 [6] clause 8.3.16,

-	or slot snonzero_CQIint=maxsfirst_CSIint,snonzero_CQI if slot snonzero_CQI was subject to interruption,

-	or the next available CSI report occasion if there is no CSI report occasion for reporting the valid CSI in slot snonzero_CQI, or slot snonzero_CQIint if slot snonzero_CQI was subject to interruption,

-	and DTX is not observed by the SS outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 7.

7.	When T2 expires, or Activation in step 5 was not acknowledged, or a fail was counted for the event “Activation” in step 6, the SS shall transmit a RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 2-9 until a test verdict has been achieved.

The event "Activation" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved.

6.5.3.10.4.3		Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with condition SCELL_CSI_ON_SPCELL with the following exceptions:

Table 6.5.3.10.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-7 with Condition Deactivated SCell |

Table 6.5.3.10.4.3-2: RRCReconfiguration in step 3: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition EN-DC_MEAS and EN-DC_SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup | CellGroupConfig-SCell | Table 6.5.3.10.4.3-5 |  |  |
| measConfig | MeasConfig | Table 6.5.3.10.4.3-2A |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.3.10.4.3-2A: MeasConfig (Table 6.5.3.10.4.3-2)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR for SCell | entry 2 Table 6.5.3.10.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 6.5.3.10.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.10.4.3-3: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

Table 6.5.3.10.4.3-4: ReportConfigNR

| Derivation Path: Table H.3.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -30 | To ensure reporting can always be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.10.4.3-5: CellGroupConfig-SCell (Table 6.5.3.10.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex of NR SpCell |  |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 6.5.3.10.4.3-6 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 6.5.3.10.4.3-7 |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| … |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.10.4.3-6: ServingCellConfig-SpCell (Table 6.5.3.10.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 1 |  |  |
| } |  |  |  |

Table 6.5.3.10.4.3-7: ServingCellConfig-SCell (Table 6.5.3.10.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 2 |  |  |
| } |  |  |  |

6.5.3.10.5		Test requirement

Table 6.5.3.10.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.5.3.10.5-1: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  | - |  |
|  | Config 2 |  | SR.1.1 TDD |  | - |  |
|  | Config 3 |  | SR.2.1 TDD |  | - |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  | - |  |
|  | Config 2 |  | CCR.1.1 TDD |  | - |  |
|  | Config 3 |  | CCR.2.1 TDD |  | - |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  | - |  |
|  | Config 2 |  | CR.1.1 TDD |  | - |  |
|  | Config 3 |  | CR.2.1 TDD |  | - |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |
| Aperiodic CSI-RS for Scell activation | Config 1 |  | N/A |  | TRS.1.3 FDD |  |
|  | Config 2 |  | N/A |  | TRS.1.3 TDD |  |
|  | Config 3 |  | N/A |  | TRS.1.4 TDD |  |
| gapBetweenBursts |  | slots | N/A |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  | - |  |
|  | Config 3 |  | 10 |  | - |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  | - |  |
|  | Config 3 |  | 5 |  | - |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | 5 |  | N/A |  |
|  | Config 3 |  | 10 |  | N/A |  |
| CSI reporting offset for SCell | Config 1,2 | slot | 2 |  | N/A |  |
|  | Config 3 |  | 4 |  | N/A |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104+TT |  |  |  |
|  | Config 3 |  | -101+TT |  |  |  |
|  |  | dB | 17+TT |  |  |  |
|  |  | dB | 17+TT |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87+TT |  | NA |  |
|  | Config 3 |  | -84+TT |  | NA |  |
| CSI-RSRPNote3 | Config 1,2 | dBm/SCS | NA |  | -87+TT |  |
|  | Config 3 |  | NA |  | -84+TT |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87+TT |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96+TT |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87+TT |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, CSI-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1 with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption. During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstATRS + 5ms, as defined in TS 38.133 [6] clause 8.3.16.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstATRS + 5ms, as defined in TS 38.133 [6] clause 8.3.16.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3.16.

The interruption on any activated serving cell shall not be more than the values specified for SA in TS 38.133 [6] clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in TS 38.133 [6] clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

----- End of changes --------