3GPP TSG-RAN5 Meeting #100	R5-235932

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5465 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to spurious emissions and additional spurious emissions test cases as part of introduction of LTE Band 54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_TDD_1670_1675MHz-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of LTE TDD Band 54 to conformance specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses related to spurious emissions and additional spurious emissions test cases have been updated to introduce Band 54 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE TDD Band 54 will be unspecified for UE conformance testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.2.3, 6.6.3.2A.0, 6.6.3.2A.2.4.1, 6.6.3.2A.2.5, 6.6.3E.2.3, 6.6.3E.2.5, 6.6.3EA.2.3, 6.6.3EA.2.5, 6.6.3EC.2.3, 6.6.3EC.2.5, 6.6.3F.2.3, 6.6.3F.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revisions: i) Corrects the meeting number, date/year and location on the coversheet header; ii) Corrected the change to fully delete Table 6.6.3.2A.0-1f and not just the text within the table. |  |  |  |  |  |  |  |  |

##### 6.6.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified E-UTRA band for coexistence with protected bands as indicated in Tables 6.6.3.2.3-1 to 6.6.3.2.3-1J.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<< Unchanged text skipped>>

Table 6.6.3.2.3-1J: Spurious emission band UE co-existence limits Rel-18

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 54, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 54, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53. 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 54, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 48, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 54, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 54 | E-UTRA Band 2, 4, 5, 8, 12, 13, 14, 17, 23, 24, 25, 26, 29, 30, 48, 50, 51, 53, 66, 70, 71, 85, 103 NR NTN Band n255, n256 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 103 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

##### 6.6.3.2A.0	Minimum conformance requirements

This clause specifies the requirements for the specified carrier aggregation configurations for coexistence with protected bands.

NOTE 1:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<< Unchanged text skipped>>

Table 6.6.3.2A.0-1f: Requirements for dual-uplink inter-band carrier aggregation limits for Rel-17

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1A-3A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_1A-5A | E-UTRA Band 1, 5, 7, 8, 22, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_1A-7A | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_1A-8A | E-UTRA Band 1, 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 11 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3, 11 |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_1A-11A | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band , n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-18A | E-UTRA Band 1, 3, 11, 21, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-19A | E-UTRA Band 1, 3, 11, 21, 28, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-20A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 38, 42, 69 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_1A-21A | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-26A | E-UTRA Band 1, 5, 7, 11, 18, 19, 21, 22, 26, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 |  | - | 1895 | -40 | 1 | 3, 12 |
|  | Frequency range | 1895 |  | - | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  | - | 1920 | +1.6 | 5 | 3, 12, 13 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | E-UTRA Band 41 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
| CA_1A-28A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 32, 42, 43, 50, 51, 52, 74, 75, 76 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 21 |
|  | E-UTRA Band 1, 65 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | Frequency range | 470 |  | - | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 |  | - | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_1A-41A | E-UTRA Band 1, 3, 5, 8, 26, 27, 28, 40, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 30 |
| CA_1A-42A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_2A-4A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 43, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-5A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 43, 53 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-7A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 29, 30, 42, 50, 51, 65, 66, 70, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-12A | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 12, 25, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 4, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-13A | E-UTRA Band 4, 5,12,13,17, 22, 26, 27, 29, 41, 42, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25, 103 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 24, 30, 43, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_2A-14A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_2A-48A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
| CA_2A-49A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
| CA_3A-5A | E-UTRA Band 1, 5, 7, 8, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 52 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
| CA_3A-7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_3A-8A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low |  | - | FDL_high | -50 | 1 | 2, 3 |
|  | E-UTRA band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 10,11 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 10, 11 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3,11,17 |
| CA_3A-11A | E-UTRA Band 1, 18, 19, 28, 34, 40, 65 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-18A | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-19A | E-UTRA Band 1, 11, 21, 28, 40, 65 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-20A | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 20 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_3A-21A | E-UTRA Band 1, 18, 19, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42 NR Band n77, n78, | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-26A | E-UTRA Band 1, 5, 7, 11, 18, 19, 21, 26, 34, 39, 40, 43, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 41, 42 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_3A-28A | E-UTRA Band 1, 11, 18, 19, 21, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 5, 7, 8, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_3A-40A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_3A-41A | E-UTRA Band 1, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 18 |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| CA_3A-42A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_4A-5A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 42, 53 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_4A-7A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 42 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_4A-12A | E-UTRA Band 2, 5, 7,13, 14, 17, 24, 25, 26, 27, 30, 41, 43, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 22, 42, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_4A-13A | E-UTRA Band 2,4, 5, 7, 12,13,17,25, 26, 27, 29, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14, 103 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 24, 30, 42, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_4A-17A | E-UTRA Band 2, 5, 7,13, 14, 17, 24, 25, 26, 27, 30, 41, 43, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 22, 42, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_4A-28A | E-UTRA Band 2, 5, 7, 14, 24, 25, 26, 27, 30, 41, 53, 70, 71 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 43, 48, 50, 51, 66, 74 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  |  | 803 | -50 | 1 |  |
| CA_5A-7A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_5A-12A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 30, 31, 43, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 4, 22, 41, 42, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_5-17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 30, 31, 43, 50, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 4, 22, 41, 42, 51, 53, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_5A-40A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 31, 34, 38, 42, 43, 45, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_7A-8A | E-UTRA Band 1, 20, 27, 28, 31, 32, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_7A-20A | E-UTRA Band 1,3, 7, 8, 22, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42, 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 2, 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 2, 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_7A-26A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 29, 30, 31, 40, 42, 43, 65, 66, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 7 |
| CA_7A-28A | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_8A-39A | E-UTRA Band 1, 28, 40, 45, 50, 51, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 22, 41, 42, 52 NR band n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_8A-41A | E-UTRA Band 1, 28, 34, 39, 40, 45, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 11 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 11 |
| CA_11A-18A | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_11A-26A | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34, 40, 42, 65 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_13A-66A | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 24, 30, 48, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_14A-30A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_14A-66A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_18A-28A | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 21 |
|  | E-UTRA Band 1, 65 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA Band 42, 43 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34, 40 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19A-21A | E-UTRA Band 1, 3, 28, 34, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19A-42A | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21A-28A | E-UTRA Band 1, 42, 65 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA Band 3, 18, 19, 34, 40 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21A-42A | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_25A-26A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 42, 48, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 53 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_25A-41A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 48, 66, 70, 71, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_26A-46A | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 65, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 1, 2 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_26A-48A | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 50, 51, 65, 66, 70, 71, 73, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low |  | - | FDL_high | -50 | 1 | 1,2 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_28A-41A | E-UTRA Band E-UTRA Band 1, 4, 22, 32, 42, 45, 43, 48, 52, 65, 66 NR Band n77, n78, n79 | FDL_low | - |  | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - |  | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA band 2, 3, 5, 8, 20, 25, 26, 27, 31, 33, 34, 40 | FDL_low | - |  | FDL_high | -50 | 1 |  |
|  | E-UTRA band 11, 21 | FDL_low | - |  | FDL_high | -50 | 1 | 5, 18, 21 |
|  | E-UTRA band 9, 18, 19 | FDL_low | - |  | FDL_high | -50 | 1 | 5, 18 |
|  | Frequency range | 470 | - |  | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 | - |  | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 662 | - |  | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 | - |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - |  | 1915.7 | -41 | 0.3 | 4, 5, 18 |
| CA_28A-42A | E-UTRA Band 1, 4, 32, 50, 51, 66, 65, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 21 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_39A-41C | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 50, 51, 52, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | - | 1855 | -40 | 1 | 20 |
|  | Frequency range | 1855 |  | - | 1880 | -15.5 | 5 | 3, 13, 20 |
| CA_40A-42A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 70, 72, 73, 74, 75, 76, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 8 |
| CA_41A-42A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 18 |
|  | NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE10: Void NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 14: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19: Void NOTE 20: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 21: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 24: Void NOTE 25: Void |  |  |  |  |  |  |  |  |

Table 6.6.3.2A.0-1g: Requirements for dual-uplink inter-band carrier aggregation limits for Rel-18

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1A-3A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_1A-5A | E-UTRA Band 1, 5, 7, 8, 22, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_1A-7A | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_1A-8A | E-UTRA Band 1, 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 11 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3, 11 |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_1A-11A | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band , n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-18A | E-UTRA Band 1, 3, 11, 21, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-19A | E-UTRA Band 1, 3, 11, 21, 28, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-20A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 38, 42, 69 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_1A-21A | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1A-26A | E-UTRA Band 1, 5, 7, 11, 18, 19, 21, 22, 26, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 |  | - | 1895 | -40 | 1 | 3, 12 |
|  | Frequency range | 1895 |  | - | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  | - | 1920 | +1.6 | 5 | 3, 12, 13 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | E-UTRA Band 41 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
| CA_1A-28A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 32, 42, 43, 50, 51, 52, 74, 75, 76 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 21 |
|  | E-UTRA Band 1, 65 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | Frequency range | 470 |  | - | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 |  | - | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_1A-41A | E-UTRA Band 1, 3, 5, 8, 26, 27, 28, 40, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 30 |
| CA_1A-42A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 1880 |  |  | 1895 | -40 | 1 | 3,12 |
|  | Frequency range | 1895 |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |
|  | Frequency range | 1915 |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |
| CA_2A-4A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 43, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-5A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 43, 53. 54 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-7A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 29, 30, 42, 50, 51, 65, 66, 70, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-12A | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 54, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 12, 25, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 4, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_2A-13A | E-UTRA Band 4, 5,12,13,17, 22, 26, 27, 29, 41, 42, 50, 51, 53, 54, 66, 70, 71, 74, 85 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25, 103 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 24, 30, 43, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_2A-14A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_2A-48A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
| CA_2A-49A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
| CA_3A-5A | E-UTRA Band 1, 5, 7, 8, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 52 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
| CA_3A-7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_3A-8A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low |  | - | FDL_high | -50 | 1 | 2, 3 |
|  | E-UTRA band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 10,11 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 10, 11 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3,11,17 |
| CA_3A-11A | E-UTRA Band 1, 18, 19, 28, 34, 40, 65 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-18A | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-19A | E-UTRA Band 1, 11, 21, 28, 40, 65 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-20A | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 20 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_3A-21A | E-UTRA Band 1, 18, 19, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42 NR Band n77, n78, | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3A-26A | E-UTRA Band 1, 5, 7, 11, 18, 19, 21, 26, 34, 39, 40, 43, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA band 22, 41, 42 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_3A-28A | E-UTRA Band 1, 11, 18, 19, 21, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 5, 7, 8, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_3A-40A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_3A-41A | E-UTRA Band 1, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 18 |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| CA_3A-42A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_4A-5A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 42, 53 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_4A-7A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 42 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_4A-12A | E-UTRA Band 2, 5, 7,13, 14, 17, 24, 25, 26, 27, 30, 41, 43, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 22, 42, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_4A-13A | E-UTRA Band 2,4, 5, 7, 12,13,17,25, 26, 27, 29, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14, 103 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 24, 30, 42, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_4A-17A | E-UTRA Band 2, 5, 7,13, 14, 17, 24, 25, 26, 27, 30, 41, 43, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 22, 42, 51, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_4A-28A | E-UTRA Band 2, 5, 7, 14, 24, 25, 26, 27, 30, 41, 53, 70, 71 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 43, 48, 50, 51, 66, 74 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  |  | 803 | -50 | 1 |  |
| CA_5A-7A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_5A-12A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 30, 31, 43, 50, 53, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 4, 22, 41, 42, 51, 54, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_5-17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 30, 31, 43, 50, 71, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 4, 22, 41, 42, 51, 53, 54, 66, 70, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 12, 85 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_5A-40A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 31, 34, 38, 42, 43, 45, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 26 | 859 |  | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_7A-8A | E-UTRA Band 1, 20, 27, 28, 31, 32, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_7A-20A | E-UTRA Band 1,3, 7, 8, 22, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 42, 52 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 2, 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 2, 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_7A-26A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 29, 30, 31, 40, 42, 43, 65, 66, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 7 |
| CA_7A-28A | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 |  | - | 2575 | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range | 2575 |  | - | 2595 | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range | 2595 |  | - | 2620 | -40 | 1 | 3, 14 |
| CA_8A-39A | E-UTRA Band 1, 28, 40, 45, 50, 51, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 22, 41, 42, 52 NR band n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
| CA_8A-41A | E-UTRA Band 1, 28, 34, 39, 40, 45, 50, 51, 54, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 11 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 11 |
| CA_11A-18A | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_11A-26A | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34, 40, 42, 65 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_13A-66A | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low |  | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 24, 30, 48, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_14A-30A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_14A-66A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3, 9 |
| CA_18A-28A | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 21 |
|  | E-UTRA Band 1, 65 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA Band 42, 43 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34, 40 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19A-21A | E-UTRA Band 1, 3, 28, 34, 40, 42, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19A-42A | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21A-28A | E-UTRA Band 1, 42, 65 NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA Band 3, 18, 19, 34, 40 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21A-42A | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_25A-26A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 42, 48, 54, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 53 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_25A-41A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 48, 54, 66, 70, 71, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| CA_26A-46A | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 65, 66, 70, 71, 85, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, 54 NR Band n77 | FDL_low |  | - | FDL_high | -50 | 1 | 1, 2 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_26A-48A | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 50, 51, 65, 66, 70, 71, 73, 74, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 54 | FDL_low |  | - | FDL_high | -50 | 1 | 1,2 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_28A-41A | E-UTRA Band E-UTRA Band 1, 4, 22, 32, 42, 45, 43, 48, 52, 65, 66 NR Band n77, n78, n79 | FDL_low | - |  | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - |  | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA band 2, 3, 5, 8, 20, 25, 26, 27, 31, 33, 34, 40 | FDL_low | - |  | FDL_high | -50 | 1 |  |
|  | E-UTRA band 11, 21 | FDL_low | - |  | FDL_high | -50 | 1 | 5, 18, 21 |
|  | E-UTRA band 9, 18, 19 | FDL_low | - |  | FDL_high | -50 | 1 | 5, 18 |
|  | Frequency range | 470 | - |  | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 | - |  | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 662 | - |  | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 | - |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - |  | 1915.7 | -41 | 0.3 | 4, 5, 18 |
| CA_28A-42A | E-UTRA Band 1, 4, 32, 50, 51, 66, 65, 74, 75, 76 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 6 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 5, 21 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_39A-41C | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 50, 51, 52, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | - | 1855 | -40 | 1 | 20 |
|  | Frequency range | 1855 |  | - | 1880 | -15.5 | 5 | 3, 13, 20 |
| CA_40A-42A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 70, 72, 73, 74, 75, 76, 103 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 8 |
| CA_41A-42A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 18 |
|  | NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE10: Void NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 14: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19: Void NOTE 20: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 21: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 24: Void NOTE 25: Void |  |  |  |  |  |  |  |  |

<< Unchanged text skipped>>

Table 6.6.3.2A.0-2i: Spurious emission band UE co-existence for intraband carrier aggregation limits for Rel18

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1C | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 50, 51, 52, 65, 67, 72, 73, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_3C | E-UTRA Band 1, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_5B | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 52, 54 NR Band n77, n78,n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_7C | E-UTRA Band 1, 3, 7, 8, 20, 22, 27, 28, 29, 30. 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_8B | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 50, 51, 54, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | E-UTRA Band 22, 41, 42, 43, 52 NR Band n77, n78,n79 | FDL_low |  | FDL_high | -50 | 1 | 2 |
| CA_38C | E-UTRA Band 1,3, 8, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_39C | E-UTRA Band 22, 34, 40, 41, 42, 44, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_40D | E-UTRA Band 1, 3, 7, 8, 20, 22, 26, 27, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 50, 51, 52, 65, 67, 72, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_41C | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_42C | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 50, 51, 65, 66, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| CA_66C | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: To meet these requirements some restriction will be needed for either the operating band or protected band NOTE 4: N/A NOTE 5: N/A NOTE 6: N/A NOTE 7: N/A NOTE 8: N/A NOTE 9: N/A NOTE 10: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 11: N/A NOTE 12: N/A NOTE 13: N/A NOTE 14: N/A |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

6.6.3.2A.2.4.1	Initial conditions

Same as in clause 6.6.3.2A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.6.3.2A.1.4.1-1  use Table 6.6.3.2A.2.4.1-1.

-	Instead of clause 6.6.3.2A.1.4.3 use clause 6.6.3.2A.2.4.3.

Table 6.6.3.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |  |
|  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB |  |  |  |  |  | SCC NRB |  |  |  |  | CC MOD |  |  |  |  | PCC & SCC RB allocation |  |  |  |  |  | CC MOD |  |  |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
|  |  |  | PCC |  |  |  |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC |  | SCC |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Band |  |  |  |  |  | Range |  |  |  |  | Band |  |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  |  |  | Low |  |  |  |  | Y |  |  |  |  | Low |  |  |  |  | Highest NRB |  |  |  |  |  | Highest NRB |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  |  | 1@0 |  |  |  |  | 1@0 |
| 2 | X |  |  |  |  | High |  |  |  |  | Y |  |  |  |  | High |  |  |  |  | Highest NRB |  |  |  |  |  | Highest NRB |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  |  | 1@RBmax |  |  |  |  | 1@RBmax |
| Test Settings for CA_1A-3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | Low |  |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  | 1 |  |  |  |  | Low |  |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 3 |  | 1 |  |  |  |  | High |  |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |  |
| 4 |  | 1 |  |  |  |  | High |  |  |  |  | 3 |  |  |  |  | Note 7 |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| Test Settings for CA_1A-7A Configuration(Note 12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |  |
| 2 |  | 1 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |  |
| Test Settings for CA_1A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  | 1 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  |  | QPSK |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| Test Settings for CA_1A-18A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  | 1 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |
| Test Settings for CA_1A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 2 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |
| 3 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 4 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |
| Test Settings for CA_1A-21A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@24 |
| 2 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@12 |
| 3 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |
| 4 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@74 |
| 5 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@37 |
| 6 |  |  | 1 |  |  |  |  |  | Mid |  |  |  |  | 21 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |
| Test Settings for CA_1A-28A Configuration (Note 16) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  | 1@49 |
| 2 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  | 1@0 |
| 3 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  | 1@47 |
| 4 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  | 1@49 |
| 5 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  | 1@0 |
| 6 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | Mid |  |  |  |  |  |  | 50 |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  | 1@47 |
| Test Settings for CA_1A-42A Configuration (Note 13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  | 1@0 |
| 2 |  |  | 1 |  |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  | 1@99 |
| 3 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 42 |  |  |  |  | Mid |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  | 1@24 |
| 4 |  |  | 1 |  |  |  |  |  | High |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  |  |  | 100 |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@49 |  | 1@0 |
| Test Settings for CA_2A-4A Configuration (Note 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 3 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 4 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |
| 5 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 6 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 7 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 8 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 9 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |
| 10 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| Test Settings for CA_2A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| Test Settings for CA_2A-48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 2 |  |  |  |  | High |  |  |  |  | 48 |  |  |  |  | Note 17 |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| Test Settings for CA_3A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 5 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 6 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| Test Settings for CA_3A-7A Configuration (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@75 |  |  |  | 1@99 |
| 4 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 6 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 7 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| Test Settings for CA_3A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 3 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 7 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 8 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 9 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| Test Settings for CA_3A-18A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |
| Test Settings for CA_3A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 2 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@24 |  |  | 1@0 |
| 3 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 25 |  |  |  |  |  | 25 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 25@0 |  |  | 25@0 |
| 4 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 5 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 1@99 |  |  | 1@0 |
| 6 |  |  | 3 |  |  |  |  |  | High |  |  |  |  | 19 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  |  | QPSK |  |  |  | 100@0 |  |  | 75@0 |
| Test Settings for CA_3A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 3 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |
| 4 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 5 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |
| Test Settings for CA_3A-26A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 3 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 4 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 6 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@74 |
| 7 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 26 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@74 |
| 8 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 26 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |
| Test Settings for CA_3A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 2 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| Test Settings for CA_3A-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 41 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 41 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 3 |  |  |  | 3 |  |  |  |  | Mid |  |  |  |  | 41 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 4 |  |  |  | 3 |  |  |  |  | Mid |  |  |  |  | 41 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 5 |  |  |  | 3 |  |  |  |  | Low |  |  |  |  | 41 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |
| Test Settings for CA_4A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@49 |
| 3 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 4 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 5 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 6 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| Test Settings for CA_4A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 3 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |
| 4 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |
| 5 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |
| 6 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 7 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 8 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |
| 9 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 10 |  |  |  | 4 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 11 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 12 |  |  |  | 4 |  |  |  |  | Low |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| Test Settings for CA_5A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@99 |
| 2 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@24 |  |  |  | 1@0 |
| 3 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@49 |  |  |  | 1@0 |
| Test Settings for CA_5A-12A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 12 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 2 |  |  |  | 5 |  |  |  |  | High |  |  |  |  | 12 |  |  |  |  | High |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_5A-17A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 5 |  |  |  |  | Low |  |  |  |  | 17 |  |  |  |  | Low |  |  |  |  | 50 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_7A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@49 |
| 2 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 3 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@49 |
| 4 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 20 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 5 |  |  |  | 7 |  |  |  |  | Low |  |  |  |  | 20 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| Test Settings for CA_7A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@74 |
| 2 |  |  |  | 7 |  |  |  |  | High |  |  |  |  | 28 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| Test Settings for CA_8A-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 8 |  |  |  |  | High |  |  |  |  | 41 |  |  |  |  | Note 18 |  |  |  |  | 100 |  |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@44 |
| Test Settings for CA_18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Note 15 |  |  |  |  | 75 |  |  |  |  |  |  | 50 |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@2 |
| 2 |  |  |  |  | 18 |  |  |  |  | Low |  |  |  |  | 28 |  |  |  |  | Note 15 |  |  |  |  | 75 |  |  |  |  |  |  | 50 |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@74 |  |  |  | 1@47 |
| Test Settings for CA_39A-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 39 |  |  |  |  | High |  |  |  |  | 41 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| Test Settings for CA_41A-42A Configuration (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 2 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Note 11 |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@22 |  |  |  | 1@99 |
| 3 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@44 |  |  |  | 1@99 |
| 4 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 5 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 6 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@44 |  |  |  | 1@99 |
| 7 |  |  |  | 41 |  |  |  |  | Note 10 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@0 |  |  |  | 1@99 |
| 8 |  |  |  | 41 |  |  |  |  | Note 8 |  |  |  |  | 42 |  |  |  |  | Low |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@0 |
| 9 |  |  |  | 41 |  |  |  |  | Note 9 |  |  |  |  | 42 |  |  |  |  | Mid |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| 10 |  |  |  | 41 |  |  |  |  | Low |  |  |  |  | 42 |  |  |  |  | High |  |  |  |  | 100 |  |  |  |  |  | 100 |  |  |  |  | QPSK |  |  |  |  | NA |  |  |  |  | QPSK |  |  |  | 1@99 |  |  |  | 1@99 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1A-19A,X=1,Y=19. Note 4: For CA configurations CA_1A-18A, CA_1A-19A, CA_1A-21A , CA_3A-19A , CA_3A-28A and CA_18A-28A each test point shall be tested. For all other CA configurations only test points with highest NRB_agg , according to UE supported BCS shall be tested. Note 5: Test Point ID 7 for CA_3A-7A is applicable only for a Rel-13 and forward UE. Note 6: Test Point ID 3 and Test Point ID 8 for CA_2A-4A are applicable only for a Rel-14 and forward UE. Note 7: Test Point ID 4 for CA_1A-3A has the centre carrier frequency 1849.9 MHz, i.e. DL E-ARFCN is 1649. Note 8: Test Point ID 1, 4 and 8 for CA_41A-42A have the centre carrier frequency of 2635 MHz in Band 41 (EARFCN=41040). Note 9: Test Point ID 2, 3, 5, 6 and 9 for CA_41A-42A has the centre carrier frequency of 2555 MHz in Band 41 (EARFCN=40240). Note 10: Test Point ID 7 for CA_41A-42A has the centre carrier frequency of 2674 MHz in Band 41 (EARFCN=41430). Note 11: Test Point ID 2 for CA_41A-42A has the centre carrier frequency of 3421 MHz in Band 42 (EARFCN=41800). Note 12: Test Point ID 2 for CA_1A-7A is applicable only for a Rel-15 and forward UE. Note 13: Test Point ID 4 for CA_1A-42A is applicable only for a Rel-15 and forward UE. Note 14: Test Point ID 10 for CA_41A-42A is applicable only for a Rel-15 and forward UE. Note 15: Test Point ID 1 and 2 for CA_18A-28A have the centre carrier frequency of 778 MHz in Band 28 (EARFCN=9410). Note 16: As an exception to Note 4, Test Point ID 3 for CA_1A-28A shall be tested for a UE supporting bandwidth combination set 0, even though it has not highest NRB_agg for this UE. Notes 17: Test Point ID 1 for CA_2A-48A have the centre carrier frequency of 3560 MHz in Band 48 (EARFCN=55340). Notes 18: Test Point ID 1 for CA_8A-41A have the centre carrier frequency of 2570 MHz in Band 41 (EARFCN=55980). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.6.3.2A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.3.2A.2.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.3.2A.2.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.3.2A.2.5-6. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.6.3.2A.2.5-6. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

6.6.3.2A.2.5	Test requirement

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.6.3.2A.2.5-6 according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3.2A.2.5-6. If the UE supports a CA configuration, which is not defined in the table corresponding to UE’s release, the requirements for this CA configuration are taken from the table of earliest release where requirements for this CA Configuration are defined. This has been described in following Table 6.6.3.2A.2.5-0.

<< Unchanged text skipped>>

Table 6.6.3.2A.2.5-1: Void

Table 6.6.3.2A.2.5-2: Void

Table 6.6.3.2A.2.5-3: Void

Table 6.6.3.2A.2.5-4: Void

Table 6.6.3.2A.2.5-5: Void

Table 6.6.3.2A.2.5-6: Requirements for dual-uplink inter-band carrier aggregation according to UE E-UTRA release

| E-UTRA CA Configuration | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| CA_1A-3A | Rel-11, Rel-12 | E-UTRA Band 1, 32, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Rel-13, Rel-14 | E-UTRA Band 1, 32, 43, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 1, 32, 43, 50, 65, 74, 75 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
| CA_1A-5A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 22, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_1A-7A | Rel-11, Rel-12, Rel-13, Rel-14 | E-UTRA Band 32 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 32, 50, 51, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_1A-8A | Rel-11, Rel-12, Rel-13 | E-UTRA band43 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA band43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_1A-18A | Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_1A-19A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_1A-21A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| CA_1A-26A | Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 22, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_1A-28A | Rel-13 | E-UTRA Band 7, 31, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  |  | Frequency range | 470 | - | 694 | -42 | 8 | 3, 22 |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 23 |
|  | Rel-14 | E-UTRA Band 7, 31, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 470 | - | 694 | -42 | 8 | 3, 22 |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 23 |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 7, 31, 41, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 470 | - | 694 | -42 | 8 | 3, 22 |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 23 |
| CA_1A-42A | Rel-13, Rel-14 | E-UTRA Band 11, 21, 31, 32 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 11, 21, 31, 32, 50, 51, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_2A-4A | Rel-13 | E-UTRA Band 4, 10, 22, 24 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14 | E-UTRA Band 4, 10, 22, 24, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 4, 10, 22, 24, 50, 66, 70, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_2A-5A | Rel-13 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_2A-7A | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 29 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_2A-48A | Rel-14, Rel-15, Rel-16, Rel-17 | No requirements |  |  |  |  |  |  |
|  | Rel-18 | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_2A-13A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_3A-5A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 5, 7, 8, 38, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 26 | 859 | - | 869 | -27 | 1 |  |
| CA_3A-7A | Rel-11, Rel-12 | E-UTRA Band 5,8, 20, 26, 27, 28, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-13 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_3A-8A | Rel-11, Rel-12, Rel-13 | E-UTRA Band 20, 28, 38, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 7, 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  |  | Frequency range | 860 | - | 890 | -40 | 1 | 3,11,17 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 20, 28, 38, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 7, 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 860 | - | 890 | -40 | 1 | 3,11,17 |
| CA_3A-19A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_3A-20A | Rel-11, Rel-12, Rel-13 | E-UTRA Band 7, 8 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 7, 8 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_3A-26A | Rel-11, Rel-12, Rel-13 | E-UTRA Band 5, 7, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 41, 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 5, 7, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 41, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| CA_3A-28A | Rel-14 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 5, 6 |
|  |  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 31, 34, 38, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 23 |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 1, 11, 18, 19, 21, 22, 42, 43, 50, 51, 52, 65, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 5, 6 |
|  |  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 5, 7, 8, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 23 |
| CA_3A-41A | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 18 |
| CA_4A-5A | Rel-13 | E-UTRA Band 5,7 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 26 | 859 | - | 869 | -27 | 1 |  |
|  |  |  |  | E-UTRA band 41, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2_R11-R13 |  |
|  |  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 |  | E-UTRA Band 5,7 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA band 41, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| CA_4A-7A | Rel-11, Rel-12, Rel-13, | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_4A-12A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | No requirement |  |  |  |  |  |  |
| CA_4A-13A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15 | E-UTRA Band 7, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16, Rel-17, Rel-18 | E-UTRA Band 7, 41, 53 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_4A-17A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | No requirement |  |  |  |  |  |  |
| CA_5A-7A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 5, 28, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 26 | 859 | - | 869 | -27 | 1 |  |
| CA_5A-12A |  | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 |  | E-UTRA Band 24, 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| CA_5A-17A |  | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 |  | E-UTRA Band 24, 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| CA_7A-20A |  | Rel-11, Rel-12, Rel-13 |  | E-UTRA Band 28 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2_R11-R13 |
|  | Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 28 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_7A-28A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_8A-41A | Rel-14, Rel-15, Rel-16, Rel-17 | No requirements |  |  |  |  |  |  |
|  | Rel-18 | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_11A-18A | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 860 | - | 890 | -40 | 1 | 3 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| CA_18A-28A | Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | Frequency range | 470 | - | 710 | -26.2 | 6 | 23 |
| CA_19A-21A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | No requirement |  |  |  |  |  |  |
| CA_39A-41A | Rel-11, Rel-12, Rel-13, Rel-14, Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_41A-42A | Rel-14 | E-UTRA Band 3, 5, 8, 26, 28, 33, 39, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 18 |
|  |  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 18 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 18 |
|  | Rel-15, Rel-16, Rel-17, Rel-18 | E-UTRA Band 3, 5, 8, 26, 28, 33, 39, 44, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 18 |
|  |  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 18 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 18 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.2-1 NOTE 2_R11-R13: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.4.2-1) for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 and Table 6.6.3.1A.0-1 from the edge of the aggregated channel bandwidth. NOTE 4_R11–R13: Void. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5-9: Void. NOTE10: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: Void. NOTE 13_R11-R13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 13: Void. NOTE 14_R11-R13: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 14: Void. NOTE 15-16: Void. NOTE 17_R11–R13: Void. NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18_R11-R13: Void. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19-21: Void. NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 24-25: Void. |  |  |  |  |  |  |  |  |

<< Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.6.3E.2	Spurious emission band UE co-existence for UE category 0

<< Unchanged text skipped>>

##### 6.6.3E.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.3.2.3 with the following exceptions:

-	Instead of Tables 6.6.3.2.3-1 through 6.6.3.2.3-1D  use Tables 6.6.3E.2.3-1 to 6.6.3E.2.3-1f.

Table 6.6.3E.2.3-1: Spurious emission band limits Rel-12

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

Table 6.6.3E.2.3-1a: Spurious emission band limits Rel-13

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

Table 6.6.3E.2.3-1b: Spurious emission band limits Rel-14

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

Table 6.6.3E.2.3-1c: Spurious emission band limits Rel-15

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

Table 6.6.3E.2.3-1d: Spurious emission band limits Rel-16

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

Table 6.6.3E.2.3-1e: Spurious emission band limits Rel-17

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

Table 6.6.3E.2.3-1f: Spurious emission band limits Rel-18

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 54 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 32 |
|  | Frequency range | 1855 |  | 1875 | -15.5 | 5 | 15,26,32 |
|  | Frequency range | 1875 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| Note 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1. Note 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1.3-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th ] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. Note 3: To meet these requirements some restriction will be needed for either the operating band or protected band. Note 4: N/A Note 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band. Note 6: N/A Note 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. Note 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. Note 9: N/A Note 10: N/A Note 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. Note 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. Note 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. Note 14: N/A Note 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth. Note 16: N/A Note 17: N/A Note 18: N/A Note 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Note 20: N/A Note 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 22: This requirement is applicable for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. Note 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. Note 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). Note 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. Note 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. Note 28: N/A Note 29: N/A Note 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz. Note 31: N/A Note 32: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1887.5 - 1889.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1890 - 1898 MHz. Note 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to [54 RB] for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. Note 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Note 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.3E.2.

This test use minimum requirements from many releases of TS 36.101 [2] due to release independence defined in TS 36.307 [16]

##### 6.6.3E.2.4	Test description

6.6.3E.2.4.1	Initial conditions

Same initial conditions as in clause 6.6.3.2.4.1 with the following exceptions:

-	Instead of Table 6.6.3.2.4.1-1 and Table 6.6.3.2.4.1-2  use Table 6.6.3E.2.4.1-1 and Table 6.6.3E.2.4.1-2.

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.6.3E.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) (Note 6, Note 7) |  |  |  | Low range, Mid range, High range (Note 6, 13, 16) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, Highest (Note 13, 16) |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Spurious Emissions testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 1 | 1 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 1 | 1 |
| 5MHz |  |  |  | QPSK | 25 | 2512 |
| 5MHz |  |  |  | QPSK | 1 | 112 |
| 5MHz |  |  |  | QPSK | 208 | - |
| 10MHz |  |  |  | QPSK | 36 | 36 |
| 10MHz |  |  |  | QPSK | 1 | 1 |
| 10MHz |  |  |  | QPSK | 3015 |  |
| 10MHz |  |  |  | QPSK | 329 | - |
| 10MHz |  |  |  | QPSK | 110, 14 | - |
| 15MHz |  |  |  | QPSK | 36 | 36 |
| 15MHz |  |  |  | QPSK | 363, 11 | 364, 17, 18 |
| 15MHz |  |  |  | QPSK | 1 | 15 |
| 20MHz |  |  |  | QPSK | 36 | 36 |
| 20MHz |  |  |  | QPSK | 363, 11 | 364, 17, 18 |
| 20MHz |  |  |  | QPSK | 1 | 15 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 3: To be used for requirements under note 15 in table 6.6.3.2.3-1 in high range at RB#, instead of full allocation. Note 4: To be used for requirements under note 16 in table 6.6.3.2.3-1 and table 6.6.3.2.3-1A, note 17 in table 6.6.3.2.3-1B, note 22 in table 6.6.3.2.3-1C in high range - 5MHz at RB#(full allocation – 54), instead of full allocation. Note 5: For requirements under note 16 in table 6.6.3.2.-1 and table 6.6.3.2.3-1A, note 17 in table 6.6.3.2.3-1B, note 22 in table 6.6.3.2.3-1C (high range – 5MHz) is tested instead of high channel. Note 6: Do not apply for requirements under Note 13 in Tables 6.6.3.2.3-1 to 6.6.3.2.3-1C. Test frequencies for these requirements are defined in Table 6.6.3.2.4.1-2. Note 7: Void Note 8: For requirements under note 17 in table 6.6.3.2.3-1A, note 18 in table 6.6.3.2.3-1, note 23 in table 6.6.3.2.3-1C (high range – 10MHz) is tested with RB #0 to RB #19 used. Note 9: For requirements under note 17 in table 6.6.3.2.3-1A, note 18 in table 6.6.3.2.3-1B, note 23 in table 6.6.3.2.3-1C high range is tested with RB #4 to RB #35 used. Note 10: For requirements under note 17 in table 6.6.3.2.3-1A, note 18 in table 6.6.3.2.3-1B, note 23 in table 6.6.3E.2.3-1C high range is tested with RB #4 and (RB#max -4) used. Note 11: To be used for requirements under note 17 in table 6.6.3.2.3-1, note 18 in table 6.6.3.2.3-1A, note 19 in table 6.6.3.2.3-1B and note 27 in table 6.6.3.2.3-1C in low range at RB#0, instead of full allocation. Note 12: For requirements under note 16 in table 6.6.3E.2.3-1 and table 6.6.3.2.3-1A, note 17 in table 6.6.3.2.3-1B, note 22 in table 6.6.3.2.3-1C, the message exception in Table 6.6.3.2.4.3-1 is used to test with the high range. Note 13: For requirements under note 31 in table 6.6.3.2.3-1C and note 34 in table 6.6.3E.2.3-1 only 720.5 MHz is tested with 5 MHz and only 723 MHz is tested with 10 MHz. Note 14: For requirements under note 31 in table 6.6.3.2.3-1C and note 34 in table 6.6.3E.2.3-1 RB #2 and RB #47 used. Note 15: To be used for requirements under note 31 in table 6.6.3.2.3-1C and note 34 in table 6.6.3E.2.3-1 at RB#2, instead of full allocation. Note 16: For requirements under note 32 in table 6.6.3.2.3-1C and note 35 in table 6.6.3E.2.3-1 only low range and 728 MHz with 10 MHz is tested. Note 17: To be used for requirements under note 32 in table 6.6.3E.2.3-1 in low range, instead of full allocation. Note 18: To be used for requirements under note 33 in table 6.6.3E.2.3-1 (high range-7 MHz), instead of full allocation and high range. |  |  |  |  |  |  |

Table 6.6.3E.2.4.1-2: Test frequencies for E-UTRA channel bandwidth for operating band 3 
with Note 13 (in Tables 6.6.3E.2.3-1)

| Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 5 | 19924 | 1782.4 | 1924 | 1877.4 |
| 10 | 19899 | 1779.9 | 1899 | 1874.9 |
| 15 | 19874 | 1777.4 | 1874 | 1872.4 |
| 20 | 19849 | 1774.9 | 1849 | 1869.9 |

6.6.3E.2.4.2	Test procedure

Same test procedure as in clause 6.6.3.2.4.2 with the following exception for HD-FDD:

-	In step 3, slots with transient periods are not under test. Half-duplex guard subframes are not under test.

6.6.3E.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

##### 6.6.3E.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in table 6.6.3E.2.3-1 according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3E.2.3-1 to 6.6.3E.2.3-1f. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.6.3E.2.5-1.

Table 6.6.3E.2.5-1: UE Requirements according to UE E-UTRA release and supported E-UTRA band

| Band | UE Requirements per release |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rel-12 | Rel-13 | Rel-14 | Rel-15 | Rel-16 | Rel-17 | Rel-18 |  |
| 2 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 3 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 4 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 5 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 8 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 13 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 20 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 39 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |
| 41 |  | Table 6.6.3E.2.3-1 | Table 6.6.3E.2.3-1a | Table 6.6.3E.2.3-1b | Table 6.6.3E.2.3-1c | Table 6.6.3E.2.3-1d | Table 6.6.3E.2.3-1e | Table 6.6.3E.2.3-1f |  |

NOTE 1:	The following is applied to Note2 in Tables 6.6.3E.2.3-1 to 6.6.3E.2.3.1f. For frequency with 2nd, 3rd or 4th harmonic spurious emissions, the measurements are covered in 6.6.3.1.

NOTE 2:	The restriction on the maximum uplink transmission to 36 RB in Notes 14 and 15 of Table 6.6.3E.2.5-1 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.6.3EA.2	Spurious emission band UE co-existence for UE category M1

##### 6.6.3EA.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

##### 6.6.3EA.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.6.3EA.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified E-UTRA band for coexistence with protected bands as indicated in Tables 6.6.3EA.2.3-1, 6.6.3EA.2.3-1A, 6.6.3EA.2.3-1B, 6.6.3EA.2.3-1C, 6.6.3EA.2.3-1D and 6.6.3EA.2.3-1E.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<< Unchanged text skipped>>

Table 6.6.3EA.2.3-1D: Spurious emission band limits Rel-17

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 48, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

Table 6.6.3EA.2.3-1E: Spurious emission band limits Rel-18

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 54, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 48, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 54, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.3.2.

This test use minimum requirements from many releases of TS 36.101 [2] due to release independence defined in TS 36.307 [16]

##### 6.6.3EA.2.4	Test description

6.6.3EA.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3EA.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.3EA.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | NC |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) (Note 6, Note 7) |  |  |  |  | Low range, Mid range, High range (Note 6, 13, 16) |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Lowest, 5MHz, Highest (Note 13, 16) |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |
| Ch BW |  | Mod'n | RB allocation |  | Mod'n | RB allocation |  |  |
|  |  |  | FDD | TDD |  | FDD | TDD | Narrowband index1 |
| 1.4MHz |  |  |  |  | π/2-BPSK | ¼@019 | ¼@019 | 0 |
| 1.4MHz |  |  |  |  | π/2-BPSK | ¼@519 | ¼@519 | 0 |
| 1.4MHz |  | N/A for Spurious Emissions testing |  |  | QPSK | 6@0 | 6@0 | 0 |
| 1.4MHz |  |  |  |  | QPSK | 1@0 | 1@0 | 0 |
| 1.4MHz |  |  |  |  | QPSK | 1@5 | 1@5 | 0 |
| 3.0MHz |  |  |  |  | π/2-BPSK | ¼@019 | ¼@019 | 0 |
| 3MHz |  |  |  |  | π/2-BPSK | ¼@519 | ¼@519 | 0 |
| 3MHz |  |  |  |  | QPSK | 6@0 | 6@0 | 0 |
| 3MHz |  |  |  |  | QPSK | 6@0 | 6@0 | 1 |
| 3MHz |  |  |  |  | QPSK | 1@0 | 1@0 | 0 |
| 3MHz |  |  |  |  | QPSK | 1@5 | 1@5 | 1 |
| 5MHz |  |  |  |  | π/2-BPSK | ¼@019 | ¼@019 | 0 |
| 5MHz |  |  |  |  | π/2-BPSK | ¼@519 | ¼@519 | 0 |
| 5MHz |  |  |  |  | QPSK | 6@08 | 6@0 | 0 |
| 5MHz |  |  |  |  | QPSK | 6@08 | 6@0 | 3 |
| 5MHz |  |  |  |  | QPSK | 1@0 | 1@0 | 0 |
| 5MHz |  |  |  |  | QPSK | 1@5 | 1@5 | 3 |
| 10MHz |  |  |  |  | π/2-BPSK | ¼@019 | ¼@019 | 0 |
| 10MHz |  |  |  |  | π/2-BPSK | ¼@519 | ¼@519 | 0 |
| 10MHz |  |  |  |  | QPSK | 6@09, 15 | 6@0 | 0 |
| 10MHz |  |  |  |  | QPSK | 6@09, 15 | 6@0 | 7 |
| 10MHz |  |  |  |  | QPSK | 1@010, 14 | 1@0 | 0 |
| 10MHz |  |  |  |  | QPSK | 1@510, 14 | 1@5 | 7 |
| 15MHz |  |  |  |  | π/2-BPSK | ¼@019 | ¼@019 | 0 |
| 15MHz |  |  |  |  | π/2-BPSK | ¼@519 | ¼@519 | 0 |
| 15MHz |  |  |  |  | QPSK | 6@03, 11 | 6@04, 18 | 0 |
| 15MHz |  |  |  |  | QPSK | 6@03, 11 | 6@04, 18 | 11 |
| 15MHz |  |  |  |  | QPSK | 1@0 | 1@05 | 0 |
| 15MHz |  |  |  |  | QPSK | 1@5 | 1@55 | 11 |
| 20MHz |  |  |  |  | π/2-BPSK | ¼@019 | ¼@019 | 0 |
| 20MHz |  |  |  |  | π/2-BPSK | ¼@519 | ¼@519 | 0 |
| 20MHz |  |  |  |  | QPSK | 6@03, 11 | 6@04, 18 | 0 |
| 20MHz |  |  |  |  | QPSK | 6@03, 11 | 6@04, 18 | 15 |
| 20MHz |  |  |  |  | QPSK | 1@0 | 1@05 | 0 |
| 20MHz |  |  |  |  | QPSK | 1@5 | 1@55 | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: For requirements under note 21 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C in high range with max narrowband index. Note 4: Void Note 5: Void Note 6: Do not apply for requirements under Note 13 in Tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C.. Test frequencies for these requirements are defined in Table 6.6.3EA.2.4.1-2. Note 7: Void Note 8: For requirements under note 23 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C (high range – 10MHz) is tested at RB #0 with narrowband index 0. Note 9: For requirements under note 23 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C high range is tested at RB #0 with narrowband index 1. Note 10: For requirements under note 23 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C high range is tested at RB #3 with narrowband index 0 and at RB#2 with narrowband index 7. Note 11: For requirements under note 27 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C in low range with narrowband index 0. Note 12: Void Note 13: For requirements under note 31 in table 6.6.3.2.3-1C and note 34 in table 6.6.3.2.3-1D only 720.5 MHz is tested with 5 MHz and only 723 MHz is tested with 10 MHz. Note 14: For requirements under note 34 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C RB#1 with narrowband index 0 and RB#4 with narrowband index 7 are used. Note 15: For requirements under note 34 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C RB#0 with narrowband index 1 is used Note 16: For requirements under note 35 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C only low range and 728 MHz with 10 MHz is tested. Note 17: Void Note 18: For requirements under note 33 in tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C (high range-7 MHz) is tested. Note 19: Only applicable for UE supporting subPRB allocation. |  |  |  |  |  |  |  |  |

Table 6.6.3EA.2.4.1-2: Test frequencies for E-UTRA channel bandwidth for operating band 3 
with Note 13 (in Tables 6.6.3EA.2.3-1)

| Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 5 | 19924 | 1782.4 | 1924 | 1877.4 |
| 10 | 19899 | 1779.9 | 1899 | 1874.9 |
| 15 | 19874 | 1777.4 | 1874 | 1872.4 |
| 20 | 19849 | 1774.9 | 1849 | 1869.9 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3EA.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.3EA.2.4.3.

6.6.3EA.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.3EA.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1C. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.6.3EA.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.6.3EA.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.3EA.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in table 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1E according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1E. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.6.3EA.2.5-1.

Table 6.6.3EA.2.5-1: UE Requirements according to UE E-UTRA release and supported E-UTRA band

| Band |  |  | UE Requirements per release |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rel-13 |  | Rel-14 |  | Rel-15 |  | Rel-16 |  | Rel-17 | Rel-18 |
| 1 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 2 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 3 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 4 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 5 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 7 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 8 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 11 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 12 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 13 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 14 | Table 6.6.3EA.2.3-1B | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 18 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 19 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 20 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 21 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 26 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 27 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 28 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 31 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 39 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 41 | Table 6.6.3EA.2.3-1 | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 42 | Table 6.6.3EA.2.3-1C | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 43 | Table 6.6.3EA.2.3-1C | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 66 | Table 6.6.3EA.2.3-1A | Table 6.6.3EA.2.3-1A |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |
| 71 | Table 6.6.3EA.2.3-1B | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1B |  | Table 6.6.3EA.2.3-1C |  | Table 6.6.3EA.2.3-1D |  | Table 6.6.3EA.2.3-1E |

NOTE 1:	The following is applied to Note2 in Tables 6.6.3EA.2.3-1 to 6.6.3EA.2.3-1E. For frequency with 2nd, 3rd or 4th harmonic spurious emissions, the measurements are covered in 6.6.3.1.

NOTE 2:	The restriction on the maximum uplink transmission to 36 RB in Notes 14 and 15 of Table 6.6.3EA.2.5-1 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future.6.6.3EA.3	Additional spurious emissions for UE category M1

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.6.3EC.2	Spurious emission band UE co-existence for UE category M2

##### 6.6.3EC.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

##### 6.6.3EC.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.3EC.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified E-UTRA band for coexistence with protected bands as indicated in Table 6.6.3EC.2.3-1, Table 6.6.3EC.2.3-1A, Table 6.6.3EC.2.3-1B. Table 6.6.3EC.2.3-1C and Table 6.6.3EC.2.3-1D.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<< Unchanged text skipped>>

Table 6.6.3EC.2.3-1C: Spurious emission band limits Rel-17

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 48, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

Table 6.6.3EC.2.3-1D: Spurious emission band limits Rel-18

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 54, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 48, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 54, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.3.2.

This test use minimum requirements from many releases of TS 36.101 [2] due to release independence defined in TS 36.307 [16]

##### 6.6.3EC.2.4	Test description

6.6.3EC.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3EC.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.3EC.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) (Note 6, Note 7) |  |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |  |
|  |  | FDD | TDD |  | FDD | TDD | Narrowband index1 |
| 1.4MHz |  |  |  | π/2-BPSK | ¼@012, 15 | ¼@015 | 0 |
| 1.4MHz |  |  |  | π/2-BPSK | ¼@512, 15 | ¼@515 | 0 |
| 1.4MHz | N/A for Spurious Emissions testing |  |  | QPSK | 6@012 | 6@0 | 0 |
| 1.4MHz |  |  |  | QPSK | 1@012 | 1@0 | 0 |
| 1.4MHz |  |  |  | QPSK | 1@512 | 1@5 | 0 |
| 3MHz |  |  |  | π/2-BPSK | ¼@06, 15 | ¼@015 | 0 |
| 3MHz |  |  |  | π/2-BPSK | ¼@56, 15 | ¼@515 | 0 |
| 3MHz |  |  |  | QPSK | 12@06 | 12@0 | 0 |
| 3MHz |  |  |  | QPSK | 1@06 | 1@0 | 0 |
| 3MHz |  |  |  | QPSK | 1@56 | 1@5 | 1 |
| 5MHz |  |  |  | π/2-BPSK | ¼@03, 7, 8, 12, 15 | ¼@09, 10, 15 | 0 |
| 5MHz |  |  |  | π/2-BPSK | ¼@53, 7, 8, 12, 15 | ¼@59, 10, 15 | 0 |
| 5MHz |  |  |  | QPSK | 24@03, 7, 8, 12 | 24@09, 10 | 0 |
| 5MHz |  |  |  | QPSK | 1@03, 7, 8, 12 | 1@09, 10 | 0 |
| 5MHz |  |  |  | QPSK | 1@53, 7, 8, 12 | 1@59, 10 | 3 |
| 10MHz |  |  |  | π/2-BPSK | ¼@05, 15 | ¼@015 | 0 |
| 10MHz |  |  |  | π/2-BPSK | ¼@55, 15 | ¼@515 | 0 |
| 10MHz |  |  |  | QPSK | 24@04 | 24@0 | 0 |
| 10MHz |  |  |  | QPSK | 24@04 | 24@0 | 4 |
| 10MHz |  |  |  | QPSK | 1@05 | 1@0 | 0 |
| 10MHz |  |  |  | QPSK | 1@55 | 1@5 | 7 |
| 15MHz |  |  |  | π/2-BPSK | ¼@015 | ¼@015 | 0 |
| 15MHz |  |  |  | π/2-BPSK | ¼@515 | ¼@515 | 0 |
| 15MHz |  |  |  | QPSK | 24@0 | 24@0 | 0 |
| 15MHz |  |  |  | QPSK | 24@0 | 24@0 | 8 |
| 15MHz |  |  |  | QPSK | 1@0 | 1@0 | 0 |
| 15MHz |  |  |  | QPSK | 1@5 | 1@5 | 11 |
| 20MHz |  |  |  | π/2-BPSK | ¼@06, 13, 14, 15 | ¼@09, 11, 15 | 0 |
| 20MHz |  |  |  | π/2-BPSK | ¼@56, 13, 14, 15 | ¼@59, 11, 15 | 0 |
| 20MHz |  |  |  | QPSK | 24@06 | 24@0 | 0 |
| 20MHz |  |  |  | QPSK | 24@06, 13, 14 | 24@09, 11 | 12 |
| 20MHz |  |  |  | QPSK | 1@06, 13, 14 | 1@09, 11 | 0 |
| 20MHz |  |  |  | QPSK | 1@56, 13, 14 | 1@59, 11 | 15 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: Requirements under note 23 in table 6.6.3EC.2.3-1 (5MHz) are tested only in High range. Note 4: Requirements under note 23 in table 6.6.3EC.2.3-1 (10MHz) are tested only in High range at RB #0 with narrowband index 4. Note 5: Requirements under note 23 in table 6.6.3EC.2.3-1 (10MHz) are tested only in High range at RB #0 with narrowband index 7. Note 6: Do not apply for requirements under Note 19, Note 34 and Note 35 in Table 6.6.3EA.2.3-1. Note 7: Requirements under note 19 in table 6.6.3EC.2.3-1 are tested only for 5MHz channel bandwidth and only in Mid and High range. Note 8: Do not apply for requirements under Note 35 in Table 6.6.3EA.2.3-1. Note 9: Requirements under note 33 in table 6.6.3EC.2.3-1 are tested only in Mid and High range. Note 10: Do not apply for requirements under Note 30 in Table 6.6.3EA.2.3-1. Note 11: Requirements under note 30 in table 6.6.3EC.2.3-1 are tested only for centre frequency at 2560 MHz. Note 12: Requirements under note 41 in table 6.6.3EC.2.3-1 are tested only in Mid and High range. Note 13: Requirements under note 41 in table 6.6.3EC.2.3-1 are tested only in High range. Note 14: Requirements under note 42 in table 6.6.3EC.2.3-1 are tested only in Low and Mid range. Note 15: Only applicable for UE supporting subPRB allocation |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3EC.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.3EC.2.4.3.

6.6.3EC.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.3EC.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.3EC.2.3-1 to 6.6.3EC.2.3-1D. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.6.3EC.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.6.3EC.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.3EC.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in tables 6.6.3EA.2.3-1 to 6.6.3EC.2.3-1D according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3EA.2.3-1 to 6.6.3EC.2.3-1D. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.6.3EC.2.5-1.

Table 6.6.3EC.2.5-1: UE Requirements according to UE E-UTRA release and supported E-UTRA band

| Band | UE Requirements per release |  | UE Requirements per release |  | UE Requirements per release |  | UE Requirements per release |  | UE Requirements per release |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rel-14 |  | Rel-15 |  | Rel-16 |  | Rel-17 |  | Rel-18 |  |
| 1 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 2 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 3 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 4 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 5 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 7 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 8 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 11 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 12 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 13 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 14 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 18 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 19 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 20 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 21 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 25 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 26 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 27 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 28 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 31 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 39 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 40 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 41 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 42 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 43 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 66 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 70 | Table 6.6.3EC.2.3-1 | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 71 | Table 6.6.3EC.2.3-1A | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 72 | Table 6.6.3EC.2.3-1A | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 73 | Table 6.6.3EC.2.3-1A | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 74 | Table 6.6.3EC.2.3-1A | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |
| 85 | Table 6.6.3EC.2.3-1A | Table 6.6.3EC.2.3-1A |  | Table 6.6.3EC.2.3-1B |  | Table 6.6.3EC.2.3-1C |  | Table 6.6.3EC.2.3-1D |  |

NOTE 1:	The following is applied to Note2 in Tables 6.6.3EC.2.3-1 to 6.6.3EC.2.3-1D. For frequency with 2nd, 3rd or 4th harmonic spurious emissions, the measurements are covered in 6.6.3.1.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.6.3F.2	Spurious emission band UE co-existence for category NB1 and NB2

Editor's note: This test case contains different requirements for different UE releases

6.6.3F.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.6.3F.2.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 13 and forward of category NB1.

This test case applies to all types of E-UTRA FDD UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

6.6.3F.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified E-UTRA band for coexistence with protected bands as indicated in Tables 6.6.3F.2.3-1, 6.6.3F.2.3-1A, 6.6.3F.2.3-1B, 6.6.3F.2.3-1C, 6.6.3F.2.3-1D, and 6.6.3F.2.3-1E.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<< Unchanged text skipped>>

Table 6.6.3F.2.3-1D: Spurious emission band UE co-existence limits Rel-17

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 53, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 22, 28, 31, 38, 40, 42, 43, 47, 50, 51, 65, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 103 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: N/A NOTE 6: N/A NOTE 7: N/A NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575 MHz or 2595-2645 MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4, 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

Table 6.6.3F.2.3-1E: Spurious emission band UE co-existence limits Rel-18

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 54, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 54, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 54, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 54, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 53, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 22, 28, 31, 38, 40, 42, 43, 47, 50, 51, 65, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 54, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 103 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: N/A NOTE 6: N/A NOTE 7: N/A NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575 MHz or 2595-2645 MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4, 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

NOTE:	The restriction on the maximum uplink transmission to 54 RB in Notes 21, 22, and 27 of Table 6.6.3F.2.3-1D and the restriction on the single-tone uplink transmission to sub-carrier index > 2 in Note 44 of Table 6.6.3F.2.3-1D are intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future.

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.3.2.

This test use minimum requirements from many releases of TS 36.101 [2] due to release independence defined in TS 36.307 [16]

##### 6.6.3F.2.4	Test description

6.6.3F.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters, and are shown in Table 6.6.3F.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.6.3F.2.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.3 (Note 2) |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Spurious Emissions testing | Modulation | Ntones | Subcarrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | BPSK | 1@0 | 15 |
| 4 |  | BPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| 6 (Note 3) |  | QPSK | 1@5 | 3.75 |
| 7 (Note 3) |  | QPSK | 1@3 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions Note 2: Do not apply for requirements under Note 13 and Note 34 in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C. Test frequencies for these requirements are defined in Table 6.6.3F.2.4.1-2 and 6.6.3F.2.4.1-3. Note 3: Only apply for requirements under Note 39 in Table 6.6.3F.2.3-1 to Table 6.6.3F.2.3-1C, Note 42 in Table 6.6.3F.2.3-1A to Table 6.6.3F.2.3-1C and Note 44 in Table 6.6.3F.2.3-1B to Table 6.6.3F.2.3-1C, replacing Configuration ID 1 and 3. |  |  |  |  |

Table 6.6.3F.2.4.1-2: Test frequencies for E-UTRA channel bandwidth for operating band 3 
with Note 13 (in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C)

| Bandwidth [kHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 200 | 19948 | 1784.8 | 1948 | 1879.8 |

Table 6.6.3F.2.4.1-3: Test frequencies for E-UTRA channel bandwidth for operating band 28 
with Note 34 (in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1C)

| Bandwidth [kHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 200 | 27361 | 718.1 | 9361 | 773.1 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.7 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3F.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.6.3F.2.4.3.

6.6.3F.2.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.6.3F.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1E. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.6.3F.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

NOTE 2:	For requirements under Note 39 in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1E, Note 42 in Table 6.6.3F.2.3-1A to 6.6.3F.2.3-1E and Note 44 in Tables 6.6.3F.2.3-1B to 6.6.3F.2.3-1E, the resolution bandwidth is <=1 kHz to improve the measurement accuracy.

6.6.3F.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

##### 6.6.3F.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 2, shall not exceed the described value in tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1E according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3F.2.3-1 to 6.6.3F.2.3-1E. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.6.3F.2.5-1.

Table 6.6.3F.2.5-1: UE Requirements according to UE NB-IoT release and supported NB-IoT band

| Band | UE Requirements per release |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rel-13 | Rel-14 | Rel-15 | Rel-16 | Rel-17 | Rel-18 |
| 1 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 2 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 3 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 4 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 5 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 7 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 8 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 11 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 12 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 13 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 14 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 17 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 18 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 19 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 20 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 21 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 25 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 26 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 28 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 31 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 42 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 43 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 65 | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 66 | Table 6.6.3F.2.3-1 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 70 | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1A | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 71 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 72 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 74 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 85 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |
| 103 | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1B | Table 6.6.3F.2.3-1C | Table 6.6.3F.2.3-1D | Table 6.6.3F.2.3-1E |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------