3GPP TSG-RAN5 Meeting #100	R5-233935r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3861 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UPIP TC 8.2.6.4.1-RRC establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPIP_SEC_LTE-RAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test case is added to check EN-DC capable UE connected to EUTRA/EPC could perform UPIP(user plane integrity protection) correctly after RRC establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UPIP TC 8.2.6.4.1 UPIP / RRC connecton establishment / Reception of SecurityModeCommand by UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPIP_SEC_LTE-RAN-UEConTest will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0367 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Table 8.2.6.4.1.3.2-1: rewording of step 4, correction of the expected sequence in step5; Editorial corrections in tables 8.2.6.4.1.3.3-4 and 8.2.6.4.1.3.3-5; Correction of algorithms in table 8.2.6.4.1.3.3-5 |  |  |  |  |  |  |  |  |

#### <Start of modified section>

#### 8.2.6.4	UPIP / RRC connection

##### 8.2.6.4.1	UPIP / RRC connection establishment / Reception of SecurityModeCommand by UE

8.2.6.4.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state configured with NR-PDCP and functionality of UP integrity algorithms }

ensure that {
  when { UE successfully completes the RRC Connection establishment procedure and receives the SecurityModeCommand message to establish a DRB }

then { UE establishes the initial security configuration in accordance with the received securityConfigSMC included in SecurityModeCommand and successfully establish the data and signalling radio bearers }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and a MN terminated MCG DRB is configured with NR-PDCP and functionality of UP integrity algorithms is taken into use on this DRB }

ensure that {
  when { SS transmits a IP data with correct integrity protection on this DRB }

then { UE loops back this IP data with correct integrity protection }

}

8.2.6.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clause 5.3.1.2, 5.3.4.3 and TS 38.331 clause 5.3.5.6.5. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.3.1.2]

...

Integrity protection is optionally supported for DRBs when using NR PDCP configured with nr-RadioBearerConfig1 or nr-RadioBearerConfig2.

...

The integrity protection algorithm is common for signalling radio bearers SRB1, SRB2 and SRB4. The integrity protection algorithm signalled in nr-RadioBearerConfig1/ nr-RadioBearerConfig2 for the DRBs configured to apply integrity protection of user data and keyToUse set to master as defined in TS 38.331 [82] is the same as the one signalled in securityAlgorithmConfig. When configured with MCG only, the ciphering algorithm is common for all radio bearers (i.e. SRB1, SRB2, SRB4 and DRBs). Neither integrity protection nor ciphering applies for SRB0.

…

The AS applies different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc) and one for the ciphering of user data (KUPenc). For the UE capable of user plane integrity protection when it is connected to E-UTRA/EPC (TS 33.401 [32]), the AS applies a security key for integrity protection of user data (KUPint) for the DRBs that are configured to apply integrity protection of user data. All AS keys are derived from the KeNB key

…

Likewise, the same integrity algorithm as signalled by nr-RadioBearerConfig1 and nr-RadioBearerConfig2 as defined in TS 38.331 [82] is used for all SRBs, and DRBs configured to apply integrity protection of user data, using the same key

…

[TS 36.331, clause 5.3.4.3]

The UE shall:

1>    derive the KeNB key, as specified in TS 33.401 [32] for E-UTRA/EPC, and TS 33.501 [86] for E-UTRA/5GC;

...

1>    if the SecurityModeCommand message passes the integrity protection check:

…

2>    if capable of user plane integrity protection:

3>    derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.401 [32];

[TS 36.331, clause 5.3.5.3]

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2>	re-establish PDCP for SRB2 configured with E-UTRA PDCP entity and for all DRBs that are established and configured with E-UTRA PDCP, if any;

2>	re-establish RLC for SRB2 and for all DRBs that are established and configured with E-UTRA RLC, if any;

2>	if the RRCConnectionReconfiguration message includes the fullConfig:

3>	perform the radio configuration procedure as specified in 5.3.5.8;

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10.0;

…

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2>	resume SRB2 and all DRBs that are suspended, if any, including RBs configured with NR PDCP;

[TS 36.331, clause 5.3.7.5]

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	except for a NB-IoT UE for which AS security has not been activated:

…

2>	if UE is connected to EPC, update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

…

2>	if capable of user plane integrity protection:

3>	derive the KUPint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

[TS 38.331, clause 5.3.5.6.5]

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

…

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

…

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

8.2.6.4.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1

UE:

-	None

Preamble:

-	UE is in state 1E-A EUTRA RRC_IDLE using generic procedure parameter Connectivity (E-UTRA/EPC) and Test Mode (On) and Test Loop Function (Off) associated with UE test loop mode B configured on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.2.

8.2.6.4.1.3.2	Test procedure sequence

Table 8.2.6.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-3 | The UE performs steps 3 to 5 of generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) | - | - | - | - |
| 4 | The SS transmits a SecurityModeCommand message to activate AS security with integrity protection algorithm. | <-- | SecurityModeCommand | - | - |
| 5 | Check: Does the UE transmit a SecurityModeComplete message and establishes the initial security configuration. | --> | SecurityModeComplete | 1 | P |
| 6 | The SS transmits an RRCConnectionReconfiguration message to establish a MN terminated MCG DRB using NR-PDCP and enable integrity protection for this DRB. The DRB is associated with the default EPS bearer context. | <-- | RRCConnectionReconfiguration | - | - |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 8-9 | Steps 1 to 2 of Loopback activation procedure in TS 36.508 Table 4.5.4.3-1 are executed to activate test loop mode B on the DRB configured in Step 7. | - | - | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n configured in step7 and uplink IP data is correctly integrity protected? | - | - | 2 | - |

8.2.6.4.1.3.3	Specific message contents

Table 8.2.6.4.1.3.3-1: SecurityModeCommand message (step 4, Table 8.2.6.4.1.3.2-1)

| Derivation Path: TS 36.508 [7] Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| integrityProtAlgorithm |  | Set according to PIXIT parameter for default integrity protection algorithm | null algorithm is not allowed |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.6.4.1.3.3-2: RRCConnectionReconfiguration (step 6, Table 8.2.6.4.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8, condition SRB2-DRB(1,0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SRB2-MCG-DRB-NR-PDCP | Table 8.2.6.4.1.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 | Not present |  |  |
| nr-RadioBearerConfig1-r15 | RadioBearerConfig-MCG-DRB-NR-PDCP | Table 8.2.6.4.2.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.1.3.3-3: RadioResourceConfigDedicated-SRB2-MCG-DRB-NR-PDCP (Table 8.2.6.4.1.3.3-2)

| Derivation Path: TS 36.508 [7], Table 4.6.3-16, condition SRB2-DRB(1,0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] | DRB-ToAddMod-MCG-DRB-NR-PDCP | entry 1 Table 8.2.6.4.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.1.3.3-4: DRB-ToAddMod-MCG-DRB-NR-PDCP (Table 8.2.6.4.1.3.3-3)

| Derivation Path: TS 36.508 [7], Table 4.8.2.1.7-1, condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DRB-ToAddMod ::= SEQUENCE { |  |  |  |  |
| eps-BearerIdentity |  | Same as the default EPS bearer Identity |  |  |
| drb-Identity |  | Same as the DRB identity associated with the default EPS bearer |  |  |
| pdcp-Config |  | Not present |  |  |
| } |  |  |  |  |

Table 8.2.6.4.1.3.3-5: RadioBearerConfig-MCG-DRB-NR-PDCP (Table 8.2.6.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132, condition MCG_NR_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | Same as the default EPS bearer Identity |  |  |
| } |  |  |  |
| drb-Identity | Same as the DRB identity associated with the default EPS bearer |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | Table 8.2.6.4.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Algorithm configured in step 4. |  |  |
| integrityProtAlgorithm | Algorithm configured in step 4. |  |  |
| } |  |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.1.3.3-6: PDCP-Config (Table 8.2.6.4.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| integrityProtection | true |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>