3GPP TSG-RAN5 Meeting #100	R5-233947r1

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2315 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of spurious emissions test requirements for CA_n20A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing spurious emissions test requirements for CA_n20A-n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added spurious emissions test requirements for CA_n20A-n78A into - Table 6.5A.3.1.1.5-1: General spurious emissions test requiurements - Table 6.5A.3.2.0.3-2: Requirements for uplink inter-band CA Rel-16 - Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table - Table 6.5A.3.2.1.5-1: Requirements for uplink inter-band carrier aggregation according to UE NR release |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions test requirements for CA_n20A-n78A will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905 ... CR 0784 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add configuration also to table 6.5A.3.2.0.3-3 for Rel-17 |  |  |  |  |  |  |  |  |

-------- Start of changes --------

6.5A.3.1.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For inter-band CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier, then it does not apply.

For intra-band contiguous CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth.

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA configuration.

Table 6.5A.3.1.1.5-1: General spurious emissions test requirements for inter-band UL CA

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| Test requirements for CA_n1A-n3A Configuration |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz | -36 dBm+TT | 100kHz |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5550 MHz 5550 MHz ≤ f ≤ 5745 MHz | -30 dBm+TT | 1MHz |  |
| Test requirements for CA_n1A-n8A Configuration |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz | -36 dBm+TT | 100kHz |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz | -30 dBm+TT | 1MHz |  |
| Test requirements for CA_n2A-n5A Configuration |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz | -36 dBm+TT | 100 kHz |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n2A-n48A Configuration |  |  |  |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 270 MHz | -36 dBm | 100 kHz |  |
| 1640 MHz ≤ f ≤ 1850 MHz 5190 MHz ≤ f ≤ 5610 MHz 7250 MHz ≤ f ≤ 7520 MHz 8950 MHz ≤ f ≤ 9310 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_n2A-n77A Configuration |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz | -36 dBm +TT | 100 kHz |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz | -30 dBm +TT | 1 MHz |  |
| Test requirements for CA_n3A-n78A Configuration |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz | -36 dBm+TT | 100 kHz |  |
| 1515MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n5A-n48A Configuration |  |  |  |
| 1852 MHz ≤ f ≤ 2052 MHz 2701 MHz ≤ f ≤ 2876 MHz 4374 MHz ≤ f ≤ 4549 MHz 5198 MHz ≤ f ≤ 5398 MHz 6251 MHz ≤ f ≤ 6576 MHz 7924 MHz ≤ f ≤ 8249 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_n5A-n77A Configuration |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz | -30 dBm +TT | 1 MHz |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n20A-n78A Configuration |  |  |  |
| 1576 MHz ≤ f ≤ 2136 MHz 2438 MHz ≤ f ≤ 2968 MHz 4132 MHz ≤ f ≤ 4662 MHz 4964 MHz ≤ f ≤ 5524 MHz 5738 MHz ≤ f ≤ 6768 MHz 7432 MHz ≤ f ≤ 8462 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n41A Configuration |  |  |  |
| 563 MHz ≤ f ≤ 825 MHz 835.5 MHz ≤ f ≤ 1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1063.5 MHz 3331.5 MHz ≤ f ≤ 3753.5 MHz 4122.5 MHz ≤ f ≤ 4350.5 MHz 5749 MHz ≤ f ≤ 6011 MHz 6618.5 MHz ≤ f ≤ 7040.5 MHz | -25 dBm+TT | 1 MHz | NS-01 signalled for n41 |
|  | -30 dBm+TT | 1 MHz | NS-04 signalled for n41 |
| Test requirements for CA_n24A-n48A Configuration |  |  |  |
| 229 MHz ≤ f ≤ 447 MHz | -36 dBm+TT | 100 kHz |  |
| 1889.5 MHz ≤ f ≤ 2073.5 MHz 5176.5 MHz ≤ f ≤ 5360.5 MHz 5439.5 MHz ≤ f ≤ 5773.5 MHz 6803 MHz ≤ f ≤ 7021 MHz 8726.5 MHz ≤ f ≤ 9060.5 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n77A Configuration |  |  |  |
| 21 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 947 MHz | -36 dBm+TT | 100 kHz |  |
| 1639.5 MHz ≤ f ≤ 2573.5 MHz 4926.5 MHz ≤ f ≤ 7521 MHz 8226.5 MHz ≤ f ≤ 10060.5 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n26A-n66A Configuration |  |  |  |
| 12 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f ≤ 152 MHz 861 MHz ≤ f ≤ 966 MHz | -36 dBm+TT | 100 kHz |  |
| 2524 MHz ≤ f ≤ 2746 MHz 3338 MHz ≤ f ≤ 3478 MHz 4234 MHz ≤ f ≤ 4409 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n26A-n70A Configuration |  |  |  |
| 3 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 82 MHz | -36 dBm+TT | 100 kHz |  |
| 846 MHz ≤ f ≤ 896 MHz | -36 dBm+TT | 100 kHz |  |
| 2509 MHz ≤ f ≤ 2606 MHz 3323 MHz ≤ f ≤ 3408 MHz 4204 MHz ≤ f ≤ 4269 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz | -25 dBm+TT | 1MHz |  |
| Test requirements for CA_n28A-n78A Configuration |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n41A-n79A Configuration |  |  |  |
| 8 MHz ≤ f ≤30 MHz | -36 dBm+TT | 10kHz |  |
| 30 MHz ≤ f ≤980 MHz | -36 dBm+TT | 100kHz |  |
| 1710MHz ≤ f ≤ 2504MHz | -30 dBm+TT | 1MHz |  |
| 6110MHz ≤ f ≤ 7690MHz |  |  |  |
| 9392MHz ≤ f ≤ 10380MHz |  |  |  |
| 11296MHz ≤ f ≤ 12690MHz |  |  |  |
| Test requirements for CA_n48A-n66A Configuration |  |  |  |
| 10 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 280 MHz | -36 dBm+TT | 100 kHz |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n48A-n70A Configuration |  |  |  |
| 130 MHz ≤ f ≤ 310 MHz | -36 dBm+TT | 100 kHz |  |
| 1840 MHz ≤ f ≤ 2005 MHz 5245 MHz ≤ f ≤ 5705 MHz 6940 MHz ≤ f ≤ 7120 MHz 8795 MHz ≤ f ≤ 9110 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n48A-n71A Configuration |  |  |  |
| 2154 MHz ≤ f ≤ 2374 MHz 2852 MHz ≤ f ≤ 3037 MHz 4213 MHz ≤ f ≤ 4398 MHz 4876 MHz ≤ f ≤ 5096 MHz 6402 MHz ≤ f ≤ 6737 MHz 7763 MHz ≤ f ≤ 8098 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |
| 314 MHz ≤ f < 454 MHz | -36 dBm+TT | 100 kHz |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n66A-n77A Configuration |  |  |  |
| 260 MHz ≤ f ≤ 780 MHz | -36 dBm +TT | 100 kHz |  |
| 1520 MHz ≤ f ≤ 2490 MHz 4820 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz | -30 dBm +TT | 1 MHz |  |
| Test requirements for CA_n70A-n71A Configuration |  |  |  |
| 299 MHz ≤ f < 384 MHz 997 MHz ≤ f < 1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1047 MHz 2358 MHz ≤ f ≤ 2408 MHz 2692 MHz ≤ f ≤ 2757 MHz 3021 MHz ≤ f ≤ 3106 MHz 4053 MHz ≤ f ≤ 4118 MHz | -30 dBm+TT | 1 MHz |  |

Table 6.5A.3.1.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

-------- Skipped unchanged clauses --------

###### 6.5A.3.2.0.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 apply on each component carrier with both component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1; Table 6.5A.3.2.0.3-2 and Table 6.5A.3.2.0.3-3 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.0.3-1: Requirements for uplink inter-band carrier aggregation (two bands) Rel-15

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.0.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Table 6.5A.3.2.0.3-2: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n2-n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, 48, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n2A-n77A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | Same requirements as in Table 6.5A.3.2.0.3-1 |  |  |  |  |  |  |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n20-n78 | E-UTRA Band 1, 3, 7, 8, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n8-n78 | Same requirements as in Table 6.5A.3.2.0.3-1 |  |  |  |  |  |  |
| CA_n28-n40 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 28, 31, 34, 38, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.3-3: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n2-n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 48, 50, 51 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n2A-n77A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 65, 66, 70, 71, 73, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1,8, 11, 20, 21, 28, 34, 39, 40, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n20-n78 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n26-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 53 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n41 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n41-n79 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 | 16 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 16 |
| CA_n48-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: The same configuration applies to corresponding NR-DC configuration in Table 4.3.1.1.7.1-1 in [5]. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in this test. |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  | UL Allocation |  |  |  |  |  |  |  |
|  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  | PCC NRB @SCS |  |  | SCC NRB@SCS |  |  | CC MOD |  | PCC & SCC RB allocation |  |  |  |  | CC MOD |  |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  | PCC |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC |  | SCC |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  | Range |  |  | Band |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n1 |  |  | Low |  |  | n3 |  |  | Low |  |  | 25@15kHz |  |  | 25@15kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  |  | 1@0 |
| 2 |  | n1 |  |  | Mid |  |  | n3 |  |  | Low |  |  | 25@15kHz |  |  | 25@15kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  |  | 1@24 |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  | Low |  |  | n8 |  |  | Low |  |  | 25@15kHz |  |  | 25@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  | Low |  |  | n78 |  |  | Low |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@272 |
| 2 | n1 |  |  | Low |  |  | n78 |  |  | 3421,14 MHz (NOTE 2) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 3 | n1 |  |  | Low |  |  | n78 |  |  | High |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  | LOW |  |  | n79 |  |  | MID |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@181 |
| 2 | n1 |  |  | LOW |  |  | n79 |  |  | 4618,41 MHz (NOTE 3) |  |  | 79@15kHz |  |  | 216@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 3 | n1 |  |  | LOW |  |  | n79 |  |  | MID |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@245 |
| 4 | n1 |  |  | LOW |  |  | n79 |  |  | 4785,54 MHz (NOTE 4) |  |  | 106@15kHz |  |  | 216@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  |  | Low |  |  | n5 |  |  | low |  |  | 20/106 |  |  | 20/106 |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 2 | n2 |  |  | High |  |  | n5 |  |  | High |  |  | 20/106 |  |  | 20/106 |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@105 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  |  | Low |  |  | n77 |  |  | Mid |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@272 |
| 2 | n2 |  |  | High |  |  | n77 |  |  | 3850,23 MHz (NOTE 5) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 3 | n2 |  |  | High |  |  | n77 |  |  | 3965,25 MHz (NOTE 6) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 |  |  | HIGH |  |  | n41 |  |  | HIGH |  |  | 160@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@151 |
| 2 | n3 |  |  | LOW |  |  | n41 |  |  | HIGH |  |  | 160@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@149 |  |  |  | 1@18 |
| 3 | n3 |  |  | LOW |  |  | n41 |  |  | HIGH |  |  | 133@15kHz |  |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@132 |  |  |  | 1@155 |
| 4 | n3 |  |  | HIGH |  |  | n41 |  |  | LOW |  |  | 160@15kHz |  |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@151 |
| 5 (NOTE 7) | n3 |  |  | HIGH |  |  | n41 |  |  | HIGH |  |  | 160@15kHz |  |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@159 |  |  |  | 1@105 |
| 6 (NOTE 7) | n3 |  |  | LOW |  |  | n41 |  |  | HIGH |  |  | 79@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | - CP-OFDM QPSK |  |  | 1@78 |  |  |  | 1@153 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 |  |  | LOW |  |  | n78 |  |  | HIGH |  |  | 160@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@159 |  |  |  | 1@272 |
| 2 | n3 |  |  | HIGH |  |  | n78 |  |  | LOW |  |  | 160@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@159 |  |  |  | 1@0 |
| 3 | n3 |  |  | LOW |  |  | n78 |  |  | MID |  |  | 133@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@262 |
| 4 | n3 |  |  | LOW |  |  | n78 |  |  | MID |  |  | 160@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@41 |
| Test Settings for CA_n5A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 |  |  | Low |  | n48 |  |  | Low |  |  |  | 106@15kHz |  |  | 106@30kHz |  | CP-OFDM QPSK |  |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |  |
| 2 | n5 |  |  | High |  | n48 |  |  | High |  |  |  | 106@15kHz |  |  | 106@30kHz |  | CP-OFDM QPSK |  |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  | 1@105 |  |
| 3 | n5 |  |  | Low |  | n48 |  |  | Low |  |  |  | 106@15kHz |  |  | 106@30kHz |  | CP-OFDM QPSK |  |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@105 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 |  |  | HIGH |  |  | n77 |  |  | MID |  |  | 79@15kHz |  |  | 162@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@161 |
| 2 | n5 |  |  | HIGH |  |  | n77 |  |  | MID |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@17 |
| 3 | n5 |  |  | LOW |  |  | n77 |  |  | 3458,79 MHz (NOTE 8) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 4 | n5 |  |  | HIGH |  |  | n77 |  |  | 3593,79 MHz (NOTE 9) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 5 | n5 |  |  | LOW |  |  | n77 |  |  | 4021,89 MHz (NOTE 10) |  |  | 52@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  |  | 1@0 |
| 6 | n5 |  |  | HIGH |  |  | n77 |  |  | HIGH |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@272 |
| 7 | n5 |  |  | LOW |  |  | n77 |  |  | 3538,29 MHz (NOTE 11) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 |  |  | HIGH |  |  | n78 |  |  | LOW |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 2 | n8 |  |  | LOW |  |  | n78 |  |  | MID |  |  | 79@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@192 |
| 3 | n8 |  |  | HIGH |  |  | n78 |  |  | HIGH |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@35 |
| 4 | n8 |  |  | LOW |  |  | n78 |  |  | LOW |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@0 |
| 5 | n8 |  |  | LOW |  |  | n78 |  |  | HIGH |  |  | 106@15kHz |  |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@139 |
| Test Settings for CA_n20A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n20 |  |  | Low |  |  | n78 |  |  | Mid |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 2 | n20 |  |  | Low |  |  | n78 |  |  | High |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@272 |
| 3 | n20 |  |  | High |  |  | n78 |  |  | Mid |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@0 |
| 4 | n20 |  |  | High |  |  | n78 |  |  | High |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@100 |
| 5 | n20 |  |  | High |  |  | n78 |  |  | 3500 MHz |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@0 |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 |  |  | High |  |  | n41 |  |  | Low |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@136 |
| 2 | n24 |  |  | Low |  |  | n41 |  |  | Low |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  |  | 1@108 |
| 3 | n24 |  |  | High |  |  | n41 |  |  | Low |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@136 |
| 4 | n24 |  |  | Low |  |  | n41 |  |  | Low |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@94 |
| 5 | n24 |  |  | High |  |  | n41 |  |  | High |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@192 |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 |  |  | High |  |  | n48 |  |  | Mid |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@136 |
| 2 | n24 |  |  | High |  |  | n48 |  |  | High |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 |  |  | Low |  |  | n77 |  |  | Mid |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@136 |
| 2 | n24 |  |  | Low |  |  | n77 |  |  | 3930 MHz |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@273 |
| 3 | n24 |  |  | Low |  |  | n77 |  |  | 3930 MHz |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@262 |
| 4 | n24 |  |  | Low |  |  | n77 |  |  | 3930 MHz |  |  | 52@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  |  | 1@196 |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 |  |  | High |  |  | n66 |  |  | High |  |  | 106@15kHz |  |  | 216@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@0 |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 |  |  | Low |  |  | n70 |  |  | Low |  |  | 106@15kHz |  |  | 79@15kHz |  |  | QPSK/CP-OFDM QPSK |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 |  | Low |  |  | n78 |  |  |  | Low |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 | n28 |  | High |  |  | n78 |  |  |  | High |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@272 |  |
| 3 | n28 |  | High |  |  | n78 |  |  |  | Low |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@0 |  |
| 4 | n28 |  | Low |  |  | n78 |  |  |  | Low |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@100 |  |
| 5 | n28 |  | High |  |  | n78 |  |  |  | Mid |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@0 |  |
| 6 | n28 |  | Mid |  |  | n78 |  |  |  | Mid |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@272 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 |  |  | LOW |  |  | n41 |  |  | HIGH |  |  | 79@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  |  | 1@157 |
| 2 | n28 |  |  | HIGH |  |  | n41 |  |  | HIGH |  |  | 79@15kHz |  |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  |  | 1@115 |
| 3 | n28 |  |  | HIGH |  |  | n41 |  |  | LOW |  |  | 79@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@194 |
| Test Settings for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 |  |  | LOW |  |  | n41 |  |  | HIGH |  |  | 216@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@137 |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 |  |  | LOW |  |  | n79 |  |  | LOW |  |  | 273@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  |  | 1@0 |
| 2 | n41 |  |  | HIGH |  |  | n79 |  |  | LOW |  |  | 273@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  |  | 1@259 |
| 3 | n41 |  |  | LOW |  |  | n79 |  |  | MID |  |  | 273@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  |  | 1@150 |
| 4 | n41 |  |  | HIGH |  |  | n79 |  |  | LOW |  |  | 273@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 5 | n41 |  |  | HIGH |  |  | n79 |  |  | LOW |  |  | 273@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@129 |  |  |  | 1@0 |
| 6 | n41 |  |  | HIGH |  |  | n79 |  |  | LOW |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 7 | n41 |  |  | HIGH |  |  | n79 |  |  | HIGH |  |  | 273@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  |  | 1@272 |
| 8 | n41 |  |  | HIGH |  |  | n79 |  |  | MID |  |  | 217@30kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@216 |  |  |  | 1@133 |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 |  |  | High |  |  | n66 |  |  | Low |  |  | 216@15kHz |  |  | 216@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 |  |  | High |  |  | n70 |  |  | High |  |  | 216@15kHz |  |  | 79@15kHz |  |  | QPSK |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  |  | 1@78 |
| 2 | n48 |  |  | High |  |  | n70 |  |  | Low |  |  | 216@15kHz |  |  | 79@15kHz |  |  | QPSK |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  |  | 1@0 |
| Test Settings for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 |  |  | HIGH |  |  | n71 |  |  | LOW |  |  | 106@15kHz |  |  | 106@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 2 | n48 |  |  | LOW |  |  | n71 |  |  | HIGH |  |  | 216@15kHz |  |  | 106@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@105 |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 |  |  | Low |  |  | n71 |  |  | Low |  |  | 216@15kHz |  |  | 106@15kHz |  |  | QPSK/CP-OFDM QPSK |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 |  |  | LOW |  |  | n77 |  |  | 3825,24 MHz (NOTE 12) |  |  | 106@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 2 | n66 |  |  | LOW |  |  | n77 |  |  | HIGH |  |  | 216@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@151 |
| 3 | n66 |  |  | LOW |  |  | n77 |  |  | 4065,18 MHz (NOTE 13) |  |  | 79@15kHz |  |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| 4 | n66 |  |  | LOW |  |  | n77 |  |  | HIGH |  |  | 216@15kHz |  |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@48 |
| 5 | n66 |  |  | LOW |  |  | n77 |  |  | HIGH |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@217 |
| 6 (NOTE 14) | n66 |  |  | LOW |  |  | n77 |  |  | HIGH |  |  | 79@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@214 |
| 7 | n66 |  |  | LOW |  |  | n77 |  |  | HIGH |  |  | 216@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@211 |
| 8 | n66 |  |  | LOW |  |  | n77 |  |  | MID |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@13 |
| 9 | n66 |  |  | HIGH |  |  | n77 |  |  | MID |  |  | 106@15kHz |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  |  | 1@24 |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 |  |  | Low |  |  | n71 |  |  | Low |  |  | 79@15kHz |  |  | 106@15kHz |  |  | QPSK/CP-OFDM QPSK |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  |  | 1@0 |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Test Point ID 2 for CA_n1A-n78A and Band n78 is defined as a carrier with CBW=10MHz and centre frequency at 3421.14MHz (NR ARFCN=628067). NOTE 3: Test Point ID 2 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4618.41MHz (NR ARFCN=707867). NOTE 4: Test Point ID 4 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4785.54MHz (NR ARFCN=719000). NOTE 5: Test Point ID 2 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3850.23MHz (NR ARFCN=656667). NOTE 6: Test Point ID 3 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3965.25MHz (NR ARFCN=664333). NOTE 7: Test point ID 5 and 6 for CA_n3A-n41A test protected NR band 100 introduced in Rel-17. NOTE 8: Test Point ID 3 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3458.79MHz (NR ARFCN=630533). NOTE 9: Test Point ID 4 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3593.79MHz (NR ARFCN=639533). NOTE 10: Test Point ID 5 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4021.89MHz (NR ARFCN=668067). NOTE 11: Test Point ID 7 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Test Point ID 1 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3825.24MHz (NR ARFCN=655000). NOTE 13: Test Point ID 3 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4065.18MHz (NR ARFCN=671000). NOTE 14: Test point ID 6 for CA_n66A-n77A tests protected E-UTRA band 103 introduced in Rel-17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are release specific and can be found in Table 6.5A.3.2.1.5-1. If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For intra-band contiguous CA, the measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.5A.3.2.1.5-3.

Table 6.5A.3.2.1.5-1: Requirements for uplink inter-band carrier aggregation according to UE NR release (two bands)

| NR CA | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration |  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | Rel-16 Rel-17 | E-UTRA Band 11, 21, 32, 43, 50, 65, 74, 75 NR Band n77, n78 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n1-n8 | Rel-16 Rel-17 | E-UTRA Band 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n1-n78 | Rel-16 Rel-17 | E-UTRA Band 3, 11, 21, 74 |  |  |  | -50+TT | 1 |  |
| CA_n1-n79 | Rel-16 Rel-17 | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n2-n5 | Rel-16 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 41, 43, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | Rel-16 Rel-17 | N/A (NOTE 16) |  |  |  |  |  |  |
| CA_n2-n77 | Rel-16 | E-UTRA Band 4, 65, 66, 70 | FDL_low | - | FDL_high | -50 +TT | 1 |  |
|  | Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 +TT | 1 | 2 |
| CA_n3-n41 | Rel-16 | E-UTRA Band 5, 8, 18, 19, 20, 26, 27, 28, 44 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Rel-17 | E-UTRA Band 5, 8, 18, 19, 20, 26, 27, 28, 44 NR Band n100 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n3-n78 | Rel-15 | E-UTRA Band 3, 34, 39, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-16 Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n5-n48 | Rel-17 | E-UTRA Band 2, 25, 70 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n77 | Rel-16 Rel-17 | E-UTRA Band 1, 2, 3, 4, 25, 30, 34, 40, 65, 66, 70 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n8-n78 | Rel-15 | E-UTRA Band 8, 20, 28, 34, 39, 40, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-16 | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n20-n78 | Rel-16 Rel-17 | E-UTRA Band 1, 3, 7, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n24-n41 | Rel-17 | E-UTRA Band 5, 12, 13, 14, 17, 26, 29, 48, 71, 85 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | NR Band n77 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n24-n48 | Rel-17 | E-UTRA Band 2, 25, 70 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n24-n77 | Rel-17 | E-UTRA Band 4, 10, 29, 30, 66, 71 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n26-n66 | Rel-17 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 42 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n26-n70 | Rel-17 | E-UTRA Band 5 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n28-n41 | Rel-16 | E-UTRA Band 2, 3, 25 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  |  | NR Band n77, n78, n79 |  |  |  | +TT |  |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3, 11 |
| CA_n28-n78 | Rel-16 Rel-17 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50+TT | 1 | 11,15 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3,11 |
| CA_n39-n41 | Rel-16 | E-UTRA Band 28, 44 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n41-n79 | Rel-16 Rel-17 | E-UTRA Band 1, 3, 5, 8, 18, 19, 21, 28, 34, 44, 65 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40+TT | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n48-n66 | Rel-16 Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n48-n70 | Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50+TT | 1 | 16 |
| CA_n48-n71 | Rel-17 | E-UTRA Band30, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n71 | Rel-16 Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n66-n77 | Rel-16 | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71,103 | FDL_low | - | FDL_high | -50 +TT | 1 |  |
| CA_n70-n71 | Rel-16 | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n78-n79 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 17: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-1a: Void

Table 6.5A.3.2.1.5-1b: Void

Table 6.5A.3.2.1.5-2: Test Tolerance for uplink inter-band carrier aggregation (two bands)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.2.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.2.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths for all CC combinations.

-------- End of changes --------