3GPP TSG-RAN5 Meeting #100	R5-233958r2

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5458 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test configuration and test requirement for CA_5A-30A-66A-66A and CA_12A-30A-66A-66A in TC7.3A.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SRTC,ZTE,Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test configuration and test requirement for the following 4DL/1UL CA combinations CA_5A-30A-66A-66A and CA_12A-30A-66A-66A in TC7.3A.9 need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 7.3A.9.4.1-2 and Table 7.3A.9.5-6 have been updated to include the CA combinations above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above CA combinations will still incomplete and related Rx TCs will still not be able to perform. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 7.3A.9.4.1-2 and Table 7.3A.9.5-6(7.3A.9) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 36.905 CR0255/CR0256 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:Correct CR number from 5457 to 5458 in coversheet. R2:Correct affected CR number from R5-233956/R5-233957 to CR0255/CR0256 in coversheet. |  |  |  |  |  |  |  |  |

<Start of Changes>

### 7.3A.9	Reference sensitivity level for 4DL CA

Editor’s note: Following aspects missing or not yet determined:

-	Reference sensitivity QPSK PREFSENS has defined and made for some CA bands, some of are incomplete.

#### 7.3A.9.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.9.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 4DL with CA configurations in Table 7.1-2b.

#### 7.3A.9.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.9.4	Test description

##### 7.3A.9.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.9.4.1-1 through table 7.3A.9.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<Unchanged sections omitted>

Table 7.3A.9.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  |  |  |  | Wgap [MHz] |  |  | SCC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wgap [MHz] |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  | Wgap [MHz] |  |  |  |  |  | SCC3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band | Range | NRB |  |  |  |  |  |  |  |  |  | Band |  |  | Range |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  |  | Range |  | NRB |  |  |  |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) |  |  |  | DLalloc |  |  |  |  |  | UL MOD |  |  | DL MOD |  |  |  |  | ULalloc |  |  |  | DLalloc |  |  |  |  |  |  |  | UL MOD |  | UL MOD |  | ULalloc |  |  | DLalloc |  |  |  |  |  |  |  |  |  | UL MOD |  |  |  |  |  | DL MOD |  | ULalloc |  |  |  | DLalloc |
| Default Test Settings for a CA_XA-XA-YA-ZA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | X |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | Y |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | Z |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| 2 | Y | Mid | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | X |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | X |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | Z |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| 3 | Z | Mid | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | X |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | X |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | Y |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| Default Test Settings for a CA_XA-ZA-YA-YA / CA_XA-YA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | Y |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | X |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | Z |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| 2 | X | Mid | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | Y |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | Y |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | Z |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| 3 | Z | Mid | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | Y |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | Y |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | X |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| Test Settings for CA_2A-2A-4A-5A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 4 | Low, High | 50@0 |  |  |  | ALL RBs |  |  | - |  |  | 5 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 5 | 4 | Low | 100@0 |  |  |  | ALL RBs |  |  | - |  |  | 5 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 5 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | Low, High |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 7 | 5 | Low | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-2A-4A-71A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 50@50 |  |  | ALL RBs |  |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 2 | 2 | CC2 | 16@57 |  |  | ALL RBs |  |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 3 | 2 | CC2 | 50@50 |  |  | ALL RBs |  |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | NOTE 18 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 4 | 2 | CC2 | 16@57 |  |  | ALL RBs |  |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 5 | 2 | CC2 | 100@0 |  |  | ALL RBs |  |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  | Mid |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | High |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 4 | Mid | 100@0 |  |  | ALL RBs |  |  |  | - |  |  | 71 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 7 | 71 | Low | 25@0 |  |  | ALL RBs |  |  |  | - |  |  | 4 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 25 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 25 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 8 | 71 | Low | 25@12 |  |  | ALL RBs |  |  |  | - |  |  | 4 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 50 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 9 | 71 | Low | 20@25 |  |  | ALL RBs |  |  |  | - |  |  | 4 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 75 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 10 | 71 | Low | 20@40 |  |  | ALL RBs |  |  |  | - |  |  | 4 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 11 | 71 | High | 100@0 |  |  | ALL RBs |  |  |  | - |  |  | 4 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-2A-5A-12A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 12 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 12 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 5 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 4 | 12 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 5 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-2A-12A-30A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 12 | Mid | 20@30 |  |  |  | ALL RBs |  | - |  |  | 30 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 5 | 30 | Mid | 25@25 |  |  |  | ALL RBs |  | - |  |  | 12 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-2A-66A-71A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 50@50 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 2 | 2 | CC1 | 12@13 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 3 | 2 | CC2 | 50@50 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | NOTE 18 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 4 | 2 | CC1 | 16@57 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 5 | 2 | CC2 | 100@0 |  |  |  | ALL RBs |  | Max |  |  | 2 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | High |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 66 | Mid | 100@0 |  |  |  | ALL RBs |  | - |  |  | 71 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 7 | 71 | Low | 25@0 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 25 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 8 | 71 | Low | 25@12 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 9 | 71 | Low | 20@25 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 75 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 10 | 71 | Low | 20@40 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 11 | 71 | Mid | 100@0 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 2 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-4A-4A-5A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 50@50 |  |  |  | ALL RBs |  | - |  |  | 4 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 4 |  |  |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  | 20 |  |  | 4 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  | 5 |  |  | 4 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  |  |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 25 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 4 | 4 | CC2 | 50@0 |  |  |  | ALL RBs |  | Max |  |  | 4 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | Low |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 5 | 4 | CC2 | 50@0 |  |  |  | ALL RBs |  | Max |  |  | 4 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 5 |  |  |  |  |  | High |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 6 | 5 | Low | 25@25 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 4 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 7 | 5 | High | 25@25 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 4 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 8 | 5 | High | 25@25 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 4 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | 20 |  |  |  |  | 4 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-4A-7A-7A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 50@50 |  |  |  | ALL RBs |  | - |  |  | 4 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 7 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | 30 |  |  |  |  | 7 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 2 | 4 | Mid | 100@0 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 7 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | 30 |  |  |  |  | 7 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 3 | 7 | CC2 | 32@68 |  |  |  | ALL RBs |  | 15 |  |  | 7 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 4 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 4 | 7 | CC2 | 45@55 |  |  |  | ALL RBs |  | 15 |  |  | 7 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 4 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 5 | 7 | CC2 | 32@68 |  |  |  | ALL RBs |  | 30 |  |  | 7 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 4 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 7 | CC2 | 45@55 |  |  |  | ALL RBs |  | 30 |  |  | 7 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 4 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-4A-7A-7A with UL CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | 50@0 |  |  |  | ALL RBs |  | - |  |  | 4 |  |  | High |  |  |  | 25@0 |  |  |  | ALL RBs |  |  |  | - |  |  |  | 7 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | 30 |  |  |  |  | 7 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 2 | 4 | Mid16 | 25@0 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid16 |  |  |  | 25@0 |  |  |  | ALL RBs |  |  |  | - |  |  |  | 7 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | 30 |  |  |  |  | 7 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2A-66A-66A-71A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | High | 50@50 |  |  |  | ALL RBs |  | - |  |  | 71 |  |  | NOTE 18 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 2 | 2 | Low | 100@0 |  |  |  | ALL RBs |  | - |  |  | 71 |  |  | High |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 3 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  | Max |  |  | 66 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 4 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  | Max |  |  | 66 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 5 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  | Max |  |  | 66 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 6 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  | Max |  |  | 66 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 71 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 7 | 71 | Mid | 50@0 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 8 | 71 | Low | 25@0 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | High |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 9 | 71 | Low | 25@12 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | High |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 10 | 71 | Low | 20@25 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | High |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 75 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 11 | 71 | Low | 20@40 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | High |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 12 | 71 | Mid | 100@0 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  |  | Max |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Default Test Settings for a CA_4A-4A-5A-30A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 4 | CC2 | 50@0 |  |  |  | ALL RBs |  |  | Low |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Low |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 4 | CC2 | 50@0 |  |  |  | ALL RBs |  |  | Low |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | High |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 5 | 5 | Mid | 50@0 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 6 | 5 | Low | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Low |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 7 | 5 | High | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Low |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 8 | 30 | Mid | 50@0 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 9 | 30 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Low |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 5 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| Test Settings for a CA_4A-4A-12A-30A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Low |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 12 | Mid | 50@0 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 5 | 12 | Mid | 20@30 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Low |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 6 | 30 | Mid | 50@0 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 12 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 7 | 30 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Low |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 12 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| Test Settings for a CA_4A-4A-29A-30A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 29 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 29 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 4 | CC2 | 50@0 |  |  |  | ALL RBs |  |  | Low |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 29 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 4 | CC2 | 100@0 |  |  |  | ALL RBs |  |  | Low |  |  | 4 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 29 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 5 | 30 | Mid | 50@0 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 29 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 6 | 30 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 4 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Low |  |  |  | 4 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 29 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| Test Settings for a CA_5A-30A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 66 | CC1 | 17@57 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 5 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 5 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 30 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 4 | 30 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 5 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | QPSK |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for a CA_12A-30A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 2 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 3 | 66 | CC1 | 50@0 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 30 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 12 | Mid | 20@30 |  |  |  | ALL RBs |  |  | - |  |  | 30 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 5 | 12 | Low | 20@5 |  |  |  | ALL RBs |  |  | - |  |  | 30 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 25 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 12 | Mid | 20@5 |  |  |  | ALL RBs |  |  | - |  |  | 30 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 7 | 12 | High | 20@30 |  |  |  | ALL RBs |  |  | - |  |  | 30 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 8 | 30 | Mid | 25@25 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | Mid |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  | N/A |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  |  |  | CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for a CA_2A-12A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 50@50 |  |  |  | ALL RBs |  | - |  |  | 12 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 50 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 2 | 12 | Low | 20@5 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 65 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 3 | 12 | Mid | 20@5 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 60 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| 4 | 12 | High | 20@30 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 55 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 75 |
| 5 | 12 | High | 20@30 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 50 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 12 | Low | 20@5 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 65 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 7 | 12 | Mid | 20@5 |  |  |  | ALL RBs |  | - |  |  | 2 |  |  | Mid |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | 50 |  |  |  |  |  | 66 |  |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for a CA_29A-66A-66A-70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS |  |  |  | ALL RBs |  | Max |  |  | 66 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 29 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 70 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 75 |
| 2 | 66 | CC1 | REFSENS |  |  |  | ALL RBs |  | Max |  |  | 66 |  |  | CC2 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 29 |  |  |  | Mid |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 70 |  |  |  |  |  | Mid |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 75 |
| 3 | 70 | Mid | REFSENS |  |  |  | ALL RBs |  | - |  |  | 66 |  |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | 66 |  |  |  | CC2 |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 29 |  |  |  |  |  | High |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 75 |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |
| Test Settings for a CA_66A-66A-70A-71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1/Low | 100@0 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  |  |  | CC2/High |  |  | N/A |  |  | ALL RBs |  |  |  |  | - |  | 70 |  |  |  |  | Mid |  |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  | 71 |  |  |  |  |  | Mid |  |  | N/A |  |  | ALL RBs |  |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 75 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |  |
| 2 | 66 | CC1/Low | 100@0 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  |  |  | CC2/High |  |  | N/A |  |  | ALL RBs |  |  |  |  | - |  | 70 |  |  |  |  | Mid |  |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  | 71 |  |  |  |  |  | Mid |  |  | N/A |  |  | ALL RBs |  |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |  |
| 3 | 70 | Mid | 75@0 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  |  |  | CC1/Low |  |  | 100@0 |  |  | ALL RBs |  |  |  |  | - |  | 66 |  |  |  |  | CC2/High |  |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  | 71 |  |  |  |  |  | Mid |  |  | N/A |  |  | ALL RBs |  |
|  | QPSK | QPSK |  |  |  |  | 75 |  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  | QPSK |  |  |  |  |  | 50 |  |
| 4 | 71 | Low | 16@12 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  |  |  | CC1/Low |  |  | 100@0 |  |  | ALL RBs |  |  |  |  | - |  | 66 |  |  |  |  | CC2/High |  |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  | 70 |  |  |  |  |  | Low |  |  | N/A |  |  | ALL RBs |  |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  | QPSK |  |  |  |  |  | 75 |  |
| 5 | 71 | High | 50@0 |  |  |  | ALL RBs |  | - |  |  | 66 |  |  |  |  | CC1/Low |  |  | 100@0 |  |  | ALL RBs |  |  |  |  | - |  | 66 |  |  |  |  | CC2/High |  |  | N/A |  |  | ALL RBs |  |  |  | - |  |  |  | 70 |  |  |  |  |  | Mid |  |  | N/A |  |  | ALL RBs |  |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  | QPSK |  |  |  |  |  | 75 |  |
| Default Test Settings for a CA_XA-XD/XD-XA Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC1 | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | X |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | X |  | CC3 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | X |  |  |  |  | CC4 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | 64-QAM |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| 2 | X | CC4 | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | X |  |  | CC3 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | X |  | CC2 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | X |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | 64-QAM |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| Default Test Settings for CA_XC-XC Configurationss (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC1 | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | X |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  | X |  | CC3 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | X |  |  |  |  | CC4 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | 64-QAM |  |  |  |  | NRB_agg |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | NRB_agg |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | NRB_agg |
| Default Test Settings for a CA_XA-XA-YB/XA-XA-YC/XC-YC Configuration (Intra-band non-contiguous/contiguous + Inter-band Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_2A-2A-12B Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 50@50 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid/CC1 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 12 |  |  |  |  | Mid/ CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 2 | 2 | CC2 | 50@50 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid/CC1 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 12 |  |  |  |  | Mid/ CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 3 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid/CC1 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 12 |  |  |  |  | Mid/ CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 4 | 2 | CC2 | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  | CC1 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 12 |  | Mid/CC1 |  |  | N/A |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 12 |  |  |  |  | Mid/ CC2 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 50 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 25 |
| 5 | 12 | Low/CC1 | 20@5 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | Low/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | Max |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 2 |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 6 | 12 | Low/CC1 | 20@30 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | Low/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | Max |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 2 |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 7 | 12 | Mid/CC1 | 20@30 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | Mid/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | Max |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 2 |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 8 | 12 | High/CC1 | 20@5 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | High/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | Max |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 2 |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| 9 | 12 | High/CC1 | 20@30 |  |  |  | ALL RBs |  |  | - |  |  | 12 |  |  | High/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | CC2 |  |  | Max |  | ALL RBs |  |  |  |  | - |  |  |  |  |  | 2 |  |  |  |  | CC1 |  | N/A |  |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 50 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_2C-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low/CC1 | 100@0 |  |  |  | ALL RBs |  |  | - |  |  | 2 |  |  | Low/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| 2 | 2 | Low/CC1 | 25@0 |  |  |  | ALL RBs |  |  | - |  |  | 2 |  |  | Low/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| 3 | 2 | High/CC1 | 100@0 |  |  |  | ALL RBs |  |  | - |  |  | 2 |  |  | High/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| 4 | 2 | High/CC1 | 25@0 |  |  |  | ALL RBs |  |  | - |  |  | 2 |  |  | High/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 25 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| 5 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | Mid/CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 2 |  |  |  | Mid/CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| 6 | 66 | CC1 | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 2 |  | Mid/CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 2 |  |  |  | Mid/CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| Test Settings for a CA_66A-66A-70C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 70 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 70 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |
| 2 | 66 | CC1 | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | 66 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 70 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  |  |  | 70 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |
| 3 | 70 | CC1 | REFSENS |  |  |  | ALL RBs |  |  |  |  |  | 70 |  |  | CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 66 |  |  |  | CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 75 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| Test Settings for CA_66C-70C Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Mid/CC1 | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | 66 |  |  | Mid/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 70 |  | Mid/CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 70 |  |  |  | Mid/CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |
| 2 | 66 | High/CC1 | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | 66 |  |  | High/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 70 |  | Mid/CC1 |  |  |  | N/A |  |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 70 |  |  |  | Mid/CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 50 |
| 3 | 70 | Mid/CC1 | REFSENS |  |  |  | ALL RBs |  |  | - |  |  | 70 |  |  | Mid/CC2 |  |  |  |  | N/A |  |  |  | ALL RBs |  |  |  | - |  |  |  | 66 |  | High/CC1 |  |  |  | REFSENS |  |  |  | ALL RBs |  |  |  |  | - |  |  |  |  | 66 |  |  |  | High/CC2 |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  | 75 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  | 100 |
| Default Test Settings for CA_XA-XA-YA-YA Configurations (Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 |  | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | X |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | Y |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | Y |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | Highest NRB_agg |  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | Highest NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | Highest NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | Highest NRB_agg |
| 2 | Y | CC2 |  | REFSENS |  |  |  | ALL RBs |  |  | Max |  |  | Y |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | X |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | X |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | Highest NRB_agg |  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | Highest NRB_agg |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | Highest NRB_agg |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | Highest NRB_agg |
| Test Settings for CA_2A-2A-4A-4A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 114 | 2 | CC2 |  | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 4 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 4 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| 215 | 2 | CC2 |  | 16@39 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 4 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 4 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| 3 | 2 | CC2 |  | 16@57 |  |  |  | ALL RBs |  |  | Max |  |  | 2 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 4 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 4 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| 4 | 4 | CC2 |  | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 2 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 2 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| 5 | 4 | CC2 |  | 100@0 |  |  |  | ALL RBs |  |  | Max |  |  | 4 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 2 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | Max |  |  |  |  |  | 2 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| Test Settings for CA_3A-3A-7A-7A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | CC2 |  |  | 16@50 |  |  | ALL RBs |  |  | 50 |  |  | 3 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 7 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 40 |  |  |  |  |  | 7 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 2 | 3 | CC2 |  |  | 32@68 |  |  | ALL RBs |  |  | 15 |  |  | 3 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 7 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 40 |  |  |  |  |  | 7 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 313 | 3 | CC2 |  |  | 16@50 |  |  | ALL RBs |  |  | 35 |  |  | 3 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 7 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 40 |  |  |  |  |  | 7 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 411 | 3 | CC2 |  |  | 16@50 |  |  | ALL RBs |  |  | 45 |  |  | 3 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 7 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 40 |  |  |  |  |  | 7 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 511 | 3 | CC2 |  |  | 32@68 |  |  | ALL RBs |  |  | 10 |  |  | 3 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 7 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 40 |  |  |  |  |  | 7 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 6 | 7 | CC2 |  |  | 32@68 |  |  | ALL RBs |  |  | 45 |  |  | 7 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 3 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 10 |  |  |  |  |  | 3 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 7 | 7 | CC2 |  |  | 45@55 |  |  | ALL RBs |  |  | 25 |  |  | 7 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 3 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 10 |  |  |  |  |  | 3 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 25 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 811 | 7 | CC2 |  |  | 32@68 |  |  | ALL RBs |  |  | 40 |  |  | 7 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 3 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 10 |  |  |  |  |  | 3 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 911 | 7 | CC2 |  |  | 45@55 |  |  | ALL RBs |  |  | 20 |  |  | 7 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 3 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 10 |  |  |  |  |  | 3 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 50 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 50 |
| 1013 | 7 | CC2 |  |  | 32@68 |  |  | ALL RBs |  |  | 30 |  |  | 7 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 3 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 35 |  |  |  |  |  | 3 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| 1113 | 7 | CC2 |  |  | 45@55 |  |  | ALL RBs |  |  | 15 |  |  | 7 |  |  |  | CC1 |  |  |  | N/A |  | ALL RBs |  | - |  |  |  |  |  | 3 |  | CC2 |  | N/A |  |  |  |  |  | ALL RBs |  |  |  | 35 |  |  |  |  |  | 3 |  | CC1 |  |  |  |  | N/A |  |  | ALL RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 100 |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |  |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X, Y, Z correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-4A-12A,X=2,Y=4,Z=12. Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A-41A,X=41. Note 7: Intra-band non-contiguous + Intra-band non-contiguous: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-4A-4A,X=2,Y=4. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: Applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. (This note is not applicable for any existing test point and only reserved for future use) Note 11: Not applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. Note 12: Applicable if only Bandwidth Combination Set 1 is supported by the UE for the CA Configuration. (This note is not applicable for any existing test point and only reserved for future use) Note 13: Not applicable if only Bandwidth Combination Set 1 is supported by the UE for the CA Configuration. Note 14: Not applicable if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 15: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 16: Band 2: fUL = 1868.3MHz (NUL = 18783), fDL = 1948.3MHz (NDL = 783). Band 4: fUL = 1735MHz (NUL = 20200), fDL = 2135MHz (NDL = 2200). Note 17: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 18: Test point that fulfils criteria of Note 16 in Table 7.3A.0-0a. Band 71: fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.55 if the UE supports 2 Rx antennas or Figure A.80 if the UE supports 4 Rx antennas in at least one band within the CA configuration..

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.3A.9.4.1-1 through Table7.3A.9.4.1-6 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.3A.9.4.3.

##### 7.3A.9.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.3A.9.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.3A.9.4.1-1 through Table7.3A.9.4.1-2 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.3A.9.4.1-1 through Table7.3A.9.4.1-2 as appropriate on PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the appropriate REFSENS value defined in Table 7.3A.9.5-1 through Table7.3A.9.5-7. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for duration sufficient to achieve statistical significance according to Annex G.2A.

##### 7.3A.9.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

#### 7.3A.9.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-1: Reference sensitivity QPSK PREFSENS for Intra-band contiguous

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_40E | 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | -1024 | -994 | -97.24 | -964 |  |
| CA_42E | 40 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.24 | -97.24 | -95.44 | -94.24 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3A.9.5-2: FFS

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-3 (originated from single carrier Tables 7.3.5-1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-3: Reference sensitivity QPSK PREFSENS for inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration |  |  |  | EUTRA band |  |  | 1.4 MHz (dBm) |  |  |  | 3 MHz (dBm) |  |  |  |  | 5 MHz (dBm) |  |  |  |  |  | 10 MHz (dBm) |  |  |  |  |  | 15 MHz (dBm) |  |  |  |  | 20 MHz (dBm) |  |  |  |  | Duplex mode |
| CA_1A-3A-7A-8A |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 7 |  |  | - |  |  |  | - |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -97.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 711,12 |  |  | - |  |  |  | - |  |  |  |  | -87.3 |  |  |  |  |  | -86.7 |  |  |  |  |  | -86.3 |  |  |  |  | -86.0 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -90.04 |  |  |  |  |  | -89.44 |  |  |  |  |  | -89.04 |  |  |  |  | -88.74 |  |  |  |  |  |
|  |  |  |  | 8 |  |  | - |  |  |  | - |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  | 811,12 |  |  | - |  |  |  | - |  |  |  |  | -96.1 |  |  |  |  |  | -93.1 |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  |  |
| CA_1A-3A-7A-32A8 |  |  |  | 1 |  |  | - |  |  |  |  | - |  |  |  |  | -99.3 |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  |  | FDD |  |
|  |  |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -102.04 |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  |  | - |  |  |  |  | -93.3 |  |  |  | -90.8 |  |  |  |  |  | -89.3 |  |  |  |  | -88.3 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -96.04 |  |  |  | -93.54 |  |  |  |  |  | -92.04 |  |  |  |  | -91.04 |  |  |  |  |  |  |  |
|  |  |  |  | 7 |  |  | - |  |  |  |  | - |  |  |  |  | -97.3 |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -100.04 |  |  |  | -97.04 |  |  |  |  |  | -97.24 |  |  |  |  | -93.04 |  |  |  |  |  |  |  |
|  |  |  |  | 32 |  |  | - |  |  |  |  | - |  |  |  |  | -99.3 |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  |  |  |  |
| CA_1A-3A-7A-32A9 |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 7 |  |  | - |  |  |  | - |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -97.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 32 |  |  | - |  |  |  | - |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  |  |
| CA_1A-3A-8A-40A |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  | FDD |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |
|  |  |  | 113 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -91.0 |  |  |  |  |  | [-88.8] |  |  |  |  |  | [-87.2] |  |  |  |  | [-86.2] |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -93.74 |  |  |  |  |  | -91.54 |  |  |  |  |  | -89.94 |  |  |  |  | -88.94 |  |  |  |  |
|  |  |  | 38 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.020 |  |  |  |  |  | -96.020 |  |  |  |  |  | -94.220 |  |  |  |  | -93.020 |  |  |  |  |
|  |  |  | 39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -93.3 |  |  |  |  |  | -90.8 |  |  |  |  |  | -89.3 |  |  |  |  | -88.3 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -96.04 |  |  |  |  |  | -93.54 |  |  |  |  |  | -92.04 |  |  |  |  | -91.04 |  |  |  |  |
|  |  |  | 313 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |
|  |  |  | 8 |  |  |  |  | - |  |  |  |  | -98.5 |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
|  |  |  | 40 |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  | TDD |  |  |
|  |  |  |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |
|  |  |  | 4014 |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -94.7 |  |  |  |  |  | -92.2 |  |  |  |  |  | -90.6 |  |  |  |  | -89.5 |  |  |  |  |
|  |  |  |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  | -97.44 |  |  |  |  |  | -94.94 |  |  |  |  |  | -93.34 |  |  |  |  | -92.24 |  |  |  |  |
|  |  |  | 4015 |  |  |  |  | - |  |  |  |  | - |  |  |  |  | [-92.7] |  |  |  |  |  | -91.2 |  |  |  |  |  | -89.7 |  |  |  |  | -88.7 |  |  |  |  |
|  |  |  |  |  |  |  |  | - |  |  |  |  | - |  |  |  |  | [-95.4]4 |  |  |  |  |  | -93.94 |  |  |  |  |  | -92.44 |  |  |  |  | -91.44 |  |  |  |  |
| CA_1A-3A-18A-42A |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  | FDD |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 38 |  |  | - |  |  |  | - |  |  |  |  |  | -93.3 |  |  |  |  |  | -90.8 |  |  |  |  |  | -89.3 |  |  |  |  | -88.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -96.04 |  |  |  |  |  | -93.54 |  |  |  |  |  | -92.04 |  |  |  |  | -91.04 |  |  |  |  |  |
|  |  |  |  | 39 |  |  | - |  |  |  | - |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 18 |  |  | - |  |  |  | - |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  | 42 |  |  | - |  |  |  | - |  |  |  |  |  | -98.0 |  |  |  |  |  | -95.0 |  |  |  |  |  | -93.2 |  |  |  |  | -92.0 |  |  |  | TDD |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -100.24 |  |  |  |  |  | -97.24 |  |  |  |  |  | -95.44 |  |  |  |  | -94.24 |  |  |  |  |  |
|  |  |  |  | 425,6 |  |  | - |  |  |  | - |  |  |  |  |  | -71.2 |  |  |  |  |  | -71.2 |  |  |  |  |  | -71.2 |  |  |  |  | -71.2 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -73.44 |  |  |  |  |  | -73.44 |  |  |  |  |  | -73.44 |  |  |  |  | -73.44 |  |  |  |  |  |
| CA_1A-3A-19A-42A5,6 |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 19 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | -71.2 |  |  |  |  |  | -71.2 |  |  |  |  |  | -71.2 |  |  |  |  | -71.2 |  |  |  |  | TDD |
| CA_1A-3A-19A-42A7 |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 19 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | -96.6 |  |  |  |  |  | -94.2 |  |  |  |  |  | -92.7 |  |  |  |  | -92.0 |  |  |  |  | TDD |
| CA_1A-3A-19A-42A |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  | FDD |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 38 |  |  | - |  |  |  | - |  |  |  |  |  | -93.3 |  |  |  |  |  | -90.8 |  |  |  |  |  | -89.3 |  |  |  |  | -88.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -96.04 |  |  |  |  |  | -93.54 |  |  |  |  |  | -92.04 |  |  |  |  | -91.04 |  |  |  |  |  |
|  |  |  |  | 39 |  |  | - |  |  |  | - |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |
|  |  |  |  | 19 |  |  | - |  |  |  | - |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 42 |  |  | - |  |  |  | - |  |  |  |  |  | -98.0 |  |  |  |  |  | -95.0 |  |  |  |  |  | -93.2 |  |  |  |  | -92.0 |  |  |  | TDD |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  |  | -100.24 |  |  |  |  |  | -97.24 |  |  |  |  |  | -95.44 |  |  |  |  | -94.24 |  |  |  |  |  |
| CA_1A-3A-21A-42A6 |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.1 |  |  |  |  |  | -96.1 |  |  |  |  |  | -94.3 |  |  |  |  | -93.1 |  |  |  |  | FDD |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.84 |  |  |  |  |  | -98.84 |  |  |  |  |  | -97.04 |  |  |  |  | -95.84 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -96.0 |  |  |  |  |  | -93.0 |  |  |  |  |  | -91.2 |  |  |  |  | -90.0 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -98.74 |  |  |  |  |  | -95.74 |  |  |  |  |  | -93.94 |  |  |  |  | -92.74 |  |  |  |  |  |
|  |  |  |  | 21 |  |  | - |  |  |  | - |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  | 42 |  |  | - |  |  |  | - |  |  |  |  | -70.7 |  |  |  |  |  | -70.7 |  |  |  |  |  | -70.7 |  |  |  |  | -70.7 |  |  |  |  | TDD |
| CA_1A-3A-21A-42A7 |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.1 |  |  |  |  |  | -96.1 |  |  |  |  |  | -94.3 |  |  |  |  | -93.1 |  |  |  |  | FDD |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.84 |  |  |  |  |  | -98.84 |  |  |  |  |  | -97.04 |  |  |  |  | -95.84 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -96.0 |  |  |  |  |  | -93.0 |  |  |  |  |  | -91.2 |  |  |  |  | -90.0 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -98.74 |  |  |  |  |  | -95.74 |  |  |  |  |  | -93.94 |  |  |  |  | -92.74 |  |  |  |  |  |
|  |  |  |  | 21 |  |  | - |  |  |  | - |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  | 42 |  |  | - |  |  |  | - |  |  |  |  | -96.1 |  |  |  |  |  | -93.7 |  |  |  |  |  | -92.2 |  |  |  |  | -91.5 |  |  |  |  | TDD |
| CA_1A-3A-21A-42A8 |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.1 |  |  |  |  |  | -96.1 |  |  |  |  |  | -94.3 |  |  |  |  | -93.1 |  |  |  |  | FDD |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.84 |  |  |  |  |  | -98.84 |  |  |  |  |  | -97.04 |  |  |  |  | -95.84 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -93.0 |  |  |  |  |  | -90.5 |  |  |  |  |  | -89.0 |  |  |  |  | -88.0 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -95.74 |  |  |  |  |  | -93.24 |  |  |  |  |  | -91.74 |  |  |  |  | -90.74 |  |  |  |  |  |
|  |  |  |  | 21 |  |  | - |  |  |  | - |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.54 |  |  |  |  |  | -98.54 |  |  |  |  |  | -96.74 |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  | 42 |  |  | - |  |  |  | - |  |  |  |  | -97.5 |  |  |  |  |  | -94.5 |  |  |  |  |  | -92.7 |  |  |  |  | -91.5 |  |  |  |  | TDD |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -99.74 |  |  |  |  |  | -96.74 |  |  |  |  |  | -94.94 |  |  |  |  | -93.74 |  |  |  |  |  |  |
| CA_1A-3A-21A-42A9 |  |  |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.1 |  |  |  |  |  | -96.1 |  |  |  |  |  | -94.3 |  |  |  |  | -93.1 |  |  |  |  | FDD |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.84 |  |  |  |  |  | -98.84 |  |  |  |  |  | -97.04 |  |  |  |  | -95.84 |  |  |  |  |  |
|  |  |  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -96.0 |  |  |  |  |  | -93.0 |  |  |  |  |  | -91.2 |  |  |  |  | -90.0 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -98.74 |  |  |  |  |  | -95.74 |  |  |  |  |  | -93.94 |  |  |  |  | -92.74 |  |  |  |  |  |
|  |  |  |  | 21 |  |  | - |  |  |  | - |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | - |  |  |  |  |  |
|  |  |  |  | 42 |  |  | - |  |  |  | - |  |  |  |  | -97.5 |  |  |  |  |  | -94.5 |  |  |  |  |  | -92.7 |  |  |  |  | -91.5 |  |  |  |  | TDD |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -99.74 |  |  |  |  |  | -96.74 |  |  |  |  |  | -94.94 |  |  |  |  | -93.74 |  |  |  |  |  |
| CA_1A-3A-41A-42A5,6 |  | 1 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.04 |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | TDD |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | -1004 |  |  |  |  | -974 |  |  |  |  |  | -95.24 |  |  |  |  | -944 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | -71.2 |  |  |  |  | -71.2 |  |  |  |  |  | -71.2 |  |  |  |  | -71.2 |  |  |  |  |  |  |  |  |
| CA_1A-3A-41A-42A7 |  | 1 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  | -90.3 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.04 |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  | -93.04 |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | TDD |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | -1004 |  |  |  |  | -974 |  |  |  |  |  | -95.24 |  |  |  |  | -944 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | -96.6 |  |  |  |  | -94.2 |  |  |  |  |  | -92.7 |  |  |  |  | -92.0 |  |  |  |  |  |  |  |  |
| CA_1A-3A-41A-42A |  | 1 |  |  | - |  |  |  | - |  |  |  |  | -99.3 |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  |  | -93.3 |  |  |  |  | FDD |  |  |  |
|  |  |  |  |  | - |  |  |  | - |  |  |  |  | -102.04 |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  |  | -96.04 |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  | - |  |  |  | - |  |  |  |  | -96.3 |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  |  | -90.3 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | - |  |  |  | - |  |  |  |  | -99.04 |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  |  | -93.04 |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  | - |  |  |  | - |  |  |  |  | -93.3 |  |  |  |  | -90.8 |  |  |  |  |  | -89.3 |  |  |  |  |  | -88.3 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | - |  |  |  | - |  |  |  |  | -96.04 |  |  |  |  | -93.54 |  |  |  |  |  | -92.04 |  |  |  |  |  | -91.04 |  |  |  |  |  |  |  |  |
|  |  | 39 |  |  | - |  |  |  | - |  |  |  |  | -96.3 |  |  |  |  | -93.3 |  |  |  |  |  | -91.5 |  |  |  |  |  | -90.3 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | - |  |  |  | - |  |  |  |  | -99.04 |  |  |  |  | -96.04 |  |  |  |  |  | -94.24 |  |  |  |  |  | -93.04 |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  | - |  |  |  | - |  |  |  |  | -97.3 |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  |  | -91.3 |  |  |  |  | TDD |  |  |  |
|  |  |  |  |  | - |  |  |  | - |  |  |  |  | -1004 |  |  |  |  | -974 |  |  |  |  |  | -95.24 |  |  |  |  |  | -944 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  | - |  |  |  | - |  |  |  |  | -98.0 |  |  |  |  | -95.0 |  |  |  |  |  | -93.2 |  |  |  |  |  | -92.0 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | - |  |  |  | - |  |  |  |  | -100.24 |  |  |  |  | -97.24 |  |  |  |  |  | -95.44 |  |  |  |  |  | -94.24 |  |  |  |  |  |  |  |  |
| CA_1A-19A-21A-42A |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 19 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 21 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | -98.0 |  |  |  |  |  | -95.0 |  |  |  |  |  | -93.2 |  |  |  |  | -92.0 |  |  |  |  | TDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.24 |  |  |  |  |  | -97.24 |  |  |  |  |  | -95.44 |  |  |  |  | -94.24 |  |  |  |  |  |
| CA_2A-4A-5A-12A11,12 |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.0 |  |  |  |  |  | -94.0 |  |  |  |  |  | -92.2 |  |  |  |  | -91.0 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.74 |  |  |  |  |  | -96.74 |  |  |  |  |  | -94.94 |  |  |  |  | -93.74 |  |  |  |  |  |
|  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | -89.3 |  |  |  |  |  | -88.8 |  |  |  |  |  | -88.3 |  |  |  |  | -87.5 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -92.04 |  |  |  |  |  | -91.54 |  |  |  |  |  | -91.04 |  |  |  |  | -90.24 |  |  |  |  |  |
|  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  | -96.8 |  |  |  |  |  | -93.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12 |  |  |  |  |  |  |  |  |  |  |  | -95.8 |  |  |  |  |  | -92.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-5A-29A |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.0 |  |  |  |  |  | -94.0 |  |  |  |  |  | -92.2 |  |  |  |  | -91.0 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.74 |  |  |  |  |  | -96.74 |  |  |  |  |  | -94.94 |  |  |  |  | -93.74 |  |  |  |  |  |
|  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | -99.0 |  |  |  |  |  | -96.0 |  |  |  |  |  | -94.2 |  |  |  |  | -93.0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.74 |  |  |  |  |  | -98.74 |  |  |  |  |  | -96.94 |  |  |  |  | -95.74 |  |  |  |  |  |
|  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 29 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-5A-30A |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -95.24 |  |  |  |  | -94.04 |  |  |  |  |  |
|  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.04 |  |  |  |  |  | -98.04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12A-30A10 |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -95.24 |  |  |  |  | -94.04 |  |  |  |  |  |
|  |  |  |  | 210 |  |  |  |  |  |  |  |  |  |  |  | -96.9 |  |  |  |  |  | -93.9 |  |  |  |  |  | -92.1 |  |  |  |  | -90.9 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.64 |  |  |  |  |  | -96.64 |  |  |  |  |  | -94.84 |  |  |  |  | -93.64 |  |  |  |  |  |
|  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.5 |  |  |  |  | -93.3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -97.24 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 410 |  |  |  |  |  |  |  |  |  |  |  | -89.3 |  |  |  |  |  | -88.8 |  |  |  |  |  | -88.3 |  |  |  |  | -87.8 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -924,10 |  |  |  |  |  | -91.54,10 |  |  |  |  |  | -91.04,10 |  |  |  |  | -90.54 |  |  |  |  |  |
|  |  |  |  | 12 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1210 |  |  |  |  |  |  |  |  |  |  |  | -95.8 |  |  |  |  |  | -92.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.04 |  |  |  |  |  | -98.04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3010 |  |  |  |  |  |  |  |  |  |  |  | -97.8 |  |  |  |  |  | -94.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.54 |  |  |  |  |  | -97.54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-29A-30A |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -96.9 |  |  |  |  |  | -93.9 |  |  |  |  |  | -92.1 |  |  |  |  | -90.9 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -99.64 |  |  |  |  |  | -96.64 |  |  |  |  |  | -94.84 |  |  |  |  | -93.64 |  |  |  |  |  |
|  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | -98.9 |  |  |  |  |  | -95.9 |  |  |  |  |  | -94.1 |  |  |  |  | -92.9 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.64 |  |  |  |  |  | -98.64 |  |  |  |  |  | -96.84 |  |  |  |  | -95.64 |  |  |  |  |  |
|  |  |  |  | 29 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  | -97.8 |  |  |  |  |  | -94.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.54 |  |  |  |  |  | -97.54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-30A-66A |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -95.24 |  |  |  |  | -94.04 |  |  |  |  |  |
|  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.04 |  |  |  |  |  | -98.04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 66 |  |  |  |  |  |  |  |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | -92.8 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.54 |  |  |  |  |  | -98.54 |  |  |  |  |  | -96.74 |  |  |  |  | -95.54 |  |  |  |  |  |
| CA_2A-12A-30A-66A |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -95.24 |  |  |  |  | -94.04 |  |  |  |  |  |
|  |  |  |  | 12 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.04 |  |  |  |  |  | -98.04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 66 |  |  |  |  |  |  |  |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | -92.8 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.54 |  |  |  |  |  | -98.54 |  |  |  |  |  | -96.74 |  |  |  |  | -95.54 |  |  |  |  |  |
| CA_2A-14A-30A-66A |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | -97.3 |  |  |  |  |  | -94.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -100.04 |  |  |  |  |  | -97.04 |  |  |  |  |  | -95.24 |  |  |  |  | -94.04 |  |  |  |  |  |
|  |  |  |  | 14 |  |  |  |  |  |  |  |  |  |  |  | -96.3 |  |  |  |  |  | -93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.04 |  |  |  |  |  | -98.04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 66 |  |  | - |  |  |  | - |  |  |  |  | -98.8 |  |  |  |  |  | -95.8 |  |  |  |  |  | -94.0 |  |  |  |  | -92.8 |  |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  | - |  |  |  |  | -101.520 |  |  |  |  |  | -98.520 |  |  |  |  |  | -96.720 |  |  |  |  | -95.520 |  |  |  |  |  |
| CA_3A-7A-20A-32A |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  | -93.5 |  |  |  |  | -92.3 |  |  |  |  | FDD |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -101.04 |  |  |  |  |  | -98.04 |  |  |  |  |  | -96.24 |  |  |  |  | -95.04 |  |  |  |  |  |
|  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | -99.3 |  |  |  |  |  | -96.3 |  |  |  |  |  | -94.2 |  |  |  |  | -93.3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -102.04 |  |  |  |  |  | -99.04 |  |  |  |  |  | -96.94 |  |  |  |  | -96.04 |  |  |  |  |  |
|  |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  | -98.3 |  |  |  |  |  | -95.3 |  |  |  |  |  | -92.5 |  |  |  |  | -91.3 |  |  |  |  |  |
|  |  |  |  | 32 |  |  |  |  |  |  |  |  |  |  |  | -99.5 |  |  |  |  |  | -96.5 |  |  |  |  |  | -94.7 |  |  |  |  | -93.5 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_3A-19A-42A, CA_3A-19A-42C, CA_1A-3A-19A-42A and CA_1A-3A-19A-42C. NOTE 6: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 8: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 10: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 and 6 in Table 7.3A.0-0a. NOTE 11: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 12: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 13: These requirements apply when the uplink is active in Band 40. NOTE 14: These requirements apply when the uplink is active in Band 3. NOTE 15: These requirements apply when the uplink is active in Band 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C shall be applied for applicable bands.

Unless otherwise specified, in case the UE supports more than one of the above 4DL inter-band carrier aggregation configurations and an E-UTRA operating band belongs to more than one 4DL inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 4DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 4DL carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional 4DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 4DL CA configurations

The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration.

The reference receive sensitivity (REFSENS) requirement for inter-band specified in Table 7.3A.9.5-3 shall be met for an uplink CA configurations than or equal to that specified in Table 7.3.5-2.

NOTE:	Table 7.3A.9.5-4 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3A.9.5-4: Void

For combinations of intra-band contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-5. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-5: Reference sensitivity QPSK PREFSENS for intra-band contiguous + inter-band/(inter-band) intra-band contiguous

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-41C5 | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  |  | - | - | -96.04 | -93.54 | -92.04 | -91.04 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -1004 | -974 | -95.24 | -944 |  |
| CA_1A-3A-41C6 | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  |  |  | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 41 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  |  |  | -1004 | -974 | -95.24 | -944 |  |
| CA_1A-3A-42C5 | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  |  | - | - | -96.04 | -93.54 | -92.04 | -91.04 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_1A-3A-42C6 | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  |  |  | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_1A-3A-42C7,8 | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 42 | - | - | -71.2 | -71.2 | -71.2 | -71.2 | TDD |
|  |  | - | - | -73.44 | -73.44 | -73.44 | -73.44 |  |
| CA_1A-3A-42C9 | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 42 | - | - | -96.6 | -94.2 | -92.7 | -92.0 | TDD |
|  |  | - | - | -98.84 | -96.44 | -94.94 | -94.24 |  |
| CA_1A-3C-8A5 | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.04 | -99.04 | -97.24 | -96.04 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  |  | - | - | -96.04 | -93.54 | -92.04 | -91.04 |  |
|  | 8 | - | -98.5 | -96.3 | -93.3 | - | - |  |
| CA_1A-3C-8A6 | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.04 | -99.04 | -97.24 | -96.04 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 8 | - | -98.5 | -96.3 | -93.3 | - | - |  |
| CA_1A-18A-42C | 1 |  |  |  | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  |  | -99.04 | -97.54 | -96.04 |  |
|  | 18 |  |  |  | -96.3 | -94.5 |  | FDD |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_1A-19A-42C | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 19 |  |  | -99.3 | -96.3 | -94.5 |  |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_1A-21A-42C | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.54 | -96.04 |  |
|  | 21 |  |  | -99.3 | -96.3 | -94.5 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.54 |  |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_1A-41A-42C | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |
|  | 41 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 42 |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  |  |  |  | -99.74 | -96.74 | -94.94 | -93.74 |  |
| CA_1A-41C-42A | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |
|  | 41 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 42 |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  |  |  |  | -99.74 | -96.74 | -94.94 | -93.74 |  |
| CA_1A-42D | 1 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_2A-5A-12B | 2 |  |  |  | -94.3 | -92.5 | -91.3 | FDD |
|  |  |  |  |  | -97.04 | -95.54 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |
| CA_2A-5B-30A | 2 |  |  |  | -94.3 | -92.5 | -91.3 | FDD |
|  |  |  |  |  | -97.04 | -95.54 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 30 |  |  | -97.8 | -94.8 |  |  |  |
|  |  |  |  | -100.54 | -97.54 |  |  |  |
| CA_2A-5B-66A | 2 |  |  |  | -94.3 | -92.5 | -91.3 | FDD |
|  |  |  |  |  | -97.04 | -95.54 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2C-12A-30A | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.54 | -94.04 |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
| CA_2C-29A-30A | 2 |  |  | -96.9 | -93.9 | -92.1 | -90.9 | FDD |
|  |  |  |  | -99.64 | -96.64 | -94.84 | -93.64 |  |
|  | 29 |  |  | -96.3 | -93.3 |  |  |  |
|  | 30 |  |  | -97.8 | -94.8 |  |  |  |
|  |  |  |  | -100.54 | -97.54 |  |  |  |
| CA_2A-66C-71A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.04 | -97.04 | -95.54 | -94.04 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 71 | - | - | -96.5 | -93.5 | -91.3 | -86.8 |  |
| CA_2A-66C-71A10,11 | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.04 | -97.04 | -95.54 | -94.04 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 71 | - | - | -69.7 | -69.7 | -69.7 | -69.7 |  |
| CA_2A-66C-71A12 | 2 | - | - | -92.4 | -93.0 | -91.5 | -90.4 | FDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 71 | - | - | -96.5 | - | - | - |  |
| CA_2A-66C-71A13 | 2 | - | - | -95.8 | -93.0 | -91.5 | -90.4 | FDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 71 | - | - | - | -93.5 | -91.3 | -86.8 |  |
| CA_3A-7C-28A | 3 | - | - | - | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | - | -96.04 | -94.24 | -93.04 |  |
|  | 7 | - | - | - | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | - | -97.04 | -95.24 | -94.04 |  |
|  | 28 | - | - | - | -94.8 | -93.0 | -90.3 |  |
| CA_3A-18A-42C | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  |  |  | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 18 |  |  |  | -96.3 | -94.5 |  |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_3A-18A-42C7,8 | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  |  |  | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 18 |  |  |  | -96.3 | -94.5 |  | FDD |
|  | 42 |  |  | -71.2 | -71.2 | -71.2 | -71.2 | TDD |
| CA_3A-19A-42C7,8 | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  |  |  | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 19 |  |  | -99.3 | -96.3 | -94.5 |  |  |
|  | 42 |  |  | -71.2 | -71.2 | -71.2 | -71.2 | TDD |
| CA_3A-19A-42C9 | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  |  |  | [-99.0]4 | [-96.0]4 | [-94.2]4 | [-93.0]4 |  |
|  | 19 |  |  | -99.5 | -96.5 | -94.7 |  |  |
|  | 42 |  |  | -96.6 | -94.2 | -92.7 | -92.0 | TDD |
| CA_3A-21A-42C | 3 |  |  | -96.0 | -93.0 | -91.2 | -90.0 | FDD |
|  |  |  |  | -98.74 | -95.74 | -93.94 | -92.74 |  |
|  | 21 |  |  | -98.8 | -95.8 | -94.0 | - |  |
|  | 42 |  |  | -70.7 | -70.7 | -70.7 | -70.7 | TDD |
| CA_3A-21A-42C | 3 |  |  | -96.0 | -93.0 | -91.2 | -90.0 | FDD |
|  |  |  |  | -98.74 | -95.74 | -93.94 | -92.74 |  |
|  | 21 |  |  | -98.8 | -95.8 | -94.0 | - |  |
|  | 42 |  |  | -96.1 | -93.7 | -92.2 | -91.5 | TDD |
| CA_3A-42D | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  |  |  | -99.04 | -96.04 | -94.24 | -93.04 |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_5B-30A-66A | 5 |  |  | -97.3 | -94.3 |  |  | FDD |
|  | 30 |  |  | -97.8 | -94.8 |  |  |  |
|  |  |  |  | -100.54 | -97.54 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_19A-21A-42C | 19 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
|  | 21 |  |  | -99.3 | -96.3 | -94.5 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.54 |  |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA-28A-40D | 28 |  |  | -94.414 | -92.214 | -90.714 | -89.814 | FDD |
|  |  |  |  | -60.010,11 | -60.010,11 | -60.010,11 | -60.010,11 |  |
|  |  |  |  | -99.5 | -97.8 | -94.8 | -93.0 |  |
|  | 40 |  |  | -96.115 | -93.415 | -91.815 | -89.115 | TDD |
|  |  |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_29A-66A-70C | 29 |  |  | -96.3 | -93.3 |  |  | FDD |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_29A-66C-70A | 29 |  |  | -96.3 | -93.3 |  |  | FDD |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 |  |  |
| CA_39A-41D | 39 |  |  |  | -96.3 | -94.5 | -93.3 | TDD |
|  |  |  |  |  | -99.04 | -97.24 | -96.04 |  |
|  | 41 |  |  |  |  |  | -91.3 |  |
|  |  |  |  |  |  |  | -94.04 |  |
| CA_39C-41C | 39 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |
|  | 41 |  |  |  |  |  | -91.3 |  |
|  |  |  |  |  |  |  | -94.04 |  |
| CA_41C-42C | 41 |  |  |  | -94.3 | -92.5 | -91.3 | TDD |
|  |  |  |  |  | -97.04 | -95.24 | -94.04 |  |
|  | 42 |  |  | -98.0 | -95.0 | -93.2 | -92.0 |  |
|  |  |  |  | -100.24 | -97.24 | -95.44 | -94.24 |  |
| CA_41A-48D | 41 |  |  |  | -94.3 | -92.5 | -91.3 | TDD |
|  | 48 |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_41C-48C | 41 |  |  |  | -94.3 | -92.5 | -91.3 | TDD |
|  | 48 |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_41D-48A | 41 |  |  |  | -94.3 | -92.5 | -91.3 | TDD |
| CA_66C-70C | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_66A-70C-71A | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
|  |  |  |  | -89.38,9 | -88.88,9 | -88.58,9 | -88.38,9 |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.2 | -86.8 |  |
| CA_66C-70A-71A | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | - |  |
|  |  |  |  | -89.38,9 | -88.88,9 | -88.58,9 | - |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.2 | -86.8 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 5 or Band 8 or Band 19 or Band 20 or Band 26 or Band 28 or Band 42, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 5 or Band 8 or Band 19 or Band 20 or Band 26 or Band 28 or Band 42, the requirement applies regardless of channel bandwidth in Band 1. NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42C, CA_1A-3A-42C and CA_3A-19A-42C. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 12: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 14: These requirements apply when active uplink channel is in Band 40. NOTE 15: These requirements apply when active uplink channel is in Band 28. |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-6. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-6: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |  |
| CA_2A-2A-4A-5A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
| CA_2A-2A-5A-12A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |
| CA_2A-2A-4A-71A6,7 | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |  |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  | 71 |  |  | -69.7 | -69.7 | -69.7 | -69.7 |  |  |
| CA_2A-2A-4A-71A9 | 2 |  |  | -92.4 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  | -95.14 | -95.74 | -94.24 | -93.14 |  |  |
|  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-2A-4A-71A10 | 2 |  |  | -95.8 | -93 | -91.5 | -90.4 | FDD |  |
|  |  |  |  | -98.54 | -95.74 | -94.24 | -93.14 |  |  |
|  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-2A-5A-30A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |  |
| CA_2A-2A-12A-30A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |  |
| CA_2A-2A-66A-71A6.7 | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |  |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 71 |  |  | -69.7 | -69.7 | -69.7 | -69.7 |  |  |
| CA_2A-2A-66A-71A9 | 2 |  |  | -92.4 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  | -95.14 | -95.74 | -94.24 | -93.14 |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-2A-66A-71A10 | 2 |  |  | -95.8 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  | -98.54 | -95.74 | -94.24 | -93.14 |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-4A-4A-5A | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |  |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
| CA_2A-4A-7A-7A | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |  |
|  | 4 |  |  | -99.3 | -96.3 | -94.2 | -93.3 |  |  |
|  |  |  |  | -102.04 | -99.04 | -96.94 | -96.04 |  |  |
|  | 7 |  |  | -97.3 | -94.3 | -92.5 | -91.3 |  |  |
| CA_2A-4A-7A-7A (Note 8) | 2 | - | - | - | - | - | -86.3 | FDD |  |
|  | 2 | - | - | - | - | - | -83.620 |  |  |
|  | 4 | - | - | -99.3 | - | - | - |  |  |
|  |  |  |  | -102.04 |  |  |  |  |  |
|  | 7 | - | - | - | - | - | -91.3 |  |  |
|  | 2 | - | - | -97.3 | - | - | - | FDD |  |
|  | 4 | - | - | -94.3 | - | - | - |  |  |
|  |  |  |  | -97.04 |  |  |  |  |  |
|  | 7 | - | - | - | - | - | -91.3 |  |  |
| CA_2A-12A-66A-66A6,7 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 66 |  |  | -89.5 | -89.0 | -88.5 | -88.0 |  |  |
|  |  |  |  | -92.24 | -91.74 | -91.24 | -90.74 |  |  |
| CA_2A-66A-66A-71A6.7 | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |  |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 71 |  |  | -69.7 | -69.7 | -69.7 | -69.7 |  |  |
| CA_2A-66A-66A-71A9 | 2 |  |  | -92.4 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  | -95.14 | -95.74 | -94.24 | -93.14 |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-66A-66A-71A10 | 2 |  |  | -95.8 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  | -98.54 | -95.74 | -94.24 | -93.14 |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_4A-4A-12A-30A6,7 | 4 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  |  |  |  | -92.74 | -92.24 | -91.74 | -91.24 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  |  |  |  | -101.24 | -98.24 |  |  |  |
| CA_4A-4A-29A-30A | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 | FDD |
|  |  |  |  | -102.34 | -99.34 | -97.54 | -96.34 |  |
|  | 29 |  |  | -97.0 | -94.0 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  |  |  |  | -101.24 | -98.24 |  |  |  |
| CA_5A-30A-66A-66A | 5 |  |  | -97.3 | -94.3 |  |  | FDD |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_12A-30A-66A-66A | 12 |  |  | -96.3 | -93.3 |  |  | FDD |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_14A-30A-66A-66A | 14 |  |  | -96.3 | -93.3 |  |  | FDD |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  | - | - | -101.520 | -98.520 | -96.720 | -95.520 |  |  |
| CA_29A-66A-66A-70A | 29 |  |  | -96.3 | -93.3 |  |  | FDD |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 |  |  |  |
| CA_66A-66A-70A-71A | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 | FDD |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | - |  |  |  |
|  |  |  |  | -89.36,7 | -88.86,7 | -88.56,7 | - |  |  |  |
|  | 71 |  |  | -96.5 | -93.5 | -91.2 | -86.8 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 8: These requirements apply when UL CA is configured and the uplink is active in Band 2 and Band 4. NOTE 9: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and intra-band contiguous carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-7. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-7: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + (inter-band) intra-band contiguous

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |  |
| CA_2A-2A-12B | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |  |
| CA_2C-66A-66A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_66A-66A-70C | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and intra-band non-contiguous carrier aggregation with four downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-8. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-8: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + intra-band non-contiguous

| CA Configuration | ID | PCC |  |  | SCC1 |  |  | SCC2 |  |  | SCC3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band | NRB | Req (dBm) | Band | NRB | Req (dBm) | Band | NRB | Req (dBm) | Band | NRB | Req (dBm) |
| CA_2A-2A-4A-4A | 1 | 2 | 100 | -91.3 | 2 | 25 | -90.11 | 4 | 100 | -93.3 -96.05 | 4 | 100 | -93.3 -96.05 |
|  | 2 | 2 | 100 | -94.0 | 2 | 25 | -90.11 | 4 | 100 | -93.3 -96.05 | 4 | 100 | -93.3 -96.05 |
|  | 3 | 2 | 100 | -91.3 | 2 | 100 | -86.71 | 4 | 100 | -93.3 -96.05 | 4 | 100 | -93.3 -96.05 |
|  | 4 | 4 | 100 | -93.3 -96.05 | 4 | 100 | -93.3 -96.05 | 2 | 100 | -91.3 -94.05 | 2 | 100 | -91.3 -94.05 |
|  | 5 | 4 | 100 | -93.3 -96.05 | 4 | 25 | -99.31 -102.01,5 | 2 | 100 | -91.3 -94.05 | 2 | 100 | -91.3 -94.05 |
| CA_3A-3A-7A-7A | 1 | 3 | 100 | -90.3 | 3 | 25 | -89.81 | 7 | 100 | -91.3 | 7 | 50 | -94.3 |
|  | 2 | 3 | 100 | -90.3 | 3 | 25 | -96.3 | 7 | 100 | -91.3 | 7 | 50 | -94.3 |
|  | 3 | 3 | 100 | -90.3 | 3 | 100 | -86.21 | 7 | 100 | -91.3 | 7 | 50 | -94.3 |
|  | 4 | 3 | 100 | -90.3 | 3 | 50 | -88.21 | 7 | 100 | -91.3 | 7 | 50 | -94.3 |
|  | 5 | 3 | 100 | -90.3 | 3 | 50 | -93.3 | 7 | 100 | -91.3 | 7 | 50 | -94.3 |
|  | 6 | 7 | 100 | -91.3 | 7 | 25 | -97.3 | 3 | 100 | -90.3 | 3 | 50 | -93.3 |
|  | 7 | 7 | 100 | -91.3 | 7 | 25 | -97.3 | 3 | 100 | -90.3 | 3 | 50 | -93.3 |
|  | 8 | 7 | 100 | -91.3 | 7 | 50 | -94.3 | 3 | 100 | -90.3 | 3 | 50 | -93.3 |
|  | 9 | 7 | 100 | -91.3 | 7 | 50 | -94.3 | 3 | 100 | -90.3 | 3 | 50 | -93.3 |
|  | 10 | 7 | 100 | -91.3 | 7 | 100 | -91.3 | 3 | 100 | -90.3 | 3 | 100 | -90.3 |
|  | 11 | 7 | 100 | -91.3 | 7 | 100 | -91.3 | 2 | 100 | -90.3 | 2 | 100 | -90.3 |
| Note 1: Requirements with additional relaxation (ΔRIBNC) as per Table 7.3A.0-3. Note 2: “ID” refers to IDs in Table 7.3A.9.4.1-6. Note 3: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 4: The signal power is specified per port. Note 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### <End of Changes>