3GPP TSG-RAN5 Meeting #100	R5-233992r1

# Toulouse, FR, Aug. 21st - 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3867 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5.3.4 for Rel-17 eNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of Rel-17 eNPN TC 6.5.3.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Rel-17 eNPN TC 6.5.3.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNPN WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0368 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impacts |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: format correction and UE procedure in NR RRC_IDLE state updated. |  |  |  |  |  |  |  |  |

6.5.3.4	SNPN Selection in Automatic Mode / Onboarding services in SNPN

6.5.3.4.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and an SNPN supporting onboarding service is available which does not match the onboarding SNPN selection information }

ensure that {

when { UE is switched on or return to coverage }

then { UE does not select the SNPN cell and continues searching for an allowable SNPN Cell }

}

(2)

with { UE in Automatic network selection mode and Registration for onboardig services on an SNPN cell is unsuccessful }

ensure that {

when { an SNPN is available for which an entry exists in the "list of subscriber data" }

then { UE performs SNPN selection not for onboarding services on the SNPN }

}

6.5.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clauses 4.9.3.1.3. Unless otherwise stated these are Rel-17 requirements.

[TS 23.122, clause 4.9.3.1.3]

When the MS needs to access an SNPN for onboarding services in SNPN, the MS shall select an SNPN indicating that onboarding is allowed and, if the onboarding SNPN selection information is pre-configured, also matching the onboarding SNPN selection information. If more than one such SNPNs are available, how the MS selects one of those SNPNs is MS implementation specific. The MS shall not select an SNPN not indicating that onboarding is allowed or not matching the onboarding SNPN selection information, if pre-configured, for onboarding services in SNPN.

The MS shall limit its search for the SNPN to the NG-RAN access technology.

Once the MS selects the SNPN, the MS shall attempt initial registration for onboarding services in SNPN on the selected SNPN using the NG-RAN access technology and the default UE credentials for primary authentication.

If successful registration is achieved, the MS may indicate to upper layers the selected SNPN. How this indication is displayed by upper layers is implementation specific.

If successful registration is not achieved and one or more other SNPNs indicating that onboarding is allowed and matching the onboarding SNPN selection information, if pre-configured, are available, the MS can select such other SNPN and attempt initial registration for onboarding services in SNPN on the selected SNPN using the NG-RAN access technology and the default UE credentials for primary authentication, or the MS can perform SNPN selection not for onboarding services in SNPN as specified in clause 4.9.3.1.1 or clause 4.9.3.1.2 depending on its SNPN selection mode.

If:

-	registration cannot be achieved because no SNPNs indicating that onboarding is allowed and matching the onboarding SNPN selection information, if pre-configured, are available; or

-	there were one or more SNPNs indicating that onboarding is allowed and matching the onboarding SNPN selection information, if pre-configured, but an LR failure made registration on all those SNPNs unsuccessful;

the MS can indicate no onboarding services to upper layers, enter limited service state and wait until a new SNPN indicating that onboarding is allowed and matching the onboarding SNPN selection information, if pre-configured, is available and then repeat the procedure, or the MS can perform SNPN selection not for onboarding services in SNPN as specified in clause 4.9.3.1.1 or clause 4.9.3.1.2 depending on its SNPN selection mode.

6.5.3.4.3	Test description

6.5.3.4.3.1	Pre-test conditions

System Simulator:

-	3 SNPN cells: NR Cell 1, NR Cell 2 and NR Cell 3 are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-26 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned default UE credentials and is pre-configured with onboarding SNPN selection information to allow access to SNPN identified by NR Cell 1.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NR Cell 3.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN, and the UE is in state Switched OFF (state 0-A).

6.5.3.4.3.2	Test procedure sequence

Table 6.5.3.4.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” and “T3” etc are applied at the points indicated in the Main behaviour description in Table 6.5.3.4.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.3.4.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | "Off" |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | -88 |  |

Table 6.5.3.4.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |

Table 6.5.3.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T1 of table 6.5.3.2.3.2-1/2. | - | - | - | - |
| 2 | The UE is Switched ON. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCSetupRequest on NR Cell 2 within the next 60 s? | --> | RRCSetupRequest | 1 | F |
| 4 | The SS adjusts cell levels according to row T2 of table 6.5.3.2.3.2-1/2. | - | - | - | - |
| 5 | Check: Does the UE send a RRCSetupRequest on NR Cell 1? | --> | RRCSetupRequest | 1 | P |
| 6-16 | Steps 3-13 of Table 4.5.2.2-2 in TS38.508-1 are performed on NR Cell 1. | - | - | - | - |
| 17 | The SS transmits a REGISTRATION REJECT with cause #74 (Temporarily not authorized for this SNPN). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 18 | The SS releases the RRC connection | - | - | - | - |
| 19 | The SS adjusts cell levels according to row T3 of table 6.5.3.2.3.2-1/2. |  |  |  |  |
| 20 | Check: Does the UE send a RRCSetupRequest on NR Cell 3? | --> | RRCSetupRequest | 2 | P |
| 21-38 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NR Cell 3. | - | - | - | - |
| NOTE 1: This is performed via MMI or AT command. |  |  |  |  |  |

6.5.3.4.3.3	Specific message contents

Table 6.5.3.4.3.3-1: REGISTRATION REQUEST (step 7, Table 6.5.3.4.3.2-3)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘101’B |  | SNPN_ONBOARDING |

Table 6.5.3.4.3.3-2: REGISTRATION REJECT for NR Cell 1 (step 17, Table 6.5.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘01001010’B | #74 (Temporarily not authorized for this SNPN). |  |

Table 6.5.3.4.3.3-3: REGISTRATION REQUEST (step 22, Table 6.5.3.4.3.2-3)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |