3GPP TSG-RAN5 Meeting #100	R5-234004r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3871 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of eDRX TC 11.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Release information is missing in the conformance requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Release information was added with some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: undo the changes of ePTW in Table 11.7.2.3.3-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

11.7.2	eDRX / Inactive / RAN-initiated paging

11.7.2.1	Test Purpose (TP)

(1)

with { UE having sent an REGISTRATION REQUEST message with extended DRX parameters IE }

ensure that {

when { the UE receives the extended DRX parameters in the REGISTRATION ACCEPT message}

then { the UE sends a REGISTRATION COMPLETE message }

}

(2)

with { UE in NR RRC_Inactive state and configured eDRX and RAN Extended Paging Cycle equals to the IDLE mode eDRX cycle }

ensure that {

when { the network sends a RAN-initiated Paging message within a paging occassion }

then { the UE resumes RRC connection }

}

(3)

with { UE in NR RRC_Inactive state and configured eDRX and RAN Extended Paging Cycle shorter than the IDLE mode eDRX cycle }

ensure that {

when { the network sends a RAN-initiated Paging message within a paging occassion }

then { the UE applies the value of ran-ExtendedPagingCycle for RAN-initiated paging and resumes RRC connection }

}

(4)

with { UE in NR RRC_INACTIVE state and configured eDRX }

ensure that {

when { network changes SIB1 to exclude eDRX-AllowedInactive IE }

then { UE applies the new system information and stops using extended DRX for RAN paging in RRC_INACTIVE state }

}

11.7.2.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.3.16 and TS 38.304, clause 7.1. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 5.3.16]

Extended DRX (eDRX) cycle is supported for a UE in N1 mode. When eDRX is requested by the UE and accepted by the network:

-	if the UE is not in NB-N1 mode, eDRX is used when the UE is in 5GMM-IDLE mode or in 5GMM-CONNECTED mode with RRC inactive indication; or

-	if the UE is in NB-N1 mode, eDRX is used when the UE is in 5GMM-IDLE mode.

The UE may request the use of eDRX cycle during a registration procedure by including the Requested extended DRX parameters IE (see 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9]). The UE shall not request the use of eDRX during a registration procedure for emergency services. The UE may use the extended idle mode DRX cycle length stored in the USIM (see 3GPP TS 31.102 [22]) when requesting the use of eDRX.

The UE and the network may negotiate eDRX parameters during a registration procedure when the UE has an emergency PDU session.

The network accepts the request to use the eDRX by providing the Negotiated extended DRX parameters IE when accepting the registration procedure. The UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session.

NOTE:	If the UE wants to keep using eDRX, the UE includes the Extended DRX parameters IE in each registration procedure.

If the UE received the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the UE shall resume eDRX.

If the network has provided the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the network shall resume eDRX.

If the UE or the network locally releases an emergency PDU session, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a registration procedure with PDU session context synchronization or upon successful completion of a service request procedure with PDU session context synchronization.

If the UE did not receive the Negotiated extended DRX parameters IE, or if the UE has an emergency PDU session, the UE shall use the stored UE specific DRX parameter, if available.

If the network did not accept the request to use eDRX, or if the UE has an emergency PDU session, the network shall use the stored UE specific DRX parameter, if available.

If the network provided the Negotiated extended DRX parameters IE and also assigned a new 5G-GUTI for the UE as described in subclause 5.5.1.3.4 during the last registration procedure, the network shall use the stored UE specific DRX parameter, if available, with the old 5G-GUTI and use the eDRX provided by the network with the new 5G-GUTI until the old 5G-GUTI can be considered as invalid by the network (see subclauses 5.4.4.4 and 5.5.1.3.4).

[TS 38.304, clause 7.1]

In RRC_INACTIVE state, if eDRX is configured by RRC, i.e., TeDRX, RAN , and/or upper layers, i.e., TeDRX, CN, as defined in clause 7.4:

-	If both TeDRX, CN and TeDRX, RAN are no longer than 1024 radio frames, T = min{TeDRX, RAN, TeDRX, CN}.

-	If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured, T is determined by the shortest of UE specific DRX value configured by RRC and TeDRX, CN.

…

11.7.2.3	Test description

11.7.2.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC.

-	System information combination NR-1 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cell 1, with SIB 1 modified as per Table 11.7.2.3.3-0.

UE:

-	UE configured to request eDRX.

Preamble:

-	UE is in state 0N-B according to TS 38.508-1 [4] Table 4.4A.2-2.

11.7.2.3.2	Test procedure sequence

Table 11.7.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-3 | Steps 2-3 of the generic procedure for NR RRC_INACTIVE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. | - | - | - | - |
| 4 | The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5-13 | Steps 5-13 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 14 | The SS transmit a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | Check: Does the UE transmit a REGISTRATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | 1 | P |
| 16-18 | Void | - | - | - | - |
| 19Aa1-20 | Steps 19Aa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 21 | The SS transmits a Paging message within a paging occasion. | <-- | NR RRC: Paging | - | - |
| 22 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 2 | P |
| 23 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 24 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 25 | Void | - | - | - | - |
| 26 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: NR RRC: RRCRelease | - | - |
| 27 | The SS transmits a Paging message to the UE in a valid PO which is derived by the value of ran-ExtendedPagingCycle IE. | <-- | NR RRC: Paging | - | - |
| 28 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 3 | P |
| 29 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 30 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 31 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 32 | The SIB1 is updated according to Table 11.7.2.3.3-0 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 33 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 34 | The SS transmits a Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. | <-- | NR RRC: Paging | - | - |
| 35 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 4 | P |
| 36 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 37 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |

11.7.2.3.3	Specific message contents

Table 11.7.2.3.3-0: SIB1 (preamble and step 32, Table 11.7.2.3.2-1)

| Derivation path: TS 38.508-1 [4] , table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| hyperSFN-r17 |  | Current H-SFN Value |  | Preamble |
|  |  | Not present |  | Step 32 |
| eDRX-AllowedIdle-r17 |  | true |  | Preamble |
|  |  | Not present |  |  |  | Step 32 |  |
| eDRX-AllowedInactive-r17 |  | true |  |  |  | Preamble |  |
|  |  | Not present |  |  |  | Step 32 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 11.7.2.3.3-1: REGISTRATION REQUEST (step 4, Table 11.7.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested extended DRX parameters |  |  |  |
| Paging Time Window | Present but contents not checked |  |  |
| eDRX value | Present but contents not checked |  |  |
| Extended Paging Time Window | Not Checked | Note 1 |  |
| Note 1: Extended Paging Time Window is an optional IE. If UE request eDRX cycle larger than 10,24 seconds, UE may include Extended Paging Time Window. |  |  |  |

Table 11.7.2.3.3-2: REGISTRATION ACCEPT (step 14, Table 11.7.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | Any allowed value | Note 1 |  |
| eDRX value | ‘0001’B | 5,12 seconds |  |
| Extended Paging Time Window | ‘00000001’B | 2,56 seconds |  |
| Note 1: In NR connected to 5GCN, the Paging Time Window field is ignored and the PTW value is included in the Extended Paging Time Window field. |  |  |  |

Table 11.7.2.3.3-3: RRCRelease (step 20, step 26 and step 31, Table 11.7.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-16 with condition NR_RRC_INACTIVE |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |  |
| ran-ExtendedPagingCycle-r17 |  | rf512 | 5.12 seconds | Step 20 |
|  |  | rf256 | 2.56 seconds | Step 26 and Step 31 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 11.7.2.3.3-4: Paging (steps 21, 27 and 34, Table 11.7.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

[End of change]