3GPP TSG-RAN5 Meeting #100	R5-234019r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3882 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to eNS NSAC TC 9.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to the conformance requirements in TS 24.501 Cl. 4.6.1, the TAIs belonging to different PLMNs, which are equivalent PLMNs, shall be included, then the TP2 could be satisfied. If the UE receives a rejected NSSAI for the maximum number of UEs reached, the registration area contains TAIs belonging to different PLMNs, which are equivalent PLMNs, the UE shall treat the received rejected NSSAI for the maximum number of UEs reached as applicable to these equivalent PLMNs when the UE is in this registration area. - According to the conformance requirements in TS 24.501, besides the storage and removal of rejected S-NSSAI with rejection cause "S-NSSAI not available due to maximum number of UEs reached", there is no clear requirements regarding whether UE needs to perfrom initial registration of not, only some “may” requirement as below. To test the deterministic UE behaviors, it`s proposed to use AT+C5GNSSAIRDP command will align with other eNS test cases. If the UE has neither allowed NSSAI for the current PLMN or SNPN nor configured NSSAI for the current PLMN or SNPN and has rejected NSSAI for the reached maximum number of UEs, and the UE wants to obtain services in the current serving cell without performing a PLMN selection or SNPN selection, the UE may stay in the current serving cell and attempt to use the rejected S-NSSAI(s) for the maximum number of UEs reached in the current serving cell after the rejected S-NSSAI(s) are removed as described in subclause 4.6.2.2 - As T3526 is implementation specific with a minimum value of 12 mins, therefore some UE may have a longer value than 12 mins. And in order to save test time, it`s suggested to speficy this timer with a short value. NOTE 9: The value of this timer is provided by the network operator during the registration procedure or the generic UE configuration update procedure along with the rejected S-NSSAI with rejection cause "S-NSSAI not available due to maximum number of UEs reached". The default value of this timer is implementation specific with a minimum value of 12 minutes and used if the network does not provide a value in the REGISTRATION ACCEPT message, the REGISTRATION REJECT message, or the CONFIGURATION UPDATE COMMAND message along with the rejected S-NSSAI with rejection cause "S-NSSAI not available due to maximum number of UEs reached". |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TAI list was added to include the TAIs of equivalent PLMN. - Use AT+C5GNSSAIRDP command to check UE behaviors for TPs. - T3526 was specified to 3 mins. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: To avoid possible registration procedure during the test, set the cells to off. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1.12.3	NSAC / Initial registration / Rejected / equivalent PLMNs

9.1.12.3.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives the rejected NSSAI in the REGISTRATION REJECT message with rejection cause of “S-NSSAI not available due to maximum number of UEs reached” }

then { UE shall store the rejected NSSAI and start the timer T3526 }

}

(2)

with { UE receives rejected NSSAI with rejection cause of “S-NSSAI not available due to maximum number of UEs reached” }

ensure that {

when { UE selects to the equivalent PLMN in the same registration area }

then { UE shall treat the received rejected NSSAI for the maximum number of UEs reached as applicable to equivalent PLMNs }

}

(3)

with { UE receives rejected NSSAI with rejection cause of “S-NSSAI not available due to maximum number of UEs reached” }

ensure that {

when { UE timer T3526 expires }

then { Remove the S-NSSAI in the rejected NSSAI for the maximum number of UEs reached }

}

9.1.12.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501, clauses 5.5.1.2.5, 4.6.1. Unless otherwise stated these are Rel-17 requirements

[TS 24.501, clause 5.5.1.2.5]

#62	(No network slices available).

The UE shall abort the initial registration procedure, set the 5GS update status to 5U2 NOT UPDATED and enter state 5GMM-DEREGISTERED. ATTEMPTING-REGISTRATION or 5GMM-DEREGISTERED.PLMN-SEARCH. Additionally, the UE shall reset the registration attempt counter.

The UE receiving the rejected NSSAI in the REGISTRATION REJECT message takes the following actions based on the rejection cause in the rejected S-NSSAI(s):

"S-NSSAI not available in the current PLMN or SNPN"

The UE shall store the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN or SNPN as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI(s) in the current PLMN or SNPN until switching off the UE, the UICC containing the USIM is removed, an entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed or deleted as described in subclause 4.6.2.2.

"S-NSSAI not available in the current registration area"

The UE shall store the rejected S-NSSAI(s) in the rejected NSSAI for the current registration area as described in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI(s) in the current registration area until switching off the UE, the UE moving out of the current registration area, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed or deleted as described in subclause 4.6.2.2.

"S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization"

The UE shall store the rejected S-NSSAI(s) in the rejected NSSAI for the failed or revoked NSSAA as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI in the current PLMN or SNPN over any access until switching off the UE, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed or deleted as described in subclause 4.6.1 and 4.6.2.2.

"S-NSSAI not available due to maximum number of UEs reached"

Unless the back-off timer value received along with the S-NSSAI is zero, the UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the maximum number of UEs reached as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI in the current PLMN or SNPN over the current access until switching off the UE, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed as described in subclause 4.6.2.2.

NOTE 6:	If the back-off timer value received along with the S-NSSAI in the rejected NSSAI for the maximum number of UEs reached is zero as specified in subclause 10.5.7.4a of TS 24.008, the UE does not consider the S-NSSAI as the rejected S-NSSAI.

If there is one or more S-NSSAIs in the rejected NSSAI with the rejection cause "S-NSSAI not available due to maximum number of UEs reached", then for each S-NSSAI, the UE shall behave as follows:

a)	stop the timer T3526 associated with the S-NSSAI, if running;

b)	start the timer T3526 with:

1)	the back-off timer value received along with the S-NSSAI, if a back-off timer value is received along with the S-NSSAI that is neither zero nor deactivated; or

2)	an implementation specific back-off timer value, if no back-off timer value is received along with the S-NSSAI; and

c)	remove the S-NSSAI from the rejected NSSAI for the maximum number of UEs reached when the timer T3526 associated with the S-NSSAI expires.

If the UE has an allowed NSSAI or configured NSSAI that contains S-NSSAI(s) which are not included in the rejected NSSAI the UE may stay in the current serving cell, apply the normal cell reselection process and start an initial registration with a requested NSSAI that includes any S-NSSAI from the allowed NSSAI or the configured NSSAI that is not in the rejected NSSAI. Otherwise the UE may perform a PLMN selection or SNPN selection according to 3GPP TS 23.122 [5] and additionally, the UE may disable the N1 mode capability for the current PLMN or SNPN if the UE does not have an allowed NSSAI and each S-NSSAI in configured NSSAI, if available, was rejected with cause "S-NSSAI not available in the current PLMN or SNPN" or "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" as described in subclause 4.9.

If the UE has neither allowed NSSAI for the current PLMN or SNPN nor configured NSSAI for the current PLMN or SNPN and,

1)	if at least one S-NSSAI in the default configured NSSAI is not rejected, the UE may stay in the current serving cell, apply the normal cell reselection process, and start an initial registration with a requested NSSAI with that default configured NSSAI; or

2)	if all the S-NSSAI(s) in the default configured NSSAI are rejected and at least one S-NSSAI is rejected due to "S-NSSAI not available in the current registration area",

i)	if the REGISTRATION REJECT message is integrity protected and the UE is not operating in SNPN access operation mode, the UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" and enter the state 5GMM-DEREGISTERED.LIMITED-SERVICE; or

ii)	if the REGISTRATION REJECT message is integrity protected and the UE is operating in SNPN access operation mode, the UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" for the current SNPN and enter the state 5GMM-DEREGISTERED.LIMITED-SERVICE.

Otherwise, the UE may perform a PLMN selection or SNPN selection according to 3GPP TS 23.122 [5] and additionally, the UE may disable the N1 mode capability for the current PLMN or SNPN if each S-NSSAI in the default configured NSSAI was rejected with cause "S-NSSAI not available in the current PLMN or SNPN" or "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" as described in subclause 4.9.

If the UE has neither allowed NSSAI for the current PLMN or SNPN nor configured NSSAI for the current PLMN or SNPN and has rejected NSSAI for the reached maximum number of UEs, and the UE wants to obtain services in the current serving cell without performing a PLMN selection or SNPN selection, the UE may stay in the current serving cell and attempt to use the rejected S-NSSAI(s) for the maximum number of UEs reached in the current serving cell after the rejected S-NSSAI(s) are removed as described in subclause 4.6.2.2.

If the message was received via 3GPP access and the UE is operating in single-registration mode, the UE shall in addition set the EPS update status to EU2 NOT UPDATED, reset the attach attempt counter and enter the state EMM-DEREGISTERED.

[TS 24.501, clause 4.6.1]

The rejected NSSAI for the maximum number of UEs reached is applicable for the whole registered PLMN or SNPN, and the access type over which the rejected NSSAI was sent. The AMF shall send a rejected NSSAI including S-NSSAI(s) with the rejection cause "S-NSSAI not available due to maximum number of UEs reached", when one or more S-NSSAIs are indicated that the maximum number of UEs has been reached. If the timer T3526 associated with the S-NSSAI(s) was started upon reception of the rejected NSSAI for the maximum number of UEs reached, the UE may remove the S-NSSAI(s) from the rejected NSSAI including S-NSSAI(s) with the rejection cause "S-NSSAI not available due to maximum number of UEs reached", if the timer T3526 associated with the S-NSSAI(s) expires. If one or more S-NSSAIs are removed from the rejected NSSAI for the maximum number of UEs reached, the timer T3526 associated with the removed S-NSSAI(s) shall be stopped, if running. The UE shall not stop the timer T3526 if the UE selects an E-UTRA cell connected to EPC.

If the UE receives a rejected NSSAI for the maximum number of UEs reached, the registration area contains TAIs belonging to different PLMNs, which are equivalent PLMNs, the UE shall treat the received rejected NSSAI for the maximum number of UEs reached as applicable to these equivalent PLMNs when the UE is in this registration area.

[TS 24.501, clause 10.2]

Table 10.2.1: Timers of 5GS mobility management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3502 | Default 12 min. NOTE 1 | 5GMM-DEREGISTERED 5GMM-REGISTERED | At registration failure and the attempt counter is equal to 5 | Transmission of REGISTRATION REQUEST message | Initiation of the registration procedure, if still required |
| … |  |  |  |  |  |
| T3526 | NOTE 9 | 5GMM-DEREGISTERED 5GMM-REGISTERED | Rejected S-NSSAI with rejection cause "maximum number of UEs per network slice reached" received. | The associated rejected S-NSSAI for the maximum number of UEs reached as specified in subclause 4.6.2.2 deleted. | Remove the S-NSSAI in the rejected NSSAI for the maximum number of UEs reached associated with the T3526 timer. |

9.1.12.3.3	Test description

9.1.12.3.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A (home PLMN) and NGC Cell E are configured according to Table 6.3.2.2-1 in TS 38.508-1 [4].

-	System information combination NR-2 in accordance with TS 38.508-1 [4] sub-clause 4.4.3.1.2 is used in NGC Cell A and NGC Cell E.

-	The UE is last authenticated and registered on NGC Cell A and NGC Cell E PLMN is provided as equivalent PLMN.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.1.12.3.3.2	Test procedure sequence

Table 9.1.12.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NGC Cell A as the "Serving cell ". - NGC Cell E as “Non-suitable "Off" cell”. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3-14 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 15 | The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #62 "No network slices available" and rejection cause in the Extended rejected S-NSSAI is set =’0011’B “S-NSSAI not available due to maximum number of UEs reached”, T3526 is present with a value of 3 minutes. | <-- | REGISTRATION REJECT | - | - |
| 16 | SS starts timer of t_Waits= T3526. | - | - | - | - |
| 17 | The SS releases the RRC connection. | - | - | - | - |
| 17A | The SS reconfigures: - NGC cell E as a "Non-suitable "Off" cell". - NGC cell A as a "Non-suitable "Off" cell". | - | - | - | - |
| 18 | Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause ‘S-NSSAI not available due to maximum number of UEs reached’’ associated with current PLMN (HPLMN) using AT/MMI command (+C5GNSSAIRDP)? | - | - | 1 | P |
| 19 | The SS reconfigures: - NGC cell E as the "Serving cell". - NGC cell A as a "Non-suitable "Off" cell". | - | - | - | - |
| 20 | Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause ‘S-NSSAI not available due to maximum number of UEs reached’’ associated with current PLMN (ePLMN) using AT/MMI command (+C5GNSSAIRDP)? | - | - | 2 | P |
| 20A | The SS reconfigures: - NGC cell E as a "Non-suitable "Off" cell". - NGC cell A as a "Non-suitable "Off" cell". | - | - | - | - |
| 21 | SS waits for the expiration of t_Waits. | - | - | - | - |
| 22 | Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=1 is not in the Rejected NSSAI list associated with current PLMN? | - |  | 3 | P |
| 23-38a1 | Void. | - | - | - | - |

9.1.12.3.3.3	Specific message contents

Table 9.1.12.3.3.3-0: REGISTRATION ACCEPT (preamble, Table 9.1.12.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=002, MNC=101 | PLMN ID of NGC Cell E |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 1100'B | 2 entries |  |
| Partial tracking area identity list 1 |  | entry 1 | TAI-1 |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | 001 | MCC of NGC Cell A |  |
| MNC |  | 01 | MNC of NGC Cell A |  |
| TAC 1 |  | 1 | TAC of NGC Cell A |  |
| Partial tracking area identity list 2 |  | entry 2 | TAI-12 |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | 002 | MCC of NGC Cell E |  |
| MNC |  | 101 | MNC of NGC Cell E |  |
| TAC 1 |  | 3 | TAC of NGC Cell E |  |
| Allowed NSSAI |  |  |  |  |
| Length of NSSAI contents |  | '0000 0010'B | 1 entry |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000001'B | SST |  |
| SST |  | '00000001'B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| Length of Configured NSSAI contents |  | '0000 0010'B | 1 entry |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000001'B | SST |  |
| SST |  | '00000001'B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.12.3.3.3-1: REGISTRATION REJECT (step 15, Table 9.1.12.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘0011 1110’B | #62 “No network slices available” |  |
| Extended rejected NSSAI |  |  |  |
| Length of Extended rejected NSSAI contents | ‘00000100’B |  |  |
| Partial extended rejected NSSAI list |  | entry 1 |  |
| Type of list | ‘001’B | S-NSSAIs with one associated back-off timer value that applies to all S-NSSAIs in the list |  |
| Number of elements | ‘0000’B | 1 element |  |
| Back-off timer value |  |  |  |
| Unit | '101'B | value is incremented in multiples of 1 minute |  |
| Timer value | '0 0011'B | 3 minutes |  |
| Rejected S-NSSAI 1 |  | entry 1 |  |
| Length of rejected S-NSSAI | ‘0100’B |  |  |
| Cause value | ‘0011’B | S-NSSAI not available due to maximum number of UEs reached. |  |
| SST | ‘00000001’B | SST value 1 |  |
| SD | ’FFFFFF’H | no SD value associated with the SST |  |
| Mapped HPLMN SST | Not Present |  |  |
| Mapped HPLMN SD | Not Present |  |  |

[End of change]