Source:	CAICT, Huawei, Qualcomm

Title:	Discussion on test point selection for Maximum input level in FR1

# Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for Maximum input level in FR1.

The purpose of this paper is to provide an analysis for the parameters in test configuration table and a selection of test points for test environment, test frequencies, channel bandwidths, SCS, RB allocation and modulation for NR Maximum input level test case in FR1. Firstly single transmission case is done, then UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

# Discussion

Maximum input level for FR1 will be defined in [2] where it is explained the scope of the Maximum input level as follows:

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to a g-NodeB.

Based on the Minimum conformance requirements for Maximum input level for FR1in [3]:

7.4	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4-1.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink and downlink modulations and RB configurations.

## Test Environment

Based on the baseline agreed in [1], the test environment for Maximum input level measurement for FR1 can be defined as in LTE. In LTE, environment condition for Maximum input level test case is specified for Normal Condition (NC).

PROPOSAL 1: Select Test Environment as Normal Condition for Maximum input level in FR1.

## Test Frequencies

In LTE, Maximum input level is required to be performed at Mid range frequency for all bands. In NR FR1, similar test frequencies shall be adopted which is consistent with proposal provided in Reference [1]

PROPOSAL 2: Select Mid range test frequency for Maximum input level in FR1.

## Test Channel Bandwidth

In LTE, Maximum input level is tested for Lowest, 5 MHz, and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE receiver related to the different channel bandwidths.

Follow similar strategy, NR Maximum input level for FR1 shall be tested for Lowest, Mid and Highest channel bandwidth across all NR FR1 band.

PROPOSAL 3: Select Lowest, Mid and Highest test channel bandwidth for Maximum input level in FR1.

## Test Subcarrier Spacing

As discussed in previous test point analysis papers, the key element in choosing subcarrier spacing for testing is to achieve maximum spectrum utilization. The conclusion is that Lowest SCS will provide maximum spectrum utilization. Details can be found in Reference [4]

PROPOSAL 4: Select Lowest supported subcarrier spacing for Maximum input level in FR1.

## Test Modulation and RB allocation

The selection of test points for modulation and RB allocation shall be based on the Minimum conformance requirements for Maximum input level in [3], which are extracted as bellow:

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

According to the above table, 64-QAM, 256-QAM and 1024 QAM are used for downlink for Maximum input level. And uplink RB allocation shall be set according to Table 7.3.2-3 in [3], downlink RB allocation shall be set according to Annex A.3.2.3/A.3.3.3 in [3] for 64-QAM, Annex A.3.2.4/A.3.3.4 in [3] for 256-QAM, and Annex A.3.2.5/A.3.3.5 in [3] for 1024-QAM

In LTE, QPSK is used for uplink for all bands and all channel bandwidths testing for Maximum input level due to its robustness. It is reasonable to follow LTE philosophy for NR Maximum input level for FR1.

In reference [4], the spectrum utilization for different modulations, subcarrier spacing and channel bandwidths are discussed in detail. The conclusion is that for a specific SCS and ChBw the number of resource blocks for DFT-s modulations is equal or lower than the used by CP modulations. According to Annex A.3.2.3/A.3.3.3 in [3] for 64-QAM, Annex A.3.2.4/A.3.3.4 in [3] for 256-QAM, and Annex A.3.2.5/A.3.3.5 in [3] for 1024-QAM, full RB allocation shall be used for downlink configuration for Maximum input level in order to achieve maximum spectrum utilization. So it is reasonable to use CP-OFDM waveform for downlink configuration.

Considering NR Maximum input level evaluates the receptor characteristics, the selection of the uplink modulations does not seem so critical. However, this receiver characteristic is measured by measuring downlink Throughput due to the HARQ feedback sent in PUSCH, so it is important to select the right uplink modulation to avoid any issue with uplink. So it is reasonable to use DFT-s-OFDM waveform which presents lower PAPR compared to CP-OFDM to avoid any uplink issues. Besides this kind of waveform is recommended in case of high power level due to it is more power efficient.

PROPOSAL 5: Select CP-OFDM 64 QAM and CP-OFDM 256 QAM for downlink configurations for Maximum input level in FR1.

PROPOSAL 6: Select DFT-s-OFDM QPSK for uplink configurations for Maximum input level in FR1.

PROPOSAL 7: Uplink RB allocation shall be set according to Table 7.3.2-3 in [3] for Maximum input level in FR1.

PROPOSAL 8: Downlink RB allocation shall be set according to Annex A.3.2.3/A.3.3.3 in [3] for 64-QAM, Annex A.3.2.4/A.3.3.4 in [3] for 256-QAM, and Annex A.3.2.5/A.3.3.5 in [3] for 1024-QAM for Maximum input level in FR1.

## Exceptions for MaxIL UL-MIMO test

Extracted from [3]

7.3D	Reference sensitivity for UL-MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.3 shall be met with the UL-MIMO configurations described in sub-clause 6.2D.1. For UL-MIMO, the parameter PUMAX is the total transmitter power over the two transmits power over the two transmit antenna connectors..

Observation 1: Same maximum input level requirements as specified for single transmission apply with UL-MIMO configuration.

According to [5], CP-OFDM is mandatory to UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the test points for MaxIL UL-MIMO measurement. Generally receiver test are not sensitive to the UL waveform and modulation configuration, straightforwardly test CO-OFDM instead of DFT-s-OFDM.

PROPOSAL 9: Test CP-OFDM QPSK for MaxIL UL-MIMO measurement in FR1, while the other test parameters are the same as single transmission case

## Exceptions for Asymmetric CH BWs

Similar to any 1UL/xDL intra-band contiguous CA, asymmetric CH BWs should not have UL impact in Tx requirements. But similarly, it should be tested in Rx requirements.

PROPOSAL 10: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Maximum input level test case definition:

PROPOSAL 1: Select Test Environment as Normal Condition for Maximum input level in FR1.

PROPOSAL 2: Select Mid range test frequency for Maximum input level in FR1.

PROPOSAL 3: Select Lowest, Mid and Highest test channel bandwidth for Maximum input level in FR1.

PROPOSAL 4: Select Lowest supported subcarrier spacing for Maximum input level in FR1.

PROPOSAL 5: Select CP-OFDM 64 QAM and CP-OFDM 256 QAM for downlink configurations for Maximum input level in FR1.

PROPOSAL 6: Select DFT-s-OFDM QPSK for uplink configurations for Maximum input level in FR1.

PROPOSAL 7: Uplink RB allocation shall be set according to Table 7.3.2-3 in [3] for Maximum input level in FR1.

PROPOSAL 8: Downlink RB allocation shall be set according to Annex A.3.2.3/A.3.3.3 in [3] for 64-QAM, Annex A.3.2.4/A.3.3.4 in [3] for 256-QAM, and Annex A.3.2.5/A.3.3.5 in [3] for 1024-QAM for Maximum input level in FR1.

PROPOSAL 9: Test CP-OFDM QPSK for MaxIL UL-MIMO measurement in FR1, while the other test parameters are the same as single transmission case.

PROPOSAL 10: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) 1 | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | symmetric channel bandwidths: 3 asymmetric channel bandwidths: 2 | 1 | 3 | symmetric channel bandwidths: 9 asymmetric channel bandwidths: 6 |

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs" (NTT DOCOMO).

- 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

- 3GPP TS 38.101-1 v17.10.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

- R5-180886, "Discussion_NR_FR1_TestPoints_For_SEM" (Keysight Technologies).

- R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019