3GPP TSG-RAN5 Meeting #100	R5-234100r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3890 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.1.6.1.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 8.1.6.1.3.10 needs to be added according to the WP for NR_ENDC_SON_MDT_enh-UEConTest. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.1.6.1.3.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-2 CR 0374 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Correctted editorial mistakes in test purpose and test procedure. Updated specific message contents for CellGroupConfig and UEInformationResponse. |  |  |  |  |  |  |  |  |

8.1.6.1.3.10	Radio Link Failure / Logging and reporting / DAPS HO

8.1.6.1.3.10.1	Test Purpose (TP)

(1)

with { UE having detected DAPS handover failure and radio link failure in source PCell while T304 was running }

ensure that {

when { UE receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable }

}

(2)

with { UE having detected DAPS handover failure and radio link failure in source PCell while T304 was running }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report for DAPS handover failure}

}

8.1.6.1.3.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.5, 5.3.10.3, 5.3.10.5, 5.3.5.8.3, 5.3.5.3 and 5.7.10.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

…

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.10.3	Test description

8.1.6.1.3.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell and NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR Cells

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.10.3.2	Test procedure sequence

Table 8.1.6.1.3.10.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.3.10.3.2-2.

Table 8.1.6.1.3.10.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | off |  |

Table 8.1.6.1.3.10.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | off |  |

Table 8.1.6.1.3.10.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel with step 2, parallel behaviour defined in table 8.1.6.1.3.10.3.2-4 is executed repeatedly. | - | - | - | - |
| 2 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 3 | Waiting for T304 expiring. |  |  |  |  |
| 4 | The SS changes the power level setting according to the row "T2".- |  |  |  |  |
| 5 | The UE transmits an RRCReestablishmentRequest message in NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 6 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security in NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 7 | Check: Does the UE transmits an RRCReestablishmentComplete message with rlf-InfoAvailable in NR Cell 1. | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 8 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 10 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 11 | Check: Does the UE transmit a UEInformationResponse message including lastHO-Type-r17 set to daps and timeConnSourceDAPS-Failure-r17 in NR Cell 2? | --> | NR RRC: UEInformationResponse | 2 | P |

Table 8.1.6.1.3.10.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits preamble to NR Cell 2. | -> | (PRACH Preamble) | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.1.6.1.3.10.3.3	Specific message contents

Table 8.1.6.1.3.10.3.3-1: SIB1 for NR Cell 1 (all steps)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms0 | 0 ms |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-2: RRCReconfiguration (step 1, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions (SRB_NR_PDCP) and (DRBn AND DAPS_PDCP) |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig)Table 8.1.6.1.3.10.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-3: CellGroupConfig (Table 8.1.6.1.3.10.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRBn | entry 3 |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | Table 8.1.4.3.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-4: RACH-ConfigGeneric (Table 8.1.6.1.3.10.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-5: RRCReestablishment (step 6, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-6: RRCReestablishmentComplete (step 7, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-5: RRCReconfiguration (step 8, Table 8.1.6.1.3.10.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.10.3.3-6: UEInformationRequest (step 10, Table 8.1.6.1.3.10.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.10.3.3-7: UEInformationResponse (step 11, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 CHOICE { |  |  |  |
| nrPreviousCell-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| eutraFailedPCellId-r16 | Not present |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | hof |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| lastHO-Type-r17 | daps |  |  |
| timeConnSourceDAPS-Failure-r17 | (0..1023) |  |  |
| timeSinceCHO-Reconfig-r17 | Not present |  |  |
| choCellId-r17 | Not present |  |  |
| choCandidateCellList-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |