3GPP TSG-RAN5 Meeting #100	R5-234105r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2823 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update default message contents of UEInformationRequest for MDT enhance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Default message contents of UEInformationRequest should be updated for MDT enhance test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added ueInformationRequest-r17 SEQUENCE in UEInformationRequest message.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Default message contents of UEInformationRequest will not be applicable to MDT enhance test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Reserve Not present value of nonCriticalExtension for other conditions. |  |  |  |  |  |  |  |  |

–	UEInformationRequest

Table 4.6.1-32A: UEInformationRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r16 |  | Not present |  |  |
|  |  | true |  | IDLE |
| logMeasReportReq-r16 |  | Not present |  |  |
|  |  | true |  | LOG |
| connEstFailReportReq-r16 |  | true |  |  |
| ra-ReportReq-r16 |  | Not present |  |  |
|  |  | true |  | RA |
| rlf-ReportReq-r16 |  | Not present |  |  |
|  |  | true |  | RLF |
| mobilityHistoryReportReq-r16 |  | Not present |  |  |
|  |  | true |  | HISTORY |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension CHOICE { |  |  |  | SHR, CL |
| ueInformationRequest-r17 SEQUENCE { |  |  |  |  |
| successHO-ReportReq-r17 |  | Not present |  |  |
|  |  | true |  | SHR |
| coarseLocationRequest-r17 |  | Not present |  |  |
|  |  | true |  | CL |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| IDLE | Idle/inactive measurement information reporting |
| LOG | Logged measurement information reporting |
| RA | Random access procedure information reporting |
| RLF | Radio link failure information reporting |
| HISTORY | Mobility history information reporting |
| SHR | Successful handover information reporting |
| CL | Coarse location information reporting |