3GPP TSG-RAN5 Meeting #100	R5-234108r2

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3893 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update eNS test case 9.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE behavior stopping timer T3526 can’t be verified. Some mistakes in test procedure and specific message contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TPs, and corrected test procedure and specific message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: updated Work item code from “eNS_Ph2-UEConTest, TEI17_Test” to “TEI17_Test, eNS_Ph2-UEConTest ”. Revision 2: Updated test purpose and test procedure sequence. Updated specific message contents in Table 9.3.1.4.3.3-2 & 3. |  |  |  |  |  |  |  |  |

9.3.1.4	NSAC / interworking with EPC

9.3.1.4.1	Test Purpose (TP)

(1)

with { UE stores one or more rejected S-NSSAIs for the maximum number of UEs reached and T3526 is running }

ensure that {

when { UE selects an E-UTRA cell connected to EPC }

then { UE shall not remove the rejected S-NSSAIs for the maximum number of UEs reached  }

}

(2)

with { UE stores one rejected S-NSSAI for the current PLMN }

ensure that {

when { UE receive ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message provided with S-NSSAI and the PLMN ID in E-UTRA cell connected to EPC }

then { UE removes the S-NSSAI from the rejected NSSAI list and sends an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message }

}

9.3.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], subclauses 4.6.1 and 4.6.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, subclause 4.6.1]

The 5GS supports network slicing as described in 3GPP TS 23.501 [8]. Within a PLMN or SNPN, a network slice is identified by an S-NSSAI, which is comprised of a slice/service type (SST) and a slice differentiator (SD). Inclusion of an SD in an S-NSSAI is optional. A set of one or more S-NSSAIs is called the NSSAI. The following NSSAIs are defined in 3GPP TS 23.501 [8]:

a)	configured NSSAI;

b)	requested NSSAI;

c)	allowed NSSAI;

d)	subscribed S-NSSAIs; and

e)	pending NSSAI.

The following NSSAIs are defined in the present document:

a)	rejected NSSAI for the current PLMN or SNPN;

b)	rejected NSSAI for the current registration area;

c)	rejected NSSAI for the failed or revoked NSSAA; and

d)	rejected NSSAI for the maximum number of UEs reached.

…

The rejected NSSAI for the maximum number of UEs reached is applicable for the whole registered PLMN or SNPN. The AMF shall send a rejected NSSAI including S-NSSAI(s) with the rejection cause "S-NSSAI not available due to maximum number of UEs reached", when one or more S-NSSAIs are indicated that the maximum number of UEs has been reached. If the timer T3526 associated with the S-NSSAI(s) was started upon reception of the rejected NSSAI for the maximum number of UEs reached, the UE may remove the S-NSSAI(s) from the rejected NSSAI including S-NSSAI(s) with the rejection cause "S-NSSAI not available due to maximum number of UEs reached", if the timer T3526 associated with the S-NSSAI(s) expires. If one or more S-NSSAIs are removed from the rejected NSSAI for the maximum number of UEs reached, the timer T3526 associated with the removed S-NSSAI(s) shall be stopped, if running. The UE shall not stop the timer T3526 if the UE selects an E-UTRA cell connected to EPC.

[TS 24.501, subclause 4.6.2.2]

If available, the configured NSSAI(s) shall be stored in a non-volatile memory in the ME as specified in annex C. For a configured NSSAI, if there is associated NSSRG information, the NSSRG information shall also be stored in a non-volatile memory in the ME as specified in annex C. The support for NSSRG information by a UE or an AMF is optional.

The allowed NSSAI(s) should be stored in a non-volatile memory in the ME as specified in annex C.

Each of the configured NSSAI stored in the UE is a set composed of at most 16 S-NSSAIs. Each of the allowed NSSAI stored in the UE is a set composed of at most 8 S-NSSAIs and is associated with a PLMN identity or SNPN identity, an access type and, if the UE supports access to an SNPN using credentials from a credentials holder, the selected entry of the "list of subscriber data" or the selected PLMN subscription. Each of the configured NSSAI except the default configured NSSAI, and the rejected NSSAI is associated with a PLMN identity or SNPN identity and, if the UE supports access to an SNPN using credentials from a credentials holder, the selected entry of the "list of subscriber data" or the selected PLMN subscription. Each of the pending NSSAI stored in the UE is a set composed of at most 16 S-NSSAIs and is associated with a PLMN identity or SNPN identity and, if the UE supports access to an SNPN using credentials from a credentials holder, the selected entry of the "list of subscriber data" or the selected PLMN subscription. The S-NSSAI(s) in the rejected NSSAI for the current registration area are further associated with one or more tracking areas where the rejected S-NSSAI(s) is not available. The S-NSSAI(s) in the rejected NSSAI for the current PLMN or SNPN shall be considered rejected for the current PLMN or SNPN regardless of the access type. The S-NSSAI(s) in the rejected NSSAI for the failed or revoked NSSAA shall be considered rejected for the current PLMN regardless of the access type. The S-NSSAI(s) in the rejected NSSAI for the maximum number of UEs reached are further associated with the access type over which the rejected NSSAI was received. There shall be no duplicated PLMN identities or SNPN identities associated with each of the list of configured NSSAI(s), pending NSSAI(s), rejected NSSAI(s) for the current PLMN or SNPN, rejected NSSAI(s) for the current registration area, rejected NSSAI(s) for the failed or revoked NSSAA, and rejected NSSAI for the maximum number of UEs reached.

The UE stores NSSAIs as follows:

…

c)	When the UE receives the S-NSSAI(s) included in the rejected NSSAI in the REGISTRATION ACCEPT message, the REGISTRATION REJECT message, the DEREGISTRATION REQUEST message or in the CONFIGURATION UPDATE COMMAND message, the UE shall:

1)	store the S-NSSAI(s) into the rejected NSSAI and the mapped S-NSSAI(s) for the rejected NSSAI based on the associated rejection cause(s);

...

When the UE receive ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message provided with S-NSSAI and the PLMN ID in the protocol configuration options IE or extended protocol configuration options IE (see subclause 6.2.2 of 3GPP TS 24.301 [15]), the UE shall remove the S-NSSAI from the rejected NSSAI for the current PLMN. When the UE receive ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message provided with S-NSSAI and the PLMN ID in the protocol configuration options IE or extended protocol configuration options IE (see subclause 6.2.2 of 3GPP TS 24.301 [15]), the UE may remove the S-NSSAI from the rejected NSSAI for the maximum number of UEs reached for each and every access type, if any, and stop the timer T3526 associated with the S-NSSAI if running.

When the UE:

1)	deregisters over an access type;

2)	successfully registers in a new registration area over an access type;

3)	enters state 5GMM-DEREGISTERED or 5GMM-REGISTERED following an unsuccessful registration in a new registration area over an access type; or

4)	performs inter-system change from N1 mode to S1 mode and the UE successfully completes tracking area update procedure;

the rejected NSSAI for the current registration area corresponding to the access type shall be deleted;

…

9.3.1.4.3	Test description

9.3.1.4.3.1	Pre test conditions

System Simulator:

-	2 cells

-	NGC Cell A as defined in TS 38.508-1 [4] Table 6.3.2.2-1. System information combination NR-6 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2.

-	E-UTRA Cell A as defined in TS 36.508 [7] Table 6.3.2.2-1. System information combination 31 as defined in TS 36.508 [7], subclause 4.4.3.1.1.

-	The SS configures the NGC Cell A as the "Serving cell" and E-UTRA Cell A as "Non-suitable "Off" cell".

UE:

-	UE is previously registered on NGC Cell A using default message contents according to TS 38.508-1 [4];

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.3.1.4.3.2	Test procedure sequence

Table 9.3.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits a REGISTRATION REQUEST message including ER-NSSAI bit | --> | REGISTRATION REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 12 | The SS transmits a REGISTRATION ACCEPT message including S-NSSAI=1 with cause value “S-NSSAI not available due to maximum number of UEs reached” and S-NSSAI=2 with cause value “S-NSSAI not available in the current PLMN or SNPN” in extended rejected NSSAI without back-off timer. Note: Now UE should start timer T3526 associated with S-NSSAI=1 | <-- | REGISTRATION ACCEPT | - | - |
| 13 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| - | EXCEPTION: Step 14a1 is performed if pc_noOf_PDUsSameConnection > 0. | - | - | - | - |
| 14a1 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| 15 | SS releases the RRC connection | - | - | - | - |
| 16 | The SS configures: - NGC Cell A as "Non-Suitable "Off" cell". - E-UTRA Cell A as "Serving cell" | - | - | - | - |
| 17 | Generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 take place and UE is camped on E-UTRA Cell A. | - | - | - | - |
| 18 | The SS configures: - NGC Cell A as "Serving cell". - E-UTRA Cell A as "Non-Suitable "Off" cell" | - | - | - | - |
| 19 | Generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 take place and UE is camped on NGC Cell A. | - | - | - | - |
| 20 | Check: Are S-NSSAI=1 and S-NSSAI=2 in the Rejected NSSAI list PLMN using AT/MMI command (+C5GNSSAIRDP)? | - | - | 1 | P |
| 21 | Void |  |  |  |  |
| 22 | Void |  |  |  |  |
| 23 | The SS configures: - NGC Cell A as "Non-Suitable "Off" cell". - E-UTRA Cell A as "Serving cell" | - | - | - | - |
| 24 | Generic test procedure with 'connected without release' in TS 38.508-1 [4] Table 4.9.7.2.2-1 take place and UE is camped on E-UTRA Cell A. | - | - | - | - |
| 25 | Cause the UE to request establishment of additional PDN connectivity. | - | - | - | - |
| 26 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN. | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST | - | - |
| 27 | The SS transmits an RRCConnectionReconfiguration message including the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to establish the default EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| 28 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 29 | The UE transmits an ULInformationTransfer message including the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | 2 | P |
| 30 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease | - | - |
| 31 | The SS configures: - NGC Cell A as "Serving cell". - E-UTRA Cell A as "Non-Suitable "Off" cell" | - | - | - | - |
| 32 | Generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 take place and UE is camped on NGC Cell A. | - | - | 2 | P |
| 33 | Check: Are S-NSSAI=2 in the Rejected NSSAI list using AT/MMI command (+C5GNSSAIRDP)? | - | - | 2 | F |

9.3.1.4.3.3	Specific message contents

Table 9.3.1.4.3.3-1: REGISTRATION REQUEST (step 2, Table 9.3.1.4.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| 5GMM capability |  |  |  |  |
| ER-NSSAI |  | ‘1’B | Extended rejected NSSAI supported |  |

Table 9.3.1.4.3.3-2: REGISTRATION ACCEPT (step 12, Table 9.3.1.4.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result |  |  |  |  |
| 5GS registration result value |  | '0001'B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| Length of NSSAI contents |  | 1 entry |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 3 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000011’B | SST value 3 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| Length of NSSAI contents |  | 2 entries |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | SST value 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Extended rejected NSSAI |  |  |  |  |
| Length of Extended rejected NSSAI contents |  | 1 entry |  |  |
| Partial extended rejected NSSAI list |  |  | entry 1 |  |
| Type of list |  | ‘000’B | S-NSSAIs without back-off timer |  |
| Number of elements |  | ‘0001’B | 2 elements |  |
| Rejected S-NSSAI 1 |  |  | S-NSSAI value 1 |  |
| Length of rejected S-NSSAI |  | ‘00000001’B | SST |  |
| Cause value |  | ‘0011’B | S-NSSAI not available due to maximum number of UEs reached |  |
| SST |  | ‘00000001’B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped HPLMN SST |  | Not present |  |  |
| Mapped HPLMN SD |  | Not present |  |  |
| Rejected S-NSSAI 2 |  |  | S-NSSAI value 2 |  |
| Length of rejected S-NSSAI |  | ‘00000001’B | SST |  |
| Cause value |  | ‘0000’B | S-NSSAI not available in the current PLMN or SNPN |  |
| SST |  | ‘00000010’B | SST value 2 |  |
| SD |  | Not Present |  |  |
| Mapped HPLMN SST |  | Not present |  |  |
| Mapped HPLMN SD |  | Not present |  |  |

Table 9.3.1.4.3.3-3: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 27, Table 9.3.1.4.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| Protocol configuration options |  |  |  |  |
| Container ID n |  | 001BH | S-NSSAI |  |
| Length of container ID n contents |  | Any allowed value |  |  |
| Container ID n contents |  | SNSSAI=2 and PLMN ID of NGC Cell A |  |  |