3GPP TSG WG RAN5 Meeting #100	R5-234118r1

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1638 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 6.5B.3.3.1 for exception and non-exception requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 6.5B.3.3.1 has led to different understandings on how to perform the test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test applicability in Minimum conformance requirements section, test procedure and test requirements were updated to clarify the correct understanding of 6.5B.3.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be different understandings on how to perform the test of 6.5B.3.3.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion in R5-234113. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Conditions about when no exception requirements shall be tested are removed as it is already included in the general section 4.5.0. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.5B.3.3.1	General spurious emissions for Inter-band EN-DC within FR1

6.5B.3.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5B.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting inter-band EN-DC.

6.5B.3.3.1.3	Minimum conformance requirements

The general spurious emissions requirements specified in subclause 6.6.3.1 of TS 36.101 [5], subclause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both uplink carriers active. Limits on configured maximum output power for the uplink according to subclause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band.  Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.1.

For exception requirements applicable for both NR and LTE, LTE anchor agnostic approach is not applied.

For no exception requirements applicable to NR or E-UTRA, LTE anchor agnostic approach is applied.

6.5B.3.3.1.4	Test description

6.5B.3.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, DC configuration specified in Table 5.5B.4.1-1 and test channel bandwidths specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1, and sub-carrier spacing based on NR operating bands specified in TS 38.521-1 [8] clause 5.3. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 6.5B.3.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A, clause A.2.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A, clause A.2.2 for E-UTRA RMC for FDD, and TS 38.521-1 [8] Annex A, clause A.2 for NR RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.3.3.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  | 5 MHz for E-UTRA CC and Lowest for NR CC Highest for E-UTRA CC and Highest for NR CC |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Left |
| 2 |  | QPSK | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Right |
| 3 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 3: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 4: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B.4. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

4.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B and TS 38.521-1 [8] Annex B for E-UTRA link and NR link respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.3.3.1.4.3.

8.	For both E-UTRA and NR UL uplink carriers active when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.3.1.4.2	Test procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.3.1.5-1, which is only test spurious emission at the specific frequency range where second and third order intermodulation products generated by the two transmitted operating bands can occur. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

5.	For UE operating on EN-DC configuration with Band n41, redo the test for frequency range 1 GHz  f < 12.75 GHz with the message content in step 7 of initial conditions with exceptions defined in Table 6.5B.3.3.1.4.3-4.

No exception requirements are tested according to LTE anchor agnostic below.

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.3.3.1.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.3.1.4.3-1: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | Sa2 |  |  |
| specialSubframePatterns | Ssp7 |  |  |
| } |  |  |  |

Table 6.5B3.3.1.4.3-1a: Void

Table 6.5B.3.3.1.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.3.3.1.4.3-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Exception for step 5 in test procedure:

Table 6.5B.3.3.1.4.3-4: Message contents

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.5B.3.3.1.5	Test Requirement

For exception requirements (with both LTE carrier and NR carrier active) of EN-DC configurations listed in Table 6.5B.3.3.1.5-1, the corresponding test requirements of the same table apply.

For exception requirements (with both LTE carrier and NR carrier active) of EN-DC configurations not listed in Table 6.5B.3.3.1.5-1, the test requirements in Table 6.5.3.1.5-1 in TS 38.521-1 [8] shall apply.

For no exception requirements (LTE anchor agnostic approach) of all EN-DC configurations, test requirements for NR carrier are the same as Table 6.5.3.1.5-1 in TS 38.521-1 [8].

Table 6.5B.3.3.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level |  | Measurement Bandwidth |  | Note |
| --- | --- | --- | --- | --- | --- |
| Test requirements for DC_1A_n3A configuration |  |  |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz | -36 dBm |  | 100 kHz |  |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5745 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_1A_n5A configuration |  |  |  |  |  |
| 222 MHz ≤ f ≤ 332 MHz |  | -36 dBm | 100 kHz |  |  |
| 1071 MHz ≤ f ≤ 1156 MHz 2744 MHz ≤ f ≤ 2829 MHz 2991 MHz ≤ f ≤ 3136 MHz 3568 MHz ≤ f ≤ 3678 MHz 4664 MHz ≤ f ≤ 4809 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_1A_n7A configuration |  |  |  |  |  |
| 520 MHz ≤ f ≤ 650 MHz |  | -36 dBm | 100 kHz |  |  |
| 1270 MHz ≤ f ≤ 1460 MHz 3020 MHz ≤ f ≤ 3220 MHz 4420 MHz ≤ f ≤ 4550 MHz 6340 MHz ≤ f ≤ 6530 MHz 6920 MHz ≤ f ≤ 7120 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_1A_n8A configuration |  |  |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz |  | -36 dBm | 100 kHz |  |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_1A_n28A configuration |  |  |  |  |  |
| 424 MHz ≤ f ≤ 574 MHz |  | -36 dBm | 100 kHz |  |  |
| 1172 MHz ≤ f ≤ 1277 MHz 2623 MHz ≤ f ≤ 2728 MHz 3092 MHz ≤ f ≤ 3257 MHz 3326 MHz ≤ f ≤ 3476 MHz 4543 MHz ≤ f ≤ 4708 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_1A_n41A configuration |  |  |  |  |  |
| 516 MHz ≤ f ≤ 770 MHz |  | -36 dBm | 100 kHz |  |  |
| 1150 MHz ≤ f ≤ 1464 MHz 3012 MHz ≤ f ≤ 3460 MHz 4416 MHz ≤ f ≤ 4670 MHz 6336 MHz ≤ f ≤ 6650 MHz 6912 MHz ≤ f ≤ 7360 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_1A_n77A Configuration |  |  |  |  |  |
| 360 MHz ≤ f ≤ 660 MHz | -36 dBm |  | 100 kHz |  |  |
| 1320 MHz ≤ f ≤ 2280 MHz 4620 MHz ≤ f ≤ 6480 MHz 7140 MHz ≤ f ≤ 8160 MHz 8520 MHz ≤ f ≤ 10380 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_1A_n78A Configuration |  |  |  |  |  |
| 40 MHz ≤ f ≤ 660 MHz | -36 dBm |  | 100 kHz |  |  |
| 1320 MHz ≤ f ≤ 1880 MHz 4620 MHz ≤ f ≤ 5780 MHz 7140 MHz ≤ f ≤ 7760 MHz 8520 MHz ≤ f ≤ 9580 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_1A_n79A Configuration |  |  |  |  |  |
| 440 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1160 MHz 2420 MHz ≤ f ≤ 3080 MHz 6320 MHz ≤ f ≤ 8080 MHz 8240 MHz ≤ f ≤ 8960 MHz 10720 MHz ≤ f ≤ 11980 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_2A_n5A Configuration |  |  |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz |  | -36 dBm | 100 kHz |  |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_2A_n41A Configuration |  |  |  |  |  |
| 586 MHz ≤ f ≤ 840 MHz |  | -36 dBm | 100 kHz |  |  |
| 1010 MHz ≤ f ≤ 1324 MHz 3082 MHz ≤ f ≤ 3530 MHz 4346 MHz ≤ f ≤ 4600 MHz 6196 MHz ≤ f ≤ 6510 MHz 6842 MHz ≤ f ≤ 7290 MHz |  | -30 dBm | 1 MHz | 2 |  |
|  |  | -25 dBm | 1 MHz | 1 |  |
| Test requirements for DC_2A_n71A Configuration |  |  |  |  |  |
| 454 MHz ≤ f ≤ 584 MHz |  | -36 dBm | 100 kHz |  |  |
| 1152 MHz ≤ f ≤ 1247 MHz 2513 MHz ≤ f ≤ 2608 MHz 3002 MHz ≤ f ≤ 3157 MHz 3176 MHz ≤ f ≤ 3306 MHz 4363 MHz ≤ f ≤ 4518 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_2A_n77A Configuration |  |  |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz |  | -36 dBm | 100 kHz |  |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_3A_n1A Configuration |  |  |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz |  | -36 dBm | 100 kHz |  |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5745 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_3A_n5A Configuration |  |  |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 137 MHz 861 MHz ≤ f ≤ 961 MHz | -36 dBm |  | 100 kHz |  |  |
| 2534 MHz ≤ f ≤ 2746 MHz 3358 MHz ≤ f ≤ 3483 MHz 4244 MHz ≤ f ≤ 4419 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_3A_n7A Configuration |  |  |  |  |  |
| 715 MHz ≤ f < 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1070 MHz 3215 MHz ≤ f ≤ 3430 MHz 4210 MHz ≤ f ≤ 4355 MHz 5920 MHz ≤ f ≤ 6140 MHz 6710 MHz ≤ f ≤ 6925 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_3A_n8A Configuration |  |  |  |  |  |
| 25 MHz ≤ f ≤ 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz | -36 dBm |  | 100 kHz |  |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_3A_n28A Configuration |  |  |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_3A_n41A Configuration |  |  |  |  |  |
| 711 MHz ≤ f < 1000 MHz |  | -36 dBm | 100 kHz |  |  |
| 1000MHz ≤ f ≤ 1074 MHz 3207MHz ≤ f ≤ 3670 MHz 4206MHz ≤ f ≤ 4475 MHz 5916MHz ≤ f ≤ 6260 MHz 6702MHz ≤ f ≤ 7165 MHz |  | -30 dBm | 1 MHz | 2 |  |
|  |  | -25 dBm | 1 MHz | 1 |  |
| Test requirements for DC_3A_n77A Configuration |  |  |  |  |  |
| 270 MHz ≤ f ≤ 780 MHz | -36 dBm |  | 100 kHz |  |  |
| 1515 MHz ≤ f ≤ 2490 MHz 4815 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7770 MHz 8310 MHz ≤ f ≤ 10185 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_3A_n78A Configuration |  |  |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz |  | -36 dBm | 100 kHz |  |  |
| 1515 MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_3A_n79A Configuration |  |  |  |  |  |
| 830 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1580 MHz 2615 MHz ≤ f ≤ 3290 MHz 6110 MHz ≤ f ≤ 6785 MHz 7015 MHz ≤ f ≤ 8570 MHz 10510 MHz ≤ f ≤ 11785 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_5A_n2A Configuration |  |  |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz | -36 dBm |  | 100 kHz |  |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_5A_n66A Configuration |  |  |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz | -36 dBm |  | 100 kHz |  |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_5A_n77A Configuration |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 9249 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_5A_n78A Configuration |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 2152 MHz 2451 MHz ≤ f ≤ 2976 MHz 4124 MHz ≤ f ≤ 4649 MHz 4948 MHz ≤ f ≤ 5498 MHz 5751 MHz ≤ f ≤ 6776 MHz 7424 MHz ≤ f ≤ 8449 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_7A_n1A Configuration |  |  |  |  |  |
| 520 MHz ≤ f ≤ 650 MHz |  | -36 dBm | 100 kHz |  |  |
| 1270 MHz ≤ f ≤ 1460 MHz 3020 MHz ≤ f ≤ 3220 MHz 4420 MHz ≤ f ≤ 4550 MHz 6340 MHz ≤ f ≤ 6530 MHz 6920 MHz ≤ f ≤ 7120 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_7A_n3A Configuration |  |  |  |  |  |
| 715 MHz ≤ f < 1000 MHz |  | -36 dBm | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1070 MHz 3215 MHz ≤ f ≤ 3430 MHz 4210 MHz ≤ f ≤ 4355 MHz 5920 MHz ≤ f ≤ 6140 MHz 6710 MHz ≤ f ≤ 6925 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_7A_n5A Configuration |  |  |  |  |  |
| 802 MHz ≤ f ≤ 922 MHz | -36 dBm |  | 100 kHz |  |  |
| 1651 MHz ≤ f ≤ 1746 MHz 3324 MHz ≤ f ≤ 3419 MHz 4148 MHz ≤ f ≤ 4316 MHz 5824 MHz ≤ f ≤ 5989 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_7A_n8A Configuration |  |  |  |  |  |
| 670 MHz ≤ f ≤ 810 MHz | -36 dBm |  | 100 kHz |  |  |
| 1585 MHz ≤ f ≤ 1690 MHz 3380 MHz ≤ f ≤ 3485 MHz 4085 MHz ≤ f ≤ 4400 MHz 5880 MHz ≤ f ≤ 6055 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_7A_n28A Configuration |  |  |  |  |  |
| 1004 MHz ≤ f ≤ 1164 MHz 1752 MHz ≤ f ≤ 1867 MHz 3203 MHz ≤ f ≤ 3318 MHz 3906 MHz ≤ f ≤ 4066 MHz 4252 MHz ≤ f ≤ 4437 MHz 5703 MHz ≤ f ≤ 5888 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_7A_n78A Configuration |  |  |  |  |  |
| 730 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1840 MHz 4030 MHz ≤ f ≤ 5100 MHz 5800 MHz ≤ f ≤ 6370 MHz 8300 MHz ≤ f ≤ 8940 MHz 9100 MHz ≤ f ≤ 10170 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_8A_n1A Configuration |  |  |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz | -36 dBm |  | 100 kHz |  |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_8A_n3A Configuration |  |  |  |  |  |
| 25 MHz ≤ f < 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz | -36 dBm |  | 100 kHz |  |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_8A_n20A Configuration |  |  |  |  |  |
| 18 MHz ≤ f < 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 83 MHz 749 MHz ≤ f ≤ 844 MHz 898 MHz ≤ f ≤ 998 MHz | -36 dBm |  | 100 kHz |  |  |
| 1712 MHz ≤ f ≤ 1777 MHz 2544 MHz ≤ f ≤ 2692 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_8A_n28A Configuration |  |  |  |  |  |
| 132 MHz ≤ f ≤ 212 MHz 491 MHz ≤ f ≤ 616 MHz | -36 dBm |  | 100 kHz |  |  |
| 1012 MHz ≤ f ≤ 1127 MHz 1583 MHz ≤ f ≤ 1663 MHz 2286 MHz ≤ f ≤ 2411 MHz 2463 MHz ≤ f ≤ 2578 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_8A_n41A Configuration |  |  |  |  |  |
| 666 MHz ≤ f ≤ 930 MHz |  | -36 dBm | 100 kHz |  |  |
| 1581MHz ≤ f ≤ 1810 MHz 3376MHz ≤ f ≤ 3605 MHz 4077MHz ≤ f ≤ 4520 MHz 5872MHz ≤ f ≤ 6295 MHz |  | -30 dBm | 1 MHz | 2 |  |
|  |  | -25 dBm | 1 MHz | 1 |  |
| Test requirements for DC_8A_n77A Configuration |  |  |  |  |  |
| 1470 MHz ≤ f ≤ 3320 MHz 4180 MHz ≤ f ≤ 9315 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_8A_n78A Configuration |  |  |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤ 4715 MHz 5060 MHz ≤ f ≤ 5630 MHz 5685 MHz ≤ f ≤ 6720 MHz 7480 MHz ≤ f ≤ 8515 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_11A_n77A Configuration |  |  |  |  |  |
| 404.2 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1344.2 MHz 1852.1 MHz ≤ f ≤ 2772.1 MHz 4727.9 MHz ≤ f ≤ 7095.8 MHz 8027.9 MHz ≤ f ≤ 9847.9 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_11A_n78A Configuration |  |  |  |  |  |
| 404.2 MHz ≤ f ≤ 944.2 MHz | -36 dBm |  | 100 kHz |  |  |
| 1852.1 MHz ≤ f ≤ 2372.1 MHz 4727.9 MHz ≤ f ≤ 6695.8 MHz 8027.9 MHz ≤ f ≤ 9047.9 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_11A_n79A Configuration |  |  |  |  |  |
| 1504.2 MHz ≤ f ≤ 2144.2 MHz 2952.1 MHz ≤ f ≤ 3572.1 MHz 5827.9 MHz ≤ f ≤ 6447.9 MHz 7255.8 MHz ≤ f ≤ 8572.1 MHz 10227.9 MHz ≤ f ≤ 11447.9 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_12A_n2A Configuration |  |  |  |  |  |
| 418 MHz ≤ f ≤ 512 MHz | -36 dBm |  | 100 kHz |  |  |
| 1134 MHz ≤ f ≤ 1211 MHz 2549 MHz ≤ f ≤ 2626 MHz 2984 MHz ≤ f ≤ 3121 MHz 3248 MHz ≤ f ≤ 3342 MHz 4399 MHz ≤ f ≤ 4536 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_12A_n66A Configuration |  |  |  |  |  |
| 278 MHz ≤ f ≤ 382 MHz 994 MHz ≤ f < 1000 MHz |  | -36 dBm | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1081 MHz 2409 MHz ≤ f ≤ 2496 MHz 2704 MHz ≤ f ≤ 2861 MHz 3108 MHz ≤ f ≤ 3212 MHz 4119 MHz ≤ f ≤ 4276 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_12A_n78A Configuration |  |  |  |  |  |
| 1868 MHz ≤ f ≤ 2402 MHz 2584 MHz ≤ f ≤ 3101 MHz 3999 MHz ≤ f ≤ 4516 MHz 4698 MHz ≤ f ≤ 5232 MHz 5884 MHz ≤ f ≤ 6901 MHz 7299 MHz ≤ f ≤ 8316 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_13A_n2A Configuration |  |  |  |  |  |
| 276 MHz ≤ f ≤ 356 MHz | -36 dBm |  | 100 kHz |  |  |
| 1063 MHz ≤ f ≤ 1133 MHz 2627 MHz ≤ f ≤ 2697 MHz 2913 MHz ≤ f ≤ 3043 MHz 3404 MHz ≤ f ≤ 3484 MHz 4477 MHz ≤ f ≤ 4607 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_13A_n66A Configuration |  |  |  |  |  |
| 136 MHz ≤ f ≤ 226 MHz 923 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1003 MHz 2487 MHz ≤ f ≤ 2567 MHz 2633 MHz ≤ f ≤ 2783 MHz 3264 MHz ≤ f ≤ 3354 MHz 4197 MHz ≤ f ≤ 4347 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_13A_n77A Configuration |  |  |  |  |  |
| 1726 MHz ≤ f ≤ 3423 MHz 4077 MHz ≤ f ≤ 5774 MHz 5813 MHz ≤ f ≤ 9187 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_14A_n2A Configuration |  |  |  |  |  |
| 254 MHz ≤ f ≤ 334 MHz | -36 dBm |  | 100 kHz |  |  |
| 1052 MHz ≤ f ≤ 1122 MHz 2638 MHz ≤ f ≤ 2708 MHz 2902 MHz ≤ f ≤ 3032 MHz 3426 MHz ≤ f ≤ 3506 MHz 4488 MHz ≤ f ≤ 4618 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_14A_n66A Configuration |  |  |  |  |  |
| 114 MHz ≤ f ≤ 204 MHz 912 MHz ≤ f ≤ 992 MHz | -36 dBm |  | 100 kHz |  |  |
| 2498 MHz ≤ f ≤ 2578 MHz 2622 MHz ≤ f ≤ 2772 MHz 3286 MHz ≤ f ≤ 3376 MHz 4208 MHz ≤ f ≤ 4358 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_18A_n77A Configuration |  |  |  |  |  |
| 1640 MHz ≤ f ≤ 3385 MHz 4115 MHz ≤ f ≤ 5860 MHz 5770 MHz ≤ f ≤ 9230 MHz | -30dBm |  | 1MHz |  |  |
| Test requirements for DC_18A_n78A Configuration |  |  |  |  |  |
| 1640 MHz ≤ f ≤ 2170 MHz 2470 MHz ≤ f ≤ 2985 MHz 4115 MHz ≤ f ≤ 4630 MHz 4930 MHz ≤ f ≤ 5460 MHz 5770 MHz ≤ f ≤ 6785 MHz 7415 MHz ≤ f ≤ 8430 MHz | -30dBm |  | 1MHz |  |  |
| Test requirements for DC_19A_n1A Configuration |  |  |  |  |  |
| 230 MHz <= f <= 320 MHz | -36 dBm |  | 100 kHz |  |  |
| 1075 MHz <= f <= 1150 MHz 2750 MHz <= f <= 2825 MHz 2995 MHz <= f <= 3130 MHz 3580 MHz <= f <= 3670 MHz 4670 MHz <= f <= 4805 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_19A_n77A Configuration |  |  |  |  |  |
| 1610 MHz ≤ f ≤ 3370 MHz 4130 MHz ≤ f ≤ 5890 MHz 5755 MHz ≤ f ≤ 9245 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_19A_n78A Configuration |  |  |  |  |  |
| 1610 MHz ≤ f ≤ 2140 MHz 2455 MHz ≤ f ≤ 2970 MHz 4130 MHz ≤ f ≤ 4645 MHz 4960 MHz ≤ f ≤ 5490 MHz 5755 MHz ≤ f ≤ 6770 MHz 7430 MHz ≤ f ≤ 8445 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_19A_n79A Configuration |  |  |  |  |  |
| 2710 MHz ≤ f ≤ 3340 MHz 3555 MHz ≤ f ≤ 4170 MHz 5230 MHz ≤ f ≤ 5845 MHz 6060 MHz ≤ f ≤ 6690 MHz 7955 MHz ≤ f ≤ 9170 MHz 9630 MHz ≤ f ≤ 10845 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_20A_n1A Configuration |  |  |  |  |  |
| 196 MHz ≤ f ≤ 316 MHz | -36 dBm |  | 100 kHz |  |  |
| 1058 MHz ≤ f ≤ 1148 MHz 2752 MHz ≤ f ≤ 2842 MHz 2978 MHz ≤ f ≤ 3128 MHz 3584 MHz ≤ f ≤ 3704 MHz 4672 MHz ≤ f ≤ 4822 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_20A_n3A Configuration |  |  |  |  |  |
| 14 MHz ≤ f < 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 121 MHz 848 MHz ≤ f ≤ 953 MHz | -36 dBm |  | 100 kHz |  |  |
| 2542 MHz ≤ f ≤ 2738 MHz 3374 MHz ≤ f ≤ 3509 MHz 4252 MHz ≤ f ≤ 4432 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_20A_n7A Configuration |  |  |  |  |  |
| 776 MHz ≤ f ≤ 906 MHz | -36 dBm |  | 100 kHz |  |  |
| 1638 MHz ≤ f ≤ 1738 MHz 3332 MHz ≤ f ≤ 3432 MHz 4138 MHz ≤ f ≤ 4308 MHz 5832 MHz ≤ f ≤ 6002 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_20A_n8A Configuration |  |  |  |  |  |
| 18 MHz ≤ f < 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 83 MHz 749 MHz ≤ f ≤ 844 MHz 898 MHz ≤ f ≤ 998 MHz | -36 dBm |  | 100 kHz |  |  |
| 1712 MHz ≤ f ≤ 1777 MHz 2544 MHz ≤ f ≤ 2692 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_20A_n28A Configuration |  |  |  |  |  |
| 84 MHz ≤ f ≤ 159 MHz 544 MHz ≤ f ≤ 664 MHz 916 MHz ≤ f ≤ 1000 MHz |  | -36 dBm | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1021 MHz 1535 MHz ≤ f ≤ 1610 MHz 2238 MHz ≤ f ≤ 2358 MHz 2367 MHz ≤ f ≤ 2472 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_20A_n78A Configuration |  |  |  |  |  |
| 1576 MHz ≤ f ≤ 2136 MHz 2438 MHz ≤ f ≤ 2968 MHz 4132 MHz ≤ f ≤ 4662 MHz 4964 MHz ≤ f ≤ 5524 MHz 5738 MHz ≤ f ≤ 6768 MHz 7432 MHz ≤ f ≤ 8462 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_21A_n1A Configuration |  |  |  |  |  |
| 457.1 MHz ≤ f ≤ 532.1 MHz 915.8 MHz ≤ f ≤ 1000 MHz |  | -36 dBm | 100 kHz |  |  |
| 2377.1 MHz ≤ f ≤ 2512.1 MHz 3367.9 MHz ≤ f ≤ 3442.9 MHz 4815.8 MHz ≤ f ≤ 4905.8 MHz 5287.9 MHz ≤ f ≤ 5422.9 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_21A_n28A Configuration |  |  |  |  |  |
| 6.9 MHz ≤ f ≤ 28.1 MHz |  | -36 dBm | 10 kHz |  |  |
| 709.9 MHz ≤ f ≤ 734.9 MHz |  | -36 dBm | 100 kHz |  |  |
| 2157.8 MHz ≤ f ≤ 2200.9 MHz 2903.9 MHz ≤ f ≤ 2938.9 MHz 3623.8 MHz ≤ f ≤ 3663.8 MHz |  | -30 dBm | 1 MHz | 4 |  |
| Test requirements for DC_21A_n77A Configuration |  |  |  |  |  |
| 374.2 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1304.2 MHz 1837.1 MHz ≤ f ≤ 2752.1 MHz 4747.9 MHz ≤ f ≤ 7125.8 MHz 8047.9 MHz ≤ f ≤ 9862.9 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_21A_n78A Configuration |  |  |  |  |  |
| 374.2 MHz ≤ f ≤ 904.2 MHz | -36 dBm |  | 100 kHz |  |  |
| 1837.1 MHz ≤ f ≤ 2352.1 MHz 4747.9 MHz ≤ f ≤ 6152.1 MHz 6195.8 MHz ≤ f ≤ 6725.8 MHz 8047.9 MHz ≤ f ≤ 9062.9 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_21A_n79A Configuration |  |  |  |  |  |
| 1474.2 MHz ≤ f ≤ 2104.2 MHz 2937.1 MHz ≤ f ≤ 3552.1 MHz 5847.9 MHz ≤ f ≤ 6462.9 MHz 7295.8 MHz ≤ f ≤ 8552.1 MHz 10247.9 MHz ≤ f ≤ 11462.9 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_25A_n41A Configuration |  |  |  |  |  |
| 581 MHz ≤ f ≤ 840 MHz | -36 dBm |  | 100 kHz |  |  |
| 1010 MHz ≤ f ≤ 1334 MHz 3077 MHz ≤ f ≤ 3530 MHz 4346 MHz ≤ f ≤ 4605 MHz 6196 MHz ≤ f ≤ 6520 MHz 6842 MHz ≤ f ≤ 7295 MHz | -30 dBm |  | 1 MHz |  | 2 |
|  | -25 dBm |  | 1 MHz |  | 1 |
| Test requirements for DC_26A_n41A Configuration |  |  |  |  |  |
| 798 MHz ≤ f < 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1062 MHz 1647 MHz ≤ f ≤ 1876 MHz 3310 MHz ≤ f ≤ 3539 MHz 4124 MHz ≤ f ≤ 4566 MHz 5806 MHz ≤ f ≤ 6229 MHz | -30 dBm |  | 1 MHz |  | 2 |
|  | -25 dBm |  | 1 MHz |  | 1 |
| Test requirements for DC_26A_n77A Configuration |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 3386 MHz 4114 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_26A_n78A Configuration |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 2172 MHz 2451 MHz ≤ f ≤ 2986 MHz 4114 MHz ≤ f ≤ 4649 MHz 4928 MHz ≤ f ≤ 5498 MHz 5751 MHz ≤ f ≤ 6786 MHz 7414 MHz ≤ f ≤ 8449 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_26A_n79A Configuration |  |  |  |  |  |
| 2702 MHz ≤ f ≤ 3372 MHz 3551 MHz ≤ f ≤ 4186 MHz 5214 MHz ≤ f ≤ 5849 MHz 6028 MHz ≤ f ≤ 6698 MHz 7951 MHz ≤ f ≤ 9186 MHz 9614 MHz ≤ f ≤ 10849 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_28A_n3A Configuration |  |  |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f < 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_28A_n5A Configuration |  |  |  |  |  |
| 76 MHz ≤ f ≤ 146 MHz 557 MHz ≤ f ≤ 672 MHz 900 MHz ≤ f ≤ 995 MHz | -36 dBm |  | 100 kHz |  |  |
| 1527 MHz ≤ f ≤ 1597 MHz 2230 MHz ≤ f ≤ 2345 MHz 2351 MHz ≤ f ≤ 2446 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_28A_n7A Configuration |  |  |  |  |  |
| 1004 MHz ≤ f ≤ 1164 MHz 1752 MHz ≤ f ≤ 1867 MHz 3203 MHz ≤ f ≤ 3318 MHz 3906 MHz ≤ f ≤ 4066 MHz 4252 MHz ≤ f ≤ 4437 MHz 5703 MHz ≤ f ≤ 5888 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_28A_n77A Configuration |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 3497 MHz 4003 MHz ≤ f ≤ 5696 MHz 5852 MHz ≤ f ≤ 9148 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_28A_n78A Configuration |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_28A_n79A Configuration |  |  |  |  |  |
| 2904 MHz ≤ f ≤ 3594 MHz 3652 MHz ≤ f ≤ 4297 MHz 5103 MHz ≤ f ≤ 5748 MHz 5806 MHz ≤ f ≤ 6496 MHz 8052 MHz ≤ f ≤ 9297 MHz 9503 MHz ≤ f ≤ 10748 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_30A_n5A Configuration |  |  |  |  |  |
| 607 MHz ≤ f ≤ 667 MHz | -36 dBm |  | 100 kHz |  |  |
| 1456 MHz ≤ f ≤ 1491 MHz 3129 MHz ≤ f ≤ 3164 MHz 3761 MHz ≤ f ≤ 3806 MHz 3953 MHz ≤ f ≤ 4013 MHz 5434 MHz ≤ f ≤ 5479 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_39A_n41A Configuration |  |  |  |  |  |
| 576 MHz ≤ f < 810 MHz | -36 dBm |  | 100 kHz |  |  |
| 1070MHz ≤ f ≤ 1344 MHz 3072MHz ≤ f ≤ 3500 MHz 4376MHz ≤ f ≤ 4610 MHz 6256MHz ≤ f ≤ 6530 MHz 6872MHz ≤ f ≤ 7300 MHz | -30 dBm |  | 1 MHz |  | 2 |
|  | -25 dBm |  | 1 MHz |  | 1 |
| Test requirements for DC_39A_n79A Configuration |  |  |  |  |  |
| 560 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1240 MHz 2480 MHz ≤ f ≤ 3120 MHz 6280 MHz ≤ f ≤ 8120 MHz 8160 MHz ≤ f ≤ 8840 MHz 10680 MHz ≤ f ≤ 11920 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_40A_n1A Configuration |  |  |  |  |  |
| 320 MHz ≤ f ≤ 480 MHz |  | -36 dBm | 100 kHz |  |  |
| 1440 MHz ≤ f ≤ 1660 MHz 2620 MHz ≤ f ≤ 2880 MHz 4220 MHz ≤ f ≤ 4380 MHz 6140 MHz ≤ f ≤ 6360 MHz 6520 MHz ≤ f ≤ 6780 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_40A_n41A Configuration |  |  |  |  |  |
| 96 MHz ≤ f < 390 MHz | -36 dBm |  | 100 kHz |  |  |
| 1910 MHz ≤ f ≤ 2304 MHz 2592 MHz ≤ f ≤ 3080 MHz 4796 MHz ≤ f ≤ 5090 MHz 7096 MHz ≤ f ≤ 7780 MHz | -30 dBm |  | 1 MHz |  | 2 |
|  | -25 dBm |  | 1 MHz |  | 1 |
| Test requirements for DC_40A_n78A Configuration |  |  |  |  |  |
| 800 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1500 MHz 4200 MHz ≤ f ≤ 5300 MHz 5600 MHz ≤ f ≤ 6200 MHz 7900 MHz ≤ f ≤ 8600 MHz 8900 MHz ≤ f ≤ 10000 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_40A_n79A Configuration |  |  |  |  |  |
| 2000 MHz ≤ f ≤ 2700 MHz 6400 MHz ≤ f ≤ 7700 MHz 9000 MHz ≤ f ≤ 9800 MHz 11100 MHz ≤ f ≤ 12400 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_41A_n28A Configuration |  |  |  |  |  |
| 1000 MHz ≤ f ≤ 1284 MHz 1748 MHz ≤ f ≤ 1987 MHz 3199 MHz ≤ f ≤ 3438 MHz 3902 MHz ≤ f ≤ 4186 MHz 4244 MHz ≤ f ≤ 4677 MHz 5695 MHz ≤ f ≤ 6128 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_41A_n77A Configuration |  |  |  |  |  |
| 610 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 2080 MHz 3910 MHz ≤ f ≤ 6890 MHz 8292 MHz ≤ f ≤ 11090 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_41A_n78A Configuration |  |  |  |  |  |
| 610 MHz ≤ f ≤ 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 2080 MHz 3910 MHz ≤ f ≤ 5104 MHz 5796 MHz ≤ f ≤ 6490 MHz 8292 MHz ≤ f ≤ 10290 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_41A_n79A Configuration |  |  |  |  |  |
| 8 MHz ≤ f ≤ 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 980 MHz | -36 dBm |  | 100 kHz |  |  |
| 1710 MHz ≤ f ≤ 2504 MHz 6110 MHz ≤ f ≤ 7690 MHz 9392 MHz ≤ f ≤ 10380 MHz 11296 MHz ≤ f ≤ 12690 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_42A_n77A Configuration |  |  |  |  |  |
| 300 MHz ≤ f ≤ 800 MHz | -36 dBm |  | 100 kHz |  |  |
| 2600 MHz ≤ f ≤ 5000 MHz 6700 MHz ≤ f ≤ 7800 MHz 10000 MHz ≤ f ≤ 12000 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_48A_n5A Configuration |  |  |  |  |  |
| 1852 MHz ≤ f ≤ 2052 MHz 2701 MHz ≤ f ≤ 2876 MHz 4374 MHz ≤ f ≤ 4549 MHz 5198 MHz ≤ f ≤ 5398 MHz 6251 MHz ≤ f ≤ 6576 MHz 7924 MHz ≤ f ≤ 8249 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_48A_n66A Configuration |  |  |  |  |  |
| 10 MHz ≤ f < 30 MHz | -36 dBm |  | 10 KHz |  |  |
| 30 MHz ≤ f ≤ 280 MHz | -36 dBm |  | 100 KHz |  |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_66A_n5A Configuration |  |  |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz | -36 dBm |  | 10 kHz |  |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz | -36 dBm |  | 100 kHz |  |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_66A_n41A Configuration |  |  |  |  |  |
| 716 MHz ≤ f < 1000 MHz | -36 dBm |  | 100 kHz |  |  |
| 1000 MHz ≤ f ≤ 1064 MHz 3212 MHz ≤ f ≤ 3670 MHz 4206 MHz ≤ f ≤ 4470 MHz 5916 MHz ≤ f ≤ 6250 MHz 6702 MHz ≤ f ≤ 7160 MHz | -30 dBm |  | 1 MHz |  | 2 |
|  | -25 dBm |  | 1 MHz |  | 1 |
| Test requirements for DC_66A_n71A Configuration |  |  |  |  |  |
| 314 MHz ≤ f ≤ 454 MHz |  | -36 dBm | 100 kHz |  |  |
| 1102 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_66A_n77A Configuration |  |  |  |  |  |
| 260MHz ≤ f ≤ 780 MHz |  | -36 dBm | 100 kHz |  |  |
| 1520 MHz ≤ f ≤ 2490MHz 5010 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz |  | -30 dBm | 1 MHz |  |  |
| Test requirements for DC_66A_n78A Configuration |  |  |  |  |  |
| 260 MHz ≤ f ≤ 380 MHz | -36 dBm |  | 100 kHz |  |  |
| 1520 MHz ≤ f ≤ 2090 MHz 4820 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7360 MHz 8310 MHz ≤ f ≤ 9380 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_71A_n2A Configuration |  |  |  |  |  |
| 454 MHz ≤ f ≤ 584 MHz | -36 dBm |  | 100 kHz |  |  |
| 1152 MHz ≤ f ≤ 1247 MHz 2513 MHz ≤ f ≤ 2608 MHz 3002 MHz ≤ f ≤ 3157 MHz 3176 MHz ≤ f ≤ 3306 MHz 4363 MHz ≤ f ≤ 4518 MHz | -30 dBm |  | 1 MHz |  |  |
| Test requirements for DC_71A_n66A Configuration |  |  |  |  |  |
| 314 MHz ≤ f ≤ 454 MHz | -36 dBm |  | 100 kHz |  |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz | -30 dBm |  | 1 MHz |  |  |
| NOTE 1: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 36.101 [5] when NS_04 is signalled. NOTE 2: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |  |  |

6.5B.3.3.2	Spurious emission band UE co-existence for Inter-band within FR1

<< End of changes >>