3GPP TSG-RAN5 Meeting #100	R5-234311r1

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2845 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add radio configuration for RadioBearerConfig and SRB4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add an radio configuration for RadioBearerConfig and SRB4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A table for RadioBearerConfig and SRB4 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The workplan will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-234311r1. Split condition removed. |  |  |  |  |  |  |  |  |

<Start of first modified section>

–	RadioBearerConfig-SRB2-DRB (n, m)

Table 4.8.1-4: RadioBearerConfig-SRB2-DRB (n, m)

| Derivation Path: Table 4.6.3-132 and condition SRB2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n+m entries |  |  |
| DRB-ToAddMod[k=1..n+m] := SEQUENCE { |  | entry (1..n+m) |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config | SDAP-Config is configured according to internal TTCN mapping (Note 1) |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  | (1<k <= n) OR (k=1 AND NOT NR_split) |
|  | PDCP-Config with condition UM |  | k > n |
|  | PDCP-Config with condition NR_split |  | k=1 AND NR_split |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Configure a default DRB or configure QoS flow to DRB mapping rule. |  |  |  |

–	RadioBearerConfig-SRB2-SRB4-DRB (n, m)

Table 4.8.1-4A: RadioBearerConfig-SRB2-SRB4-DRB (n, m)

| Derivation Path: Table 4.6.3-132 and conditions SRB2 and SRB4. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n+m entries |  |  |
| DRB-ToAddMod[k=1..n+m] := SEQUENCE { |  | entry (1..n+m) |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config | SDAP-Config is configured according to internal TTCN mapping (Note 1) |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  | 1<k <= n |
|  | PDCP-Config with condition UM |  | k > n |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Configure a default DRB or configure QoS flow to DRB mapping rule. |  |  |  |

<End of modified section>