3GPP TSG-RAN5 Meeting #100	R5-234312r2

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2846 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add radio configuration for CellGroupConfig and SRB4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add an radio configuration for CellGroupConfig and SRB4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A table for CellGroupConfig and SRB4 has been added. Editorial correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The workplan will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-234312r1. Duplicated Table 4.8.1-2B removed. The header is corrected. The remark for number of entries is updated. 2nd revision R5-234312r2. The value/remark is updated. Message contents added. |  |  |  |  |  |  |  |  |

<Start of first modified section>

–	CellGroupConfig-DRB(n, m)

Table 4.8.1-2: CellGroupConfig-DRB(n, m)

| Derivation Path: Table 4.6.3-19 . |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | n+m entries | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  |
| RLC-BearerConfig[k, k=1..n] | RLC-BearerConfig with conditions AM and DRBj (with j=BID+k) | entry (1..n+1) | n>0 |
| RLC-BearerConfig[k, k=n+1..n+m] | RLC-BearerConfig with conditions UM and DRBj (with j=BID+k) | entry (n+1..n+m) | m>0 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| n>0 | n is greater than zero |
| m>0 | m is greater than zero |

–	CellGroupConfig-SRB3

Table 4.8.1-2A: CellGroupConfig-SRB3

| Derivation Path: Table 4.6.3-19 and condition EN-DC. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB3 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

–	CellGroupConfig-SRB2-DRB(n, m)

Table 4.8.1-2B: CellGroupConfig-SRB2-DRB(n, m)

| Derivation Path: Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n+m entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[k, k=2..n+1] | RLC-BearerConfig with conditions AM and DRBj | entry (2..n+1) j is allocated according to internal TTCN mapping | n>0 |
| ... |  | ... |  |
| RLC-BearerConfig[k, k=n+2..n+m+1] | RLC-BearerConfig with conditions UM and DRBj | entry (n+2..n+m+1) j is allocated according to internal TTCN mapping | m>0 |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

–	CellGroupConfig-SRB2-SRB4-DRB(n, m)

Table 4.8.1-2C: CellGroupConfig-SRB2-SRB4-DRB(n, m)

| Derivation Path: Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n+m entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB4 | entry 2 |  |
| RLC-BearerConfig[k, k=3..n+2] | RLC-BearerConfig WithSRB4 with conditions AM and DRBj | entry (3..n+2) j is allocated according to internal TTCN mapping. | n>0 |
| ... |  | ... |  |
| RLC-BearerConfig[k, k=n+3..n+m+2] | RLC-BearerConfig WithSRB4 with conditions UM and DRBj | entry (n+3..n+m+2) j is allocated according to internal TTCN mapping. | m>0 |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 4.8.1-2D: RLC-BearerConfigWithSRB4

| Derivation Path: Table 4.6.3-148 and condition DRBn |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | n+4 |  |  |
| } |  |  |  |

<End of modified section>