3GPP TSG-RAN5 Meeting #100	R5-234313r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5232 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Eutra RLC test case 7.2.3.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The prose has to be modified to introduce a step after Step 10 to state that SS transmits a STATUS PDU (ACK_SN = 1) as the UE, at Step 10, is sending a AMD PDU which has the Poll Bit = 1 requesting RLC STATUS PDU from the SS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add Step 11 for transmission of STATUS PDU (ACK_SN = 1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-234313r1 : Updated the Step 11 to indicate ACK SN = 2. |  |  |  |  |  |  |  |  |

#### 7.2.3.21	AM RLC / RLC re-establishment at RRC connection reconfiguration including mobilityControlInfo IE

7.2.3.21.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE is requested to perform a RRC Connection reconfiguration including mobilityControlInfo IE }

then { UE discards the remaining AMD PDUs; and discards all RLC SDUs in the transmitting side; and reset all state variables to their initial values.  }

}

7.2.3.21.2		Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.322, clause 5.4 and TS 36.331 clause 5.3.5.4.

[TS 36.322, clause 5.4]

RLC re-establishment is performed upon request by RRC, and the function is applicable for AM, UM and TM RLC entities.

When RRC indicates that an RLC entity should be re-established, the RLC entity shall:

…

-	if it is an AM RLC entity:

-	when possible, reassemble RLC SDUs from any byte segments of AMD PDUs with SN < VR(MR) in the receiving side, remove RLC headers when doing so and deliver all reassembled RLC SDUs to upper layer in ascending order of the RLC SN, if not delivered before;

-	discard the remaining AMD PDUs and byte segments of AMD PDUs in the receiving side;

-	discard all RLC SDUs and AMD PDUs in the transmitting side;

-	discard all RLC control PDUs.

-	stop and reset all timers;

-	reset all state variables to their initial values.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

….

1>	re-establish RLC for all RBs that are established;

…

7.2.3.21.3	Test description

7.2.3.21.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18] with the exceptions listed in table 7.2.3.21.3.1-1.

Table 7.2.3.21.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-Reordering | ms150 |
| t-PollRetransmit | ms150 |

7.2.3.21.3.2	Test procedure sequence

Table 7.2.3.21.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 1 | SS creates 3 RLC SDUs of size 40 bytes segmented into two AMD PDUs each. AMD PDU#1 and AMD PDU#2 belong to RLC SDU#1, AMD PDU#3 and #4 belong to RLC SDU#2 and AMD PDU#5 and #6 belong to RLC SDU#3. SS transmits AMD PDU#1 (SN=0), AMD PDU#2 (SN=1) and AMD PDU#4 (SN=3). | <-- | AMD PDU#1 AMD PDU#2 AMD PDU#4 | - | - |
| 1A | 30 ms after step 1 the SS allocates 1 UL grant of default size (UL grant allocation type 3). | <-- | (UL grant) |  |  |
| 2 | The UE returns RLC SDU#1. | --> | (RLC SDU#1) | - | - |
| 3 | SS does not acknowledge the reception of RLC SDU#1. | - | - | - | - |
| 4 | 90 ms after step 1 SS performs a RRC Connection Reconfiguration procedure including the mobilityControlInfo IE triggering RLC-reestablishment. (Note 1) | - | - | - | - |
| 4AA | The SS starts the UL default grant transmissions | - | - | - | - |
| 4A | The UE retransmits RLC SDU #1. (Note 1A) | --> | (RLC SDU#1) | - | - |
| 4B | SS transmits a STATUS PDU (ACK_SN = 1). | <-- | STATUS PDU | - | - |
| 5 | SS transmits AMD PDU#5 with SN=0 and the P field set to "1" | <-- | AMD PDU#5 | - | - |
| 6 | Check: Does the UE transmit a STATUS PDU? (Note 2) | --> | STATUS PDU (ACK_SN = 1) | 1 | P |
| 7 | SS transmits AMD PDU#6 with SN=Receiving_AM_Window_Size+2 | <-- | AMD PDU#6 | - | - |
| 8 | Check: Does the UE return RLC SDU#3 within 1s? (Note 3) | --> | (RLC SDU#3) | 1 | F |
| 9 | SS transmits AMD PDU#6 with SN=1 | <-- | AMD PDU#6 | - | - |
| 10 | Check: Does the UE return RLC SDU#3 with its first AMD PDU set to SN=1? | --> | (RLC SDU#3) | 1 | P |
| 11 | SS transmits a STATUS PDU (ACK_SN = 2). | <-- | STATUS PDU | - | - |
| Note 1: Upon a RLC re-establishment a conformant UE discards any remaining AMD PDUs in the receiver and transmitter side, stops and resets all timers and resets all state variables to their initial values. In case of CAT-M1 UEs an UL Grant of 56 bits will be provided to the UE. This is the same size as used in the regular RACH procedure. Note 1A: The UE will retransmit the PDCP SDU associated with RLC SDU#1 in accordance to TS 36.323 clause 5.2.1.1. Note 2: AMD PDU#4 is discarded by a conformant UE in step 4. Note 3: AMD PDU#6 is discarded by a conformant UE due to being outside the receiving window size. |  |  |  |  |  |

7.2.3.21.3.3	Specific message contents

Table 7.2.3.21.3.3-1: RRCConnectionReconfiguration (step 4, table 7.2.3.21.3.2-1)

| Derivation Path: 36.508 table 4.6.1-8: RRCConnectionReconfiguration, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo SEQUENCE { |  |  |  |
| targetPhysCellId | Set to the physical cell identity of cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO-MOD |  |  |
| radioResourceConfigCommon | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 1 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.2.3.21.3.3-2: RadioResourceConfigDedicated-HO-MOD (Table 7.2.3.21.3.3-1)

| Derivation Path: 36.508 table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO-MOD ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-SR |  |  |
| } |  |  |  |

Table 7.2.3.21.3.3-3: PhysicalConfigDedicated-SR (Table 7.2.3.21.3.3-2)

| Derivation Path: 36.508, Table 4.8.2.1.6-1, condition RBC-HO |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment | Condition |  |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |  |
| schedulingRequestConfig |  |  | SchedulingRequest-Config-DSR |  |  |  |  |
| } |  |  |  |  |  |  |

Table 7.2.3.21.3.3-4: SchedulingRequest-Config-DSR (Table 7.2.3.21.3.3-3)

| Derivation Path: 36.508, Table 4.6.3.-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| dsr-TransMax | n32 |  |  |
| } |  |  |  |
| } |  |  |  |