3GPP TSG-RAN5 Meeting #100	R5-234439r1

# Toulouse, France, Augest 21nd - May 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0566 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for FR1 L1 RSRP relative accuracy TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To fix following issues in TT analysis for FR1 L1-RSRP relative accuracy test cases (impacted TCs are: 4.7.4.1.2, 4.7.4.2.2, 6.7.6.1.2 and 6.7.6.2.2): - It’s inappropriate that Es/Noc uncertainties of L1-RSRP RSs are not considered when determing the effect on difference L1-RSRP. Nominal value of differential L1-RSRP can be expressed as: differential L1-RSRP = RS#0 Es/Noc + RF channel 1 Noc – (RS#1 Es/Noc + RF channel 1 Noc). One can note that RF channel 1 Noc is canceled out during calculation, so its uncertainty has no effect on differential L1-RSRP. However, RS#0 Es/Noc and RS#1 Es/Noc may have different uncertainty direction, so Es/Noc uncertainty needs to be counted twice during calculation. We purpose to use sensitive factor 2 in cell M67-U67. - The reporting range calculation in cell V87-AD87, V89-AD89 are incorrect. Step size of 1dB is used in these cells to calculate L1-RSRP reporting value. However, according to 38.133 Table 10.1.6.1-2, the step size for differential RSRP is 2dB rather than 1dB. Furthermore, the reporting value can only be positive values. The equation used in cells mentioned above should be changed to: Floor(differential RSRP/2), if RS#0 L1-RSRP is larger than RS#1 L1-RSRP; Floor(-differential RSRP/2), if RS#0 L1-RSRP is no larger than RS#1 L1-RSRP |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Issues mentioned above are fixed. - Grouping table is updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 2543 2546 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: TT analysis is updated according to comments received. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Add the following attached .zip file to TR 38.903 attachment:

" 38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v2.zip"

Remove the following attached .zip file from TR 38.903 attachment:

" 38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT.zip"

<End of modified section 1>

<Start of modified section 2>

8	Grouping of test cases defined in TS 38.533

Table 8-1: Grouping of FR1 test cases defined in Clauses 4, 4A, 6, 8, 16 and 18 of TS 38.533

| Group | Test Case Numbers |  | .zip file name | Comments |  |
| --- | --- | --- | --- | --- | --- |
| SCell_Activation_01 | 4.5.3.1 4.5.3.2 4.5.3.3 6.5.3.1 6.5.3.2 6.5.3.3 |  | “38.533 4.5.3 +6.5.3 TT.zip” | “2 Inter Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |  |
| <Unchanged contents are omitted> |  |  |  |  |  |
| L1-RSRP_Abs_Acc_01 | 4.7.4.1.1 6.7.4.1.1 4.7.4.2.1 6.7.4.2.1 |  | “38.533 4.7.4.1.1+4.7.4.2.1+6.7.4.1.1+6.7.4.2.1 TT.zip” | 1 NR Cell, periodic L1-RSRP reporting, No fading |  |
| L1-RSRP_Rel_Acc_01 | 4.7.4.1.2 6.7.4.1.2 4.7.4.2.2 6.7.4.2.2 |  | “38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v2.zip” | 1 NR Cell with 2 Beams, periodic L1-RSRP reporting, No fading |  |
| SSB_Based_BFR | 4.5.5.1 4.5.5.2 6.5.5.1 6.5.5.2 16.5.2.2 |  | “38.533 4.5.5.1+4.5.5.2+6.5.5.1+6.5.5.2 TT v3.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” Note: The spreadsheet “16.5.2.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |  |
| <Unchanged contents are omitted> |  |  |  |  |  |
| RRC_reconfiguration_delay_01 | 7.5.11.1 | “38.533 7.5.11.1 TT.zip” |  |  | “2 NR Cells, 3 Time Periods, No Fading” |
| UE_Rx_Tx_difference_PDC_02 | 7.6.13.1 | “38.533 7.6.13.1 TT.zip” |  |  | “1 NR FR2 Cell, 2 time periods, no fading” |

<End of modified section 2>