3GPP TSG-RAN5 Meeting #100	R5-234464r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3923 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS Broadcast for TC 14.1.2.2-14.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.SIB1 for NR Cell configuration in 14.1.2.2 and 14.1.2.3 are not correct |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Correct the SIB1 for NR Cell. 2.Editorial update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SIB1 for NR Cell configuration in 14.1.2.2 and 14.1.2.3 are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.2.2,14.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - update the si-SchedulingInfo-v1700 message structure to align with latest 38.331 SI-SchedulingInfo-v1700 ::= SEQUENCE { schedulingInfoList2-r17 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17, dummy SI-RequestConfig OPTIONAL } |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.2.2.3	Test description

14.1.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1) and NR Cell 3 (TAI-1).

-	The SS configures the NR Cell 1 as the "Serving cell" and NR Cell 3 as "Non-suitable "Off" cell".

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

-	System information combination NR-25 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 3.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.1.2.2.3.2	Test procedure sequence

Table 14.1.2.2.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0", and "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 14.1.2.2.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for inter-frequency neighbour NR cell 3(measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |

Table 14.1.2.2.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for inter-frequency neighbour NR cell 3(measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |

Table 14.1.2.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is made interested in receiving a MBS service with MBS Service ID ‘000001’H associated with the MBS FSAI-1 (Note 1). | - | - | - | - |
| 2 | The UE is made aware that the MBS Service ID ‘000001’H is ongoing (Note 1). | - | - | - | - |
| 3 | The SS starts to broadcast SIB21 (according to System information combination NR-24) as defined in TS 38.508-1 [4] clause 4.4.3.1.2) on NR Cell 1 including mbs-FSAI-InterFreqList-r17 indicating MBS-FSAI-r17=1 for the frequency of NR Cell 3. | - | - | - | - |
| 4 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification on NR Cell 1. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 5 | Check: Does the UE transmit MBSInterestIndication message. | --> | MBSInterestIndication | 1 | P |
| 6 | The SS waits for 1s. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message to setup inter frequency measurement on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1 to confirm the setup of inter- frequency measurement. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS changes NR Cell 1 and NR Cell 3 level according to the row "T1" in table 14.1.2.2.3.2-1/2. | - | - | - | - |
| 10 | The UE transmits a MeasurementReport message to report event A3 on NR Cell 1 with the measured RSRP, RSRQ value for NR Cell 3. | --> | MeasurementReport | - | - |
| 11 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform inter-frequency handover to NR Cell 3. | <-- | RRCReconfiguration | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message on NR Cell 3? | --> | RRCReconfigurationComplete | - | - |
| 13 | UE transmits an MBSInterestIndication message on NR Cell 3 | --> | MBSInterestIndication | 3 | P |
| 14 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 15 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 3. | - | - | - | - |
| 16a1-16a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 3 with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 17 is repeated 5 times | - | - | - | - |
| 17 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 18 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 19 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 20 | Check: Is the number of reported MBS Packets received on the MTCH in step 19 greater than zero? | - | - | 2 | P |
| 21 | The SS starts to broadcast SIB20 (according to System information combination NR-25) as defined in TS 38.508-1 [4] clause 4.4.3.1.2) on NR Cell 1. | - | - | - | - |
| 22 | The UE is made interested in receiving a MBS service with MBS Service ID ‘000000’H associated with the MBS FSAI-0 and not interested in receiving a MBS service with MBS Service ID ‘000001’H associated with the MBS FSAI-1 (Note 1). | - | - | - | - |
| 23 | The UE is made aware that the MBS Service ID‘000000’H is ongoing (Note 1). | - | - | - | - |
| 24 | UE transmits an MBSInterestIndication message on NR Cell 3 | --> | MBSInterestIndication | 4 | P |
| 25 | The SS transmits an RRCReconfiguration message on NR Cell 3 to order the UE to perform inter-frequency handover to NR Cell 1 within 1s. | <-- | RRCReconfiguration | - | - |
| 26 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1? | --> | RRCReconfigurationComplete | - | - |
| 27 | UE transmits an MBSInterestIndication message on NR Cell 1 | --> | MBSInterestIndication | 5 | P |
| 28 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 29 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. | - | - | - | - |
| - | Exception: Step 30 is repeated 5 times | - | - | - | - |
| 30 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 31 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 32 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 33 | Check: Is the number of reported MBS Packets received on the MTCH in step 32 greater than the number of reported in step 19? | - | - | 1 | P |
| Note 1: The request may be performed by MMI or AT command. |  |  |  |  |  |

14.1.2.2.3.3	Specific message contents

Table 14.1.2.2.3.3-1: SIB1 of NR Cell 1 and NR Cell 3 (preamble and all steps for NR Cell 3, step 21 for NR Cell 1, Table 14.1.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.2.2.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  | 2 entries |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 3 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType20 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| SchedulingInfo2-r17 [2] SEQUENCE { |  |  | entry 2 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 4 | entry number for si-SchedulingInfo in SIB1 +2 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType21 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.2.2.3.3-1A: SIB1 of NR Cell 1 (step 3, Table 14.1.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.2.2.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  | 1 entry |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 3 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType21 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.2.2.3.3-2: ServingCellConfigCommonSIB (Table 14.1.2.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.2.2.3.3-3 |  |
| } |  |  |  |

Table 14.1.2.2.3.3-3: DownlinkConfigCommonSIB (Table 14.1.2.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.2.2.3.3-4 |  |
| } |  |  |  |

Table 14.1.2.2.3.3-4: BWP-DownlinkCommon (Table 14.1.2.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditioni MBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.2.3.3-5: SIB21 of NR Cell 1 and NR Cell 3 (preamble and all steps, Table 14.1.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |
| mbs-FSAI-IntraFreq-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000000’H | entry 1 OCTET STRING (SIZE (3)) FSAI-0 | NR Cell 1 |
|  | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) FSAI-1 | NR Cell 3 |
| } |  |  |  |
| mbs-FSAI-InterFreqList-r17 SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 { | 1 entry |  |  |
| MBS-FSAI-InterFreq-r17[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq-r17 | ARFCN-ValueNR of NR Cell 3 |  | NR Cell 1 |
|  | ARFCN-ValueNR of NR Cell 1 |  | NR Cell 3 |
| mbs-FSAI-List-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) FSAI-1 | NR Cell 1 |
|  | ‘000000’H | entry 1 OCTET STRING (SIZE (3)) FSAI-0 | NR Cell 3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>

<Start of modified section 2>

14.1.2.3.3	Test description

14.1.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1) and NR Cell 2 (TAI-1).

-	The SS configures the NR Cell 1 as the "Serving cell" and NR Cell 2 as "Non-suitable "Off" cell".

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 2.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.1.2.3.3.2	Test procedure sequence

Table 14.1.2.3.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0", and "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 14.1.2.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 2 (measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -88 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 1(measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |

Table 14.1.2.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 2 (measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 1(measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |

Table 14.1.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is made interested in receiving a MBS service with MBS Service ID‘000001’H associated with the MBS FSAI -1 (Note 1). | - | - | - | - |
| 2 | The UE is made aware that the MBS Service ID‘000001’H is ongoing (Note 1). | - | - | - | - |
| 3 | The SS starts to broadcast SIB21 (according to System information combination NR-22) as defined in TS 38.508-1 [4] clause 4.4.3.1.2) on NR Cell 1 including mbs-FSAI-IntraFreqList-r17 indicating MBS-FSAI-r17=1. | - | - | - | - |
| 4 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification on NR Cell 1. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 5 | Check: Does the UE transmit MBSInterestIndication message. | --> | MBSInterestIndication | 1 | P |
| 6 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 7 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. | - | - | - | - |
| 8a1-8a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 9 is repeated 5 times | - | - | - | - |
| 9 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 10 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 11 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 12 | Check: Is the number of reported MBS Packets received on the MTCH in step 11 greater than zero? | - | - | 2 | P |
| 13 | The SS waits for 1s. | - | - | - | - |
| 14 | The SS transmits an RRCReconfiguration message to setup intra frequency measurement on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1 to confirm the setup of intra- frequency measurement. | --> | RRCReconfigurationComplete | - | - |
| 16 | The SS changes NR Cell 1 and NR Cell 2 level according to the row "T1" in table 14.1.2.3.3.2-1/2. | - | - | - | - |
| 17 | The UE transmits a MeasurementReport message to report event A3 on NR Cell 1 with the measured RSRP, RSRQ value for NR Cell 2. | --> | MeasurementReport | - | - |
| 18 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform intra-frequency handover to NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 19 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2? | --> | RRCReconfigurationComplete | - | - |
| 20 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 21 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 2. | - | - | - | - |
| - | Exception: Step 22 is repeated 5 times | - | - | - | - |
| 22 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 23 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 24 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 25 | Check: Is the number of reported MBS Packets received on the MTCH in step 24 greater than the number of reported in step 11? | - | - | 2 | P |
| 26 | The SS transmits an RRCReconfiguration message to setup intra-frequency measurement on NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 27 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2 to confirm the setup of intra-frequency measurement. | --> | RRCReconfigurationComplete | - | - |
| 28 | The SS changes NR Cell 1 and NR Cell 2 level according to the row "T2" in table 14.1.2.3.3.2-1/2. | - | - | - | - |
| 29 | The UE transmits a MeasurementReport message to report event A3 on NR Cell 2 with the measured RSRP, RSRQ value for NR Cell 1. | --> | MeasurementReport | - | - |
| 30 | The SS transmits an RRCReconfiguration message on NR Cell 2 to order the UE to perform intra-frequency handover to NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 31 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1? | --> | RRCReconfigurationComplete | - | - |
| 32 | UE transmits an MBSInterestIndication message on NR Cell 1 | --> | MBSInterestIndication | 3 | P |
| 33 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 34 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. | - | - | - | - |
| - | Exception: Step 35 is repeated 5 times | - | - | - | - |
| 35 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 36 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 37 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 38 | Check: Is the number of reported MBS Packets received on the MTCH in step 37 greater than the number of reported in step 25? | - | - | 2 | P |
| 39 | The UE is made secondly interested in receiving a MBS service with MBS Service ID‘000002’H associated with the MBS FSAI-1 (Note 1). | - | - | - | - |
| 40 | The UE is made aware that the MBS Service ID‘000002’H is ongoing (Note 1). | - | - | - | - |
| 41 | UE transmits an MBSInterestIndication message on NR Cell 1 to update the mbs-ServiceList | --> | MBSInterestIndication | 4 | P |
| Note 1: The request may be performed by MMI or AT command. |  |  |  |  |  |

14.1.2.3.3.3	Specific message contents

Table 14.1.2.3.3.3-1: SIB1 of NR Cell 1 and NR Cell 2 (preamble and all steps for NR Cell 2, preamble for NR Cell 1, Table 14.1.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.2.3.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE { |  | 1 entry |  |  |
| schedulingInfoList2-r17 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  |  |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 2 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType20 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| areaScope-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| dummy |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.2.3.3.3-1A: SIB1 of NR Cell 1 (step3, Table 14.1.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.2.3.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE { |  | 2 entries |  |  |
| schedulingInfoList2-r17 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  |  |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 2 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType20 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| areaScope-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| SchedulingInfo2-r17 [2] SEQUENCE { |  |  | entry 2 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 3 | entry number for si-SchedulingInfo in SIB1 +2 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType21 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| areaScope-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| dummy |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.2.3.3.3-2: ServingCellConfigCommonSIB (Table 14.1.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.2.3.3.3-3 |  |
| } |  |  |  |

Table 14.1.2.3.3.3-2: ServingCellConfigCommonSIB (Table 14.1.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.2.3.3.3-3 |  |
| } |  |  |  |

Table 14.1.2.3.3.3-3: DownlinkConfigCommonSIB (Table 14.1.2.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.2.3.3.3-4 |  |
| } |  |  |  |

Table 14.1.2.3.3.3-4: BWP-DownlinkCommon (Table 14.1.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditioni MBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.3.3.3-5: SIB21 of NR Cell 1 (step 3, Table 14.1.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |
| mbs-FSAI-IntraFreq-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) FSAI-1 |  |
| } |  |  |  |
| mbs-FSAI-InterFreqList-r17 | Not present |  |  |
| } |  |  |  |

<End of modified section 2>